***Sub Engineer***

1. Flint glass

a. has slightly the greater refracting power than crown glass

b. has roughly double refracting power than that of crown

c. and crown glass proportions yield the required focal length and neutralise the dispersion produced by the convex lens at the emergence from the concave

d. all the above.

2. Simpson's rule for calculating areas states that the area enclosed by a curvilinear figure divided into an even number of strips of equal width, is equal to

a. one third the width of a strip, multiplied by the sum of two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

b. one third the width of a strip, multiplied by the sum of two extreme offsets, four times the sum of the remaining odd offsets, and twice the sum of the even offsets

c. one sixth the width of a strip, multiplied by the sum of the two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

d. none of these

3. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

4. The total change in level along the line is equal to total back sights

a. minus total fore sights

b. the total rises minus total falls

c. the reduced level of last point minus reduced level of the first point

d. all the above.

5. Duco paints are

a. cellulose paints

b. emulsion paints

c. bituminous paints

d. oil paints.

6. Pick up the correct statement from the following:

a. The bulking of fine sand is more than that of coarse sand

b. If the percentage content of moisture exceeds 10%, increase in bulk of sand starts increasing

c. The volume of fully saturated sand equals that of dry sand

d. All the above.

7. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

8. Based on its dry weight, a freshly felled tree may contain water

a. 25%

b. 50%

c. 75%

d. 100%.

9. A material which obeys Hook's law, is subjected to direct stress σ0. At its elastic limit, the following statement is true,

a. Maximum shear stress =

b. Strain energy =

c. Shear strain energy =

d. All the above.

10. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

11. In a three hinged arch, the shear force is usually

a. maximum at crown

b. maximum at springings

c. maximum at quarter points

d. varies with slope.

12. The type of butt joints in common use, is :

a. single inverted V-butt joint

b. double V-butt joint

c. double U-butt joint

d. single V-butt joint.

13. Pick up the correct statement from the following :

a. For tranquil flow h > 2g or v <

b. For critical flow

c. For torrential flow h >  or v < gh

d. None of these.

14. Total pressure on the top of a closed cylindrical vessesl completely filled with liquid, is directly proportional to

a. (radius)2

b. (radius)3

c. (radius)4

d. none of these.

15. The ratio of the percentagge error in the discharge and percentage error in the measurement of head over a triangular notch, is

a. 2/3

b. 3/2

c. 2/5

d. 5/2

16. Capillary rise of water is

a. directly proportional to surface tension

b. inversely proportional to water density

c. inversely proportional to diameter of the tube

d. All of these.

17. Pick up the correct statement from the following:

a. The percent settlement at any time is called degree of consolidation

b. Time factor is a dimensionless quantity

c. The initial curve on either side of the point of unloading and reloading is called 'virgin curve'

d. All the above.

18. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

19. The quantity of seepage of water through soils is proportional to

a. coefficient of permeability of soil

b. total head loss through the soil

c. neither (a) nor (b)

d. both (a) and (b).

20. The critical exist gradient of seepage water in soils, increases with

a. a decrease in specific gravity

b. a decrease in void ratio

c. both (a) and (c)

d. none of these.

21. After prestressing process is completed, a loss of stress is due to

a. elastic shortening of concrete

b. creep of concrete

c. creep of steel

d. all the above.

22. If the sides of a slab simply supported on edges and spanning in two directions are equal, the maximum bending moment is multiplied by

a. 0.3

b. 0.4

c. 0.5

d. 0.7

23. Lapped splices in tensile reinforcement are generally not used for bars of size larger than

a. 24 mm diameter

b. 30 mm diameter

c. 36 mm diameter

d. 32 mm diameter

24. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

25. Pick up the correct statements from the following :

a. cracks appear on the plastered surface in the form of hair cracks

b. in brick work, the efflorescence is removed by applying a solution of zinc sulphate and water

c. excessive thermal variations in the backing or plaster causes the plaster to fall

d. all the above.

26. Which one of the following activities is not correct as applicable to brick corbels

a. the maximum projection of the corbel should not be more than the thickness of the wall

b. the maximum projection of each corbel course should be limited to a quarter brick at a time

c. the discontinuous corbels are used to carry heavy concentrated loads

d. stretcher bond is generally used for the construction of brick corbel.

27. An aggregate is said to be flaky if its least dimension is less than

a. 2/5th of mean dimension

b. 3/5th of mean dimension

c. 4/5th of mean dimension

d. none of these.

28. Placing of concrete should preferably be done at a temperature of

a. 10°C

b. 20°C

c. 23°C

d. 27 ± 2°C

29. The requirement of water per capita per day, is

a. 90 litres

b. 150 litres

c. 250 litres

d. 400 litres.

30. The most important and widely used tube well in India, is

a. strainer well

b. cavity well

c. slotted well

d. perforated pipe well.

31. For detecting the nitrites in the sewage, the matching colour may be developed by adding

a. Pottassium permanganate

b. Sulphuric acid and napthamine

c. Phenol-di-sulphuric acid pottassium hydroxide

d. None of these.

32. The following is the physical characteristic of sewage

a. colour

b. odour

c. temperature

d. all the above.

33. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

34. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

35. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

36. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

37. The minimum ratio of the radii of two circular curves of a compound curve, is kept

a. 1.25

b. 1.5

c. 1.75

d. 2.0

38. Reinforcement in cement concrete slab of road pavements, is placed

a. transversely

b. longitudinally and transversely

c. in the form of welded wire mesh

d. none of these.

39. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

40. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

41. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

42. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

43. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

44. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

47. The final selection of a construction site, is done by

a. local civil authority representative

b. representative of engineer authority

c. representative of administration

d. all the above.

48. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

49. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.

50. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.