***Sub Engineer***

1. Pick up the correct statement from the following :

a. to locate a gross error in bearing that may exist in controlled theodolite traverse, we may plot the traverse from each end. The traverse station having the same coordinates by each route is the one where the error lies

b. to locate a gross error in bearing, in a controlled traverse, we plot the traverse and the station through which perpendicular to sector of the closing line passes is the station at which the error was made

c. to locate a gross error due to taping in a controlled traverse, we plot the traverse to a convenient scale. The bearing of the closing error will be approximately the same as that of the leg in which the gross error consists

d. all the above.

2. On a diagonal scale, it is possible to read up to

a. one dimension

b. two dimensions

c. three dimensions

d. four dimensions.

3. The area of a plane triangle ABC, having its base AC and perpendicular height h, is

a.  ba sin C

b.  bc sin A

c. S(S - a)(S - b)(S - c) where S is

d. all the above

4. The distance between steps for measuring down hill to obtain better accuracy

a. decreases with decrease of slope

b. increases with increase of slope

c. decreases with increase of slope

d. decreases with decrease of weight of the chain.

5. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

6. Elastomers can extend upto

a. five times their original dimensions

b. seven times their original dimensions

c. ten times their original dimensions

d. three times their original dimensions.

7. If the furnace is provided with insufficient fuel at low temperatures, the type of pig iron produced, is called

a. Bessemer pig

b. Grey or foundry pig

c. White or forge pig

d. Mottled pig.

8. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

9. The ratio of maximum and average shear stresses on a rectangular section, is

a. 1.25

b. 1.5

c. 2.0

d. 2.5

10. The ratio of lateral strain to axial strain of a homogeneous material, is known

a. Yield ratio

b. Hooke's ratio

c. Poisson's ratio

d. Plastic ratio.

11. The shape of the bending moment diagram over the length of a beam, carrying a uniformly increasing load, is always

a. linear

b. parabolic

c. cubical

d. circular.

12. The under mentioned type is simple strain

a. compressive strain

b. shear strain

c. volumetric strain

d. all the above.

13. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

14. Pick up the incorrect statement from the following :

a. In radial flow, fluid flows such that pressure and velocity at any point change with respect to its distance from the central axis

b. In radial flow, velocity of flow is in a radial direction

c. In radial flow, flow may take place radially inward to or outward from the centre

d. In radial flow, flow is one dimensional with stream lines parallel.

15. For a long pipe, the head loss

a. at the entrance is ignored

b. at the outlet is ignored

c. at the entrance and outlet both are ignored

d. due to friction is ignored.

16. Centre of buoyancy is

a. centroid of the floating body

b. centroid of the fluid displaced

c. centre of pressure of the displaced liquid

d. none of these.

17. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

18. Plasticity index is defined as the range of water content between

a. liquid and plastic limit

b. plastic limit and semi solid limit

c. semi-solid limit and liquid limit

d. liquid limit and solid limit.

19. Soil classification of composite soils, exclusively based on the particle size distribution, is known

a. particle classification

b. textural classification

c. High Way Research Board classification

d. unified soil classification.

20. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

21. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

22. As per I.S. 456 - 1978, the pH value of water shall be

a. less than 6

b. equal to 6

c. not less than 6

d. equal to 7

23. A part of the slab may be considered as the flange of the T-beam if

a. flange has adequate reinforcement transverse to beam

b. it is built integrally with the beam

c. it is effectively bonded together with the beam

d. all the above.

24. A foundation rests on

a. base of the foundation

b. subgrade

c. foundation soil

d. both (b) and (c)

25. For effective drainage, the finished surface of flat roof should have a minimum slope of

a. 1 in 20

b. 1 to 50

c. 1 in 10

d. 1 in 5.

26. The pile which supports the load partly by friction and partly by resting on hard stratum, is called

a. friction pile

b. bearing pile

c. friction bearing pile

d. rough pile.

27. Le-Chatelier's apparatus is used for testing

a. soundness of cement

b. hardness of cement

c. strength of cement

d. durability of cement.

28. The diameter of the Vicat plunger is 10 mm and its length varies from

a. 30 mm to 40 mm

b. 40 mm to 50 mm

c. 50 mm to 60 mm

d. none of these.

29. The formula  for the head loss in conduits is generally known as

a. Hazen-William's formula

b. Manning's formula

c. Darcy-Weisbach formula

d. Nikuradse formula.

30. In centrifugal pump installation,

a. the diameter of the pipe at inlet as well as at outlet, is kept smaller than the delivery pipe

b. the foot valve fitted at the bottom end of the suction pipe, prevents the movement of water from the pump when it is stopped

c. the motor may burn out if it is started with empty delivery pipe and with open gate valve

d. all the above.

31. Hazin's formula  is used for the settlement velocity of the particles of diameter

a. less than 0.01 mm

b. less than 0.05 mm

c. less than 0.1 mm

d. greater than 0.1 mm.

32. Pick up the correct statement from the following :

a. Indore method of composting involves decomposition under aerobic conditions

b. Bangalore method of composting involves decomposing under anaerobic conditions

c. Fully stabilised refuse by the Bangalore method of compositing is a powdery mass called humus

d. all of these.

33. Garnett's diagrams are used for graphical solution of design equations of a canal by

a. Lacey's theory

b. Kennedy's theory

c. Gibb's theory

d. Lindlay theory.

34. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

35. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

36. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

37. Set-back distance is the distance between

a. road land boundary and building line

b. road land boundary and control line

c. building line and control line

d. road land boundary and control line.

38. The head of public works department of any Indian state, is

a. Transport Minister

b. Chief Engineer

c. Superintending Engineer

d. Executive Engineer.

39. Deviation of the alignment of a trace cut may be permitted in areas involving

a. land slides

b. sand dunes

c. dens

d. none of these.

40. In ideal pavement is constructed with

a. hard soil

b. Portland cement concrete

c. tar

d. none of these.

41. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

42. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

43. The cost of the earthwork in excavation for the surface drain of cross-section shown in the given figure for a total length of 5 metres @ Rs. 450% cum, is

a. Rs. 400

b. Rs. 425

c. Rs. 450

d. Rs. 500

44. The correct prismoidal formula for volume is

a. D [first area + last area + ∑ Even area + 2 ∑ odd areas]

b.  [first area + last area + 4 ∑ Even area + 2 ∑ odd areas]

c.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas]

d.  [first area + last area + 2 ∑ Even area + 4 ∑ odd areas].

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

47. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

48. If D is the duration, ES and EF are the earliest start and finish, LS and LF are latest start and latest finish time, then the following relation holds good

a. LS = LF - D

b. LF = LS + D

c. D = EF - ES

d. all the above.

49. From the end of an instrumental runway, the approach surface rises outwards

a. 1 in 20

b. 1 in 30

c. 1 in 40

d. 1 in 50

50. Pick up the correct statement from the following:

a. The basic length of a runway is increased at a rate of 7% per 300 m of elevation of M.S.L.

b. The standard temperature at the site is obtained by reducing the standard sea level temperature of 15°C at the rate of 6.5°C per 1000 m rise in elevation

c. The aerodrome reference temperature is the monthly mean of the mean daily temperature for the hottest month of the year

d. All the above.