***Sub Engineer***

1. The reduced bearing of a line is N 87° W. Its whole circle bearing is

a. 87°

b. 273°

c. 93°

d. 3°

2. Pick up the incorrect statement from the following :

a. while measuring a distance with a tape of length 100.005 m, the distance to be increasing by 0.005 m for each tape length

b. an increase in temperature causes a tape to increase in length and the measured distance is too large

c. the straight distance between end points of a suspended tape is reduced by an amount called the sag correction

d. a 100 m tape of cross section 10 mm x 0.25 mm stretches about 10 mm under 5 kg pull.

3. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

4. A back sight

a. is always taken on a point of known elevation or can be computed

b. is added to the known level to obtain the instrument height

c. taken on an inverted staff is treated as negative

d. all the above.

5. Non acid-resistant asbestos is :

a. tremolite asbestos

b. chrysolite abestos

c. amosite absestos

d. none of these.

6. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

7. A good brick earth should contain :

a. about 50% to 60% of silica

b. not more than 5% of lime

c. about 5 to 6% of oxide of lime

d. All the above.

8. Lacquer paints

a. are less durable as compared to enamel paints

b. consist of resin and nitro-cellulose

c. contain alcohol as thinner

d. all the above.

9. A simply supported beam which carries a uniformly distributed load has two equal overhangs. To have maximum B.M. produced in the beam least possible, the ratio of the length of the overhang to the total length of the beam, is

a. 0.207

b. 0.307

c. 0.407

d. 0.508

10. For calculating the allowable stress of long columns  is the empirical formula, known as

a. Straight line formula

b. Parabolic formula

c. Perry's formula

d. Rankine's formula.

11. A 8 metre long simply supported rectangular beam which carries a distributed load 45 kg/m. experiences a maximum fibre stress 160 kg/cm2. If the moment of inertia of the beam is 640 cm4, the overall depth of the beam is

a. 10 cm

b. 12 cm

c. 15 cm

d. 16 cm

12. A beam is said to be of uniform strength, if

a. deflection is same throughout the beam

b. bending stress is same throughout the beam

c. shear stress is same throughout the beam

d. none of these.

13. The intensity of pressure due to sudden closure of a valve of a pipe in which water flows with velocity v, is directly proportional to :

a. square root of the bulk modulus of elasticity of water

b. bulk modulus of elasticity of water

c. specific weight of water

d. none of these.

14. A cylindrical vessel 40 cm high is revolved about its vertical axis so that the water touches the bottom when it just spills out. If the radius of the cylinder is 5 cm, the angular velocity of rotation, is

a. 2 rad/sec

b. 3 rad/sec

c. 4 rad/sec

d. 5 rad/sec

15. A stepped notch is a combination of

a. rectangular notches of different sizes

b. triangular notches of different sizes

c. rectangular and triangular notches

d. all the above.

16. For mountaneous regions having steep slope, wagons for carrying liquid are made with bottom

a. parallel to the road surface

b. parallel to the horizontal surface

c. inclined upward while moving upwards

d. inclined downward while moving downwards.

17. Pick up the clay soil group which does not swell when wet from the following :

a. Kaolinite group

b. Illite group

c. Vermiculite group

d. Montrorillonite group.

18. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

19. Determination of water content of a soil sample suspected to contain gypsum is made by drying the sample for longer period at a temperature not more than

a. 60°C

b. 80°C

c. 100°C

d. 110°C

20. The ratio of the volume of water present in a given soil mass to the total volume of its voids, is known

a. porosity

b. void ratio

c. percentage voids

d. degree of saturation.

21. Pick up the correct statement from the following:

a. Lateral reinforcement prevents the shearing of concrete on diagonal plane

b. Lateral reinforcement stops breaking away of concrete cover, due to buckling

c. Lateral reinforcement in R.C.C. columns, is kept not less than 5 mm diameter

d. All the above.

22. The minimum head room over a stair must be

a. 205 cm

b. 210 cm

c. 200 cm

d. 230 cm

23. If the depth of actual neutral axis of a doubly reinforced beam

a. is greater than the depth of critical neutral axis, the concrete attains its maximum stress earlier

b. is less than the depth of critical neutral axis, the steel in the tensile zone attains its maximum stress earlier

c. is equal to the depth of critical neutral axis, the concrete and steel attain their maximum stresses simultanesouly

d. all the above.

24. Pick up the true statement from the following:

a. In the absence of beams, it is easier to install piping

b. In the absence of beams, it is easier to paint

c. A flat slab is capable to withstand concentrated loads

d. All the above.

25. The size of a floor tile commonly used, is

a. 15 cm x 15 cm x 1.8 cm

b. 20 cm x 20 cm x 2 cm

c. 22.5 x 22.5 cm x 2.2 cm

d. all the above.

26. In English garden wall bond

a. one course of headers to three or five course of stretchers

b. queen closer in provided in each heading course

c. the middle course of stretchers is started with a header to give proper vertical joints

d. all the above.

27. Workability improved by adding

a. foaming agent

b. oily-agent

c. aluminium compound

d. all the above.

28. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

29. In pressure supply mains, water hammer pressure is reduced by providing

a. sluice valves

b. air valves

c. pressure relief valves

d. none of the these.

30. For the prediction of future population of a city, the factor to be considered, is

a. births

b. deaths

c. migrants

d. all the above.

31. Assertion (A) : Discharging the effluents from the oxidation ponds just up stream of lakes or reservoirs is undesirable. Reason (R) : The discharged algae get settled in the reservoirs and cause anaerobic decomposition and other water qualities.

a. Both A and R are true and R is the correct explanation of A

b. Both A and R are true but R is not a correct explanation of A

c. A is true but R is false

d. A is false but R is true.

32. The non-clog pump which permits solid matter to pass out with the liquid sewage, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

33. Pick up the correct statement from the following:

a. Gravity water is harmful to crops

b. Hygroscopic water remains attached to soil molecules by chemical bond

c. Capillary moisture held in the soil pores against gravity by surface tension, is utilised by plants

d. All the above.

34. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

35. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

36. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

37. Increase in traffic volume, due to increase in transport vehicles, is known as

a. normal traffic growth

b. generated traffic growth

c. current traffic

d. none of these.

38. The head of public works department of any Indian state, is

a. Transport Minister

b. Chief Engineer

c. Superintending Engineer

d. Executive Engineer.

39. The steepest gradient permitted on roads which, in ordinary conditions, does not exceed, is known

a. maximum gradient

b. exceptional gradient

c. floating gradient

d. all the above.

40. To indicate proper control of consistency of a freshly mixed concrete for pavement construction, the slump should be between

a. 4 to 6 cm

b. 5 to 7 cm

c. 7 to 10 cm

d. 10 to 12 cm

41. Pick up the correct statement from the following:

a. Cement is paid per 50 kg bag

b. Lime is paid per quintal

c. Brick aggregates is paid per cum

d. All the above.

42. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

43. Carpet area does not include the area of

a. verandah, corridor and passage

b. bath room and lavatory

c. kitchen and pantry

d. All the above.

44. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

48. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

49. International Civil Aviation Organisation (I.C.A.O.) was set up at Montreal (Canada), in

a. 1929

b. 1939

c. 1947

d. 1950

50. Pick up the correct statement from the following:

a. Localizer indicates to the pilot his position with respect to the proposed alignment

b. The glide slope indicates the correct angle of descent

c. L.O.M. and L.M.M. help the pilot to judge his position for the runway

d. All the above.