***Sub Engineer***

1. If θ is the slope of the ground and l is the measured distance, the correction is

a. 2l sin2 θ/2

b. 2l cos2 θ/2

c. 2l tan2 θ/2

d. 2l cot2 θ/2.

2. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

3. If the whole circle bearing of a line is 270°, its reduced bearing is

a. N 90° W

b. S 90° W

c. W 90°

d. 90° W.

4. Pick up the wrong statement from the following :

a. the diaphragm is placed between the eyepiece and the objective but nearer to the later

b. the outer component of the objective is a double-convex lens of crown glass

c. the inner component of the objective is a flint glass, convexo-concave

d. all the above.

5. Chlorite, a green colour mineral is mainly derived from the decomposition of

a. augite

b. biotite

c. horn blende

d. All of these.

6. The minimum compressive strength of 1st class bricks should be

a. 90 kg/cm2

b. 100 kg/cm2

c. 120 kg/cm2

d. 130 kg/cm2.

7. Chemically, marble is known as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

8. Kaolin is chemically classified as

a. metamorphic rock

b. argillaceous rock

c. calcareous rock

d. silicious rock.

9. A shaft subjected to a bending moment M and a torque T, experiences

a. maximum bending stress =

b. maximum shear stress =

c. both (a) and (b)

d. neither (a) nor (b)

10. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

11. In a solid arch, shear force acts

a. along the axis of the arch

b. perpendicular to the axis of arch

c. tangentially to the arch

d. none of these.

12. The equivalent length of a column fixed at both ends, is

a. 0.5 l

b. 0.7 l

c. l

d. 2 l

13. The pressure rise due to water hammer depends upon

a. the length of pipe

b. time taken to close the valve

c. the elasticity of the pipe material

d. All of above.

14. The rise of the liquid along the walls of a revolving cylinder above the initial level, is

a. greater than the depression of the liquid at the axis of rotation

b. lesser than the depression of the liquid at the axis of rotation

c. the same as the depression of the liquid at the axis of rotation

d. none of these.

15. In Chezy's formula V = C mi

a. V is the mean velocity of flow

b. m is the hydraulic mean depth

c. i is the loss of head per unit length of pipe

d. All the above.

16. The formula Q = m x L x 2g x H3/2 where  was suggested by :

a. Bazin

b. Francis

c. Cipolletti

d. None of these.

17. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

18. If the unit weight of sand particles is 2.696 g/cc. and porosity in loose state is 44%, the critical hydraulic gradient for quick sand condition, is

a. 0.92

b. 0.93

c. 0.94

d. 0.95

19. For determining the moisture content of a soil sample, the following data is available Weight of container = 260 g, Weight of soil sample and = 320 g container, Weight of soil sample (dried) and = 310 g container. The moisture content of the soil sample, is

a. 15%

b. 18%

c. 20%

d. 25%

20. If the coefficients of volume change and compressibility of a soil sample are respectively 6.75 x 10-2 and 3 x 10-2, the void ratio of the soil sample, is

a. 1.15

b. 1.20

c. 1.25

d. 1.30

21. Post tensioning system

a. was widely used in earlier days

b. is not economical and hence not generally used

c. is economical for large spans and is adopted now a days

d. none of these.

22. If the permissible compressive and tensile stresses in a singly reinforced beam are 50 kg/cm2 and 1400 kg/cm2 respectively and the modular ratio is 18, the percentage area At of the steel required for an economic section, is

a. 0.596%

b. 0.696%

c. 0.796%

d. none of these.

23. The width of the flange of a T-beam should be less than

a. one-third of the effective span of the T-beam

b. distance between the centres of T-beam

c. breadth of the rib plus twelve times the thickness of the slab

d. least of the above.

24. A continuous beam shall be deemed to be a deep beam if the ratio of effective span to overall depth, is

a. 2.5

b. 2.0

c. less than 2

d. less than 2.5

25. Rotary drilling

a. is not suitable for deposits containing very coarse gravel

b. hinders the ground water observations and permeability test

c. is not economical for holes of less than 10 cm.

d. all the above.

26. The size of a floor tile commonly used, is

a. 15 cm x 15 cm x 1.8 cm

b. 20 cm x 20 cm x 2 cm

c. 22.5 x 22.5 cm x 2.2 cm

d. all the above.

27. To prevent segregation, the maximum height for placing concrete, is

a. 100 cm

b. 125 cm

c. 150 cm

d. 200 cm

28. Pick up the incorrect statement from the following:

a. The axis of the mould is perpendicular to the end faces

b. The internal surface of the mould is kept clean and free from set cement

c. The mould is in the form of a frustum of hexagonal pyramid

d. None of these.

29. Standard process of chlorination of water, is done by

a. plain chlorination

b. pre-chlorination

c. post-chlorination

d. double chlorination.

30. From the surface of reservoir, evaporation may be minimised by sprinkling

a. Hydrochloric acid

b. Cetyl alcohol

c. Methane

d. None of these.

31. Pick up the correct statement from the following :

a. Thermophilic organisms digest the sludge if the temperature ranges from 40° to 60°C

b. Mesophilic organisms digest the sludge if the temperature is between 25° and 40°C

c. 90% of digestion takes place in 30 days at 29°C, the optimum mesophilic temperature

d. All the above.

32. Disposal to sewage in large cities, is done in

a. irrigation

b. dilution

c. oxidation

d. putrifaction.

33. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

34. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

35. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

36. If D is the depth of water upstream of the throat above its sill, B is the width of the throat, to achieve critical flow in an open venturi flume, the theoretical maximum flow Q, is

a. Q = 1.71 BD

b. Q = 1.71 BD3/2

c. Q = 1/71 BD2/3

d. none of these.

37. According to Indian Road Congress, the width of carriageway, is

a. 7.0 m for two lanes without raised kerbs

b. 7.5 m for two lanes with raised kerbs

c. 10.5 m for three lanes pavements

d. All the above

38. At a road junction, 16 cross conflict points are severe, if

a. both are one-way roads

b. both are two-way roads

c. one is two-way road and other is one-way road

d. none of these.

39. Thickness of broken centre line markings for a four lane road, is generally kept

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm

40. The G.T. road from Lahore to Calcutta in undivided India, was constructed during

a. 3495-3500 BC

b. 265-270 BC

c. 1540-1545 AD

d. 1440-1450

41. The reduced levels of points, 30 metres apart along the longitudinal section of a road portion between chainages 5 and 9 are shown in the given figure. If there is a uniform up-gradient of the road 120 in 1, the chainage of the point with no filling or cutting is

a. (6 + 15) chains

b. (6 + 12) chains

c. (6 + 18) chains

d. None of these.

42. Pick up the correct statement from the following:

a. In a gully trap, a water seal of 6 to 7.5 cm is provided

b. The gully trap collects waste water from the kitchen, sink, wash basins, etc.

c. The gully trap disconnects the sullage drain from the main drainage system

d. The grating provided over gully traps is 23 cm square.

43. The area is measured correct to the nearest

a. 0.01 sqm

b. 0.02 sqm

c. 0.03 sqm

d. 0.04 sqm

44. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

45. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. The first stage of a construction, is

a. survey of the site

b. initiation of proposal

c. preparation of tender

d. allotment of funds.

48. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

49. Pick up the correct statement from the following:

a. The speed of the aircraft relative to the ground, is called cruising speed

b. The speed of the aircraft relative to wind, is called air speed

c. When wind is blowing the direction of the flight, air speed is less than cruising speed

d. All the above.

50. For the proposed runway stated in if the aerodrome reference temperature is 17°.2, the net designed length of the runway is

a. 2716 m

b. 2816 m

c. 2916 m

d. 3016 m