***Sub Engineer***

1. Total latitude of a point is positive if it lies

a. north of the reference parallel

b. south of the reference parallel

c. east of the reference parallel

d. west of the reference parallel.

2. The Huygen's telescope eye piece

a. is aplanatic

b. achromatic

c. both (a) and (b)

d. neither (a) nor (b).

3. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

4. Staff readings on pegs x and y from X station are 1.755 m and 2.850 m, and from station Y on staff head at Y and X are 0.655 m and 1.560 m. If reduced level of X is 105.5 m, the reduced level of Y is

a. 104.0 m

b. 104.5 m

c. 105.0 m

d. 105.5 m

5. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

6. Whitworth compressed steel is obtained when molten steel is subjected to a pressure of

a. 9 kg/mm2

b. 13 kg/mm2

c. 15 kg/mm2

d. 10 kg/mm2.

7. The rocks which are formed due to cooling of magma at a relatively shallow depth from the earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Ignoeous rocks.

8. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

9. The locus of the end point of the resultant of the normal and tangential components of the stress on an inclined plane, is

a. circle

b. parabola

c. ellipse

d. straight line.

10. Total strain energy theory for the failure of a material at elastic limit, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

11. A cantilever beam rectangular in cross-section is subjected to an isolated load at its free end. If the width of the beam is doubled, the deflection of the free end will be changed in the ratio of

a. 1/8

b. 1/2

c. 2

d. 3

12. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

13. Pick up the correct statement from the following :

a. The fluids which follow  are known as Newtonian fluids

b. The fluids which do not follow the linear relationship between shear and rate of strain are known as non Newtonian fluids

c. The substances which flow after yield strains, are known as plastics

d. all the above.

14. The best side slope for most economical trapezoidal section, is

a. 30°

b. 45°

c. 60°

d. None of these.

15. Pick up the correct statement from the following :

a. Discharge over a triangular notch is proportional to H5/2

b. Discharge over a rectangular notch is proportional to H3/2

c. Both (a) and (b)

d. Neither (a) nor (b).

16. For solving the problems is hydraulic engineering, the velocity used is

a. velocity at the centre of pipe section

b. average velocity of flow over a section

c. mean of the velocities at the centre and that along the pipe surface

d. none of these.

17. The ultimate Settlement of a soil is directly proportional to:

a. compressive index

b. void ratio

c. both (a) and (b)

d. none of these.

18. Pick up the correct statement from the following:

a. An unconfined compression test is a special case of triaxial compression test

b. An unconfined compression test is a special case of direct shear test

c. The confining pressure is maximum during an unconfined compression test

d. The cylindrical specimen of a soil is subjected to major principal stress till it fails due to shearing along the plane of the failure.

19. The reduction in volume of soil due to squeezing out of water from the voids, is termed

a. primary consolidation

b. primary compression

c. primary time effect

d. all the above.

20. A soil sample of mass specific gravity 1.92, has a moisture content 30%. If the specific gravity of solids is 2.75, the degree of saturation, is

a. 95.4%

b. 95.5%

c. 95.6%

d. 95.7%

21. The live load to be considered for an inaccessible roof, is

a. Nil

b. 75 kg/m2

c. 150 kg/cm2

d. 200 kg/m2

22. Lapped splices in tensile reinforcement are generally not used for bars of size larger than

a. 24 mm diameter

b. 30 mm diameter

c. 36 mm diameter

d. 32 mm diameter

23. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

24. Pick up the assumption for the design of a prestressed concrete member from the following :

a. A transverse plane section remains a plane after bending

b. During deformation limits, Hook's law is equally applicable to concrete as well as to steel

c. Variation of stress in reinforcement due to changes in external loading is negligible

d. All the above.

25. For brick construction, the lime-sand mortar, is

a. 1:2

b. 1:3

c. 1:4

d. 1:5.

26. The thickness of a reinforced brick partition wall, is generally kept

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm.

27. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

28. In slump test, each layer of concrete is compacted by a steel rod 60 cm long and of 16 mm diameter for

a. 25 times

b. 30 times

c. 40 times

d. 50 times

29. The standard B.O.D. at 20&Deg;C, is taken for the consumption in

a. 2 days

b. 3 days

c. 4 days

d. 5 days.

30. A strainer type well sunk through three pervious layers intervened by three impervious aquicludes, draws water from

a. top most pervious layer

b. central pervious layer

c. lowest pervious layer

d. all the pervious layers.

31. The formula  used for determining flow velocities in sewers, is known as

a. Chezy's formula

b. Bazin's formula

c. Kutter's formula

d. Manning's formula.

32. The temperature affects the

a. biological activity of bacterias in sewage

b. solubility of gases in sewage

c. viscosity of sewage

d. all the above.

33. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

34. A fall which maintains its depth, is

a. a trapezoidal notch fall

b. a low weir fall

c. a rectangular notch fall

d. all the above.

35. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

36. In a concrete canal the approximate permissible velocity of water should not exceed

a. 1 m/sec

b. 1.5 m/sec

c. 2 m/sec

d. 2.5 m/sec

37. Thickness of broken centre line markings for a four lane road, is generally kept

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm

38. Pick up the correct statement from the following:

a. Various geometric design features and generally guided by ruling design speed

b. The design speed for a given highway should preferably be uniform

c. Abrupt change in the design speed should not be permitted

d. all the above.

39. For a comfortable travel on Highways, the centrifugal ratio should not exceed

a. 0.15

b. 0.20

c. 0.25

d. 0.30

40. Pick up the correct statement from the following:

a. Seasonal cycle of traffic volume during April and November, is usually near the annual average

b. Mid-winter seasonal cycle of traffic is least

c. Mid-summer seasonal cycle of traffic is highest

d. All the above.

41. A portion of an embankment having a uniform up-gradient 1 in 500 is circular with radius 1000 m of the centre line. It subtends 180° at the centre. If the height of the bank is 1 m at the lower end, and side slopes 2:1, the earth work involved.

a. 26, 000 m3

b. 26, 500 m3

c. 27, 000 m3

d. 27, 500 m3

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

44. Pick up the correct statement from the following:

a. The actual expenditure involved to complete a work including incidental, establishment and travelling charges, is called actual cost

b. The formal acceptance by the administrative department for incurring an expenditure on the work, is called administrative approval

c. The order of a competent authority sanctioning a properly detailed estimate of the cost of a work of construction or repair is called technical sanction

d. All the above.

45. Military organisation is known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. none of these.

46. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

47. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

48. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

49. The maximum value of the angle of turning of the nose gear large jet aircrafts, is limited to

a. 20°

b. 30°

c. 45°

d. 60°

50. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.