***Sub Engineer***

1. Pick up the correct statement from the following :

a. Cross staff is used for setting out right angles

b. Gradiometer is used for setting out any required gradient

c. Line ranger is used for locating intermediate stations on a survey line

d. All the above.

2. Pick up the correct statement from the following :

a. the principal axes of both the lenses coincide the optical axis of the telescope

b. the lens nearer the object to be viewed is convex and is called objective

c. the lens nearer the eye is called eyepiece

d. all the above.

3. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

4. If the plane table is not horizontal in a direction at right angles to the alidade, the line of sight is parallel to the fiducial edge only for

a. horizontal sights

b. inclined sights upward

c. inclined sight downward

d. none of these.

5. Seasoning of timber is done

a. to make it water proof

b. to paint its surface

c. to increase its temperature

d. to remove water.

6. Pick up the correct statement from the following:

a. The phenol is either extracted from coal-tar or prepared from benzene

b. Phenol reacts with formaldehyde, to form phenol formaldehyde resin

c. The plastics prepared from phenol-formaldehyde are used for paints, varnishes, w.c. seats

d. All the above.

7. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

8. Turpentine oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

9. The force in BF of the truss shown in given figure, is

a. 4t compression

b. 4.5t tension

c. 4.5t compression

d. zero.

10. The strain energy stored in a spring when subjected to greatest load without being permanently distorted, is called

a. stiffness

b. proof resilience

c. proof stress

d. proof load.

11. If the length of a cantilever carrying an isolated load at its free end is doubled, the deflection of the free end will increase by

a. 8

b. 1/8

c. 1/3

d. 2

12. In the cantilever truss as shown in below figure, the horizontal component of the reaction at A, is

a. 30 tonnes

b. 60 tonnes

c. 90 tonnes

d. 120 tonnes

13. The property of steam function ψ is :

a. the flow around any path in the fluid is zero for continuous flow

b. the rate of change of ψ with distance in an arbitrary direction, is proportional to the component of velocity normal to that direction

c. the velocity vector may be found by differentiating the stream function

d. all the above.

14. Uniform flow is said to occur when

a. size and shape of the cross-section in a particular length remain constant

b. size and shape of the cross-section change along a length

c. frictional loss in the particular length of the channel will the more than the drop in its elevation

d. frictional loss in the particular length of the channel, will be less than the drop in elevation.

15. Which one of the following statements is true ?

a. The value of kinetic energy correction factor for turbulent flow lies between 1.03 to 1.06

b. The value of kinetic energy correction factor for laminar flow is 2

c. The practical value of kinetic energy correction factor for turbulent flow is unity

d. all the above.

16. Atmospheric pressure varies with

a. altitude

b. temperature

c. weather conditions

d. all of the above.

17. A partially saturated soil is classified as

a. one phase soil

b. two phase soil

c. three phase soil

d. four phase soil.

18. The minimum centre to centre distance of friction piles of 1 m diameter, is

a. 2 m

b. 2 m to 3 m

c. 3 m to 4 m

d. 5 m

19. In a purely cohesive soil, the critical centre lies at the intersection of

a. perpendicular drawn at l/3rd slope from toe and the locus of the centre

b. perpendicular drawn at 2/3rd slope from toe and the locus of the centre

c. directional angles

d. none of these.

20. Stoke's law sates that the velocity at which a grain settles out of suspension, the other factors remaining constant, is dependent upon

a. weight of grain

b. size of grain

c. shape and size of grain

d. shape, size and weight of grain.

21. If a rectangular prestressed beam of an effective span of 5 meters and carrying a total load 3840 kg/m, is designed by the load balancing method, the central dip of the parabolic tendon should be

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

22. In a simply supported slab the minimum spacing of distribution reinforcement, should be four times the effective thickness of the slab or

a. 30 cm

b. 40 cm

c. 50 cm

d. 60 cm

23. A reinforced concrete cantilever beam is 3.6 m long, 25 cm wide and has its lever arm 40 cm. It carries a load of 1200 kg at its free end and vertical stirrups can carry 1800 kg. Assuming concrete to carry one-third of the diagonal tension and ignoring the weight of the beam, the number of shear stirrups required, is

a. 35

b. 40

c. 45

d. 50

24. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

25. Pick up the correct statement from the following :

a. the first coat of stucco plaster is called scratch coat

b. the second coat of stucco plaster is called brown coat

c. the third coat of stucco plaster is called white coat

d. all the above.

26. To stagger vertical joints in successive courses of a wall, a piece of brick is generally used at the end of the course, which is known as

a. bat

b. header

c. stretcher

d. closer.

27. While compacting the concrete by a mechanical vibrator, the slump should not exceed

a. 5.0 cm

b. 7.5 cm

c. 10 cm

d. 15 cm

28. The 28 days cube strength of mass concrete using aggregates of maximum size 5 cm for gravity dams should be

a. between 350 to 600 kg/cm2

b. between 150 to 500 kg/cm2

c. below 200 kg/cm2

d. none of these.

29. The level of underground water is called

a. water level

b. water table

c. negative level

d. invert level.

30. The permissible amount of nitrites present in potable water, is

a. 15 ppm

b. 20 ppm

c. 45 ppm

d. Nil.

31. Discrete or granular particles change their

a. size

b. shape

c. weight

d. none of these.

32. Antisyphonage pipe is fitted

a. at the end of septic tanks

b. on manholes

c. with a W.C. trap

d. at the beginning of sewer line.

33. Pick up the correct statement from the following:

a. The full supply level of a canal should be above ground level

b. According to Lacey, regime conditions require a particular slope for a given discharge and silt factor

c. In case the ground slope is less than the required bed slope, the silt factor must be reduced by permitting the entry of coarse silt

d. All the above.

34. Lacey's equation for calculating flood discharge in rivers, is

a. V = 10.8 R1/2 S1/2

b. V = 10.8 R2/3 S1/2

c. V = 10.8 R2/3 S1/3

d. V = 10.8 R1/3 S2/3

35. A river training work is generally required when the river is

a. meandering

b. aggrading

c. degrading

d. all the above.

36. The measure to remove water logging of land, is

a. to reduce percolation from canals and water courses

b. to increase outflow from the ground water reservoir

c. both (a) and (b)

d. neither (a) nor (b)

37. From the point of tangency before an intersection, the route markers are fixed at a distance of

a. 20 m to 35 m

b. 40 m to 50 m

c. 50 m to 75 m

d. 100 m to 150

38. If cross slope of a country is upto 10% the terain is classified as

a. plain

b. rolling

c. mountainous

d. steep.

39. Super-elevation on roads in snow bound areas, should generally not exceed

a. 12%

b. 10%

c. 7%

d. 6%

40. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. Pick up the correct statement from the following:

a. The earth work of cutting in trenches or borrow pits in fairly uniform ground is measured with the help of average depths of the dead men

b. The earth work in trenches or borrow pits in irregular ground is measured by taking the difference in levels before and after completion of work

c. The earth work in trenches or borrow pits, where neither a nor b is feasible, are measured from the fillings after deduction of voids

d. All the above.

43. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

44. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

45. If t is the duration of an activity, t1 is the latest finish possible moment of its preceding activity and t2 is the earliest start possible moment, the independent float of the activity is

a. (t1 - t2) - t

b. t - (t1 - t2)

c. (t1 + t2) - t

d. t + (t1 - t2)

46. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

47. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

48. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. According to I.C.A.O. all markings on the runways are painted white and on taxiways

a. black

b. red

c. yellow

d. green

50. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m