***Sub Engineer***

1. Removal of parallax, may be achieved by focussing

a. the objective

b. the eye-piece

c. the objective and the eye-piece

d. none of these.

2. A standard steel tape of length 30 m and cross-section 15 x 1.0 mm was standardised at 25°C and at 30 kg pull. While measuring a base line at the same temperature, the pull applied was 40 kg. If the modulus of elasticity of steel tape is 2.2 x 106 kg/cm2, the correction to be applied is

a. - 0.000909 m

b. + 0.0909 m

c. 0.000909 m

d. none of these

3. Pick up the correct statement from the following :

a. Cross staff is used for setting out right angles

b. Gradiometer is used for setting out any required gradient

c. Line ranger is used for locating intermediate stations on a survey line

d. All the above.

4. For locating a distant object visible from two transit stations, the method usually preferred to, is

a. Angles and distances from transit stations

b. Angles from two transit stations

c. distance from two transit stations

d. Angle from one transit station and distance from the other.

5. Spirit varnish generally consists of

a. alcohol, wax and turpentine

b. pigment and synthetic resin

c. spirit and shellac

d. none of these.

6. In the cement the compound quickest to react with water, is

a. Tricalcium aluminate

b. Tetra-calcium alumino-ferrite

c. Tricalcium silicate

d. Dicalcium silicate.

7. Stuco paints are suitable for

a. stone masonry

b. brick walls

c. both (a) and (b)

d. neither (a) nor (b).

8. Most commonly used solvent in oil paints, is

a. petroleum

b. spirit

c. coaltar

d. turpentine.

9. The degree of indeterminacy of the frame in the given figure, is

a. 1

b. 2

c. 3

d. zero

10. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

11. If the stress produced by a prismatic bar is equal to the working stress, the area of the cross-section of the prismatic bar, becomes

a. zero

b. infinite

c. maximum

d. minimum.

12. For structural analysis of forces, the method refers to

a. moment-area-theorem

b. three-moment equation

c. Maxwell's reciprocal theorem

d. none of these.

13. The dimensionless parameter not applicable to flowing liquids, is

a. Weber number

b. Pressure coefficient

c. Kinematic viscosity

d. Friction factor.

14. Falling drops of water become spheres due to

a. adhesion

b. cohesion

c. surface tension

d. viscosity.

15. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

16. Discharge over an ogee weir remains the same as that of

a. sharp crested weir

b. triangular weir

c. cippoletti weir

d. drowned weir.

17. Maximum size of clay particles, is :

a. 0.002 mm

b. 0.04 mm

c. 0.06 mm

d. 0.08 mm

18. The water content of soil is defined as the ratio of

a. volume of water to volume of given soil

b. volume of water to volume of voids in soil

c. weight of water to weight of air in voids

d. weight of water to weight of solids of given mass of soil.

19. A saturated soil sample has water content of 40% and specific gravity of soil particles 2.7. The void ratio of the soil, is

a. 0.4

b. 0.52

c. 1.08

d. none of these.

20. Pick up the correct statement from the following:

a. When water table is above the base of a footing, the dry weight m should be used for soil below water table

b. When water table is located somewhat below the base of a footing, the elastic wedge is partly of moist soil and partly of submerged soil, and a suitable reduction factor is used

c. When water table is just at the base of the footing, no reduction factor is used

d. None of these.

21. For the design of a simply supported T-beam the ratio of the effective span to the overall depth of the beam is limited to

a. 15

b. 20

c. 25

d. 30

22. The diameter of transverse reinforcement of columns should be equal to one-fourth of the diameter of the main steel rods but not less than

a. 5 mm

b. 6 mm

c. 7 mm

d. 8 mm

23. An R.C.C. roof slab is designed as a two way slab if

a. it supports live loads in both directions

b. the ratio of spans in two directions is less than 2

c. the slab is continuous over two supports

d. the slab is discontinuous at edges.

24. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

25. The window which is provided in flat roof of a room, is known

a. dormer window

b. lantern window

c. louvered window

d. sky window.

26. A covering of concrete placed on the exposed top of an external wall, is known as

a. cornice

b. coping

c. frieze

d. lintal.

27. The bulk density of aggregates, depends upon

a. shape

b. grading

c. compaction

d. all the above.

28. Setting time of cement increases by adding

a. gypsum

b. hydrogen peroxide

c. calcium chloride

d. sodium oxide

29. Standard process of chlorination of water, is done by

a. plain chlorination

b. pre-chlorination

c. post-chlorination

d. double chlorination.

30. If the chosen diameter of a pipe, is less than the economical diameter

a. cost of pipe will be less

b. head loss will be high

c. cost of pumping will be more than saving

d. all the above.

31. The effluent of a septic tank is

a. fit for discharge into any open drain

b. foul and contains dissolved and suspended solids

c. as good as that from a complete treatment

d. none of these.

32. For treating the sewage of a large city, you will recommend

a. a sedimentation tank and an activated sludge treatment plant

b. a plant consisting of Imhoff tanks with low rate trickling filters

c. sedimentation tanks with high rate trickling filters

d. none of these.

33. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

34. If d1 is the depth of cutting, d2 is the height of the bank from bed level r2 : 1 and r1 : 1 are the slopes in filling and cutting respectively, the horizontal distance n between the bed and bank, is

a. x = r1 d1

b. x = r2 d2

c. x = d1 / r1

d. x = d2 / r2

35. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

36. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

37. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

38. Border Roads Organisation for hilly regions, was formed in

a. 1954

b. 1958

c. 1960

d. 1962

39. According to Highway Research Board of U.S.A. practical land width, is

a. 3.0 m

b. 3.3 m

c. 3.6 m

d. 4.5 m

40. While calculating the sight distances, the driver's eye above road surface, is assumed

a. 100 cm

b. 110 cm

c. 120 cm

d. 150 cm

41. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

42. The brick work is measured in sq metre, in case of

a. Honey comb brick work

b. Brick flat soling

c. Half brick walls or the partition

d. All the above.

43. The value of 'C' of Indian type W.C. shown in the given figure is :

a. 400 mm

b. 450 mm

c. 500 mm

d. 550 mm

44. A cement concrete road is 1000 m long, 8 m wide and 15 cm thick over the sub-base of 10 cm thick gravel. The box cutting in road crust is

a. 500 m3

b. 1000 m3

c. 1500 m3

d. 2000 m3

45. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

46. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

47. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

48. Pick up the correct statement from the following:

a. The duration of time by which an activity can be delayed without affecting the succeeding activity, is called free float

b. The float which affects neither the processor nor the successor activities, is called independent float

c. The difference between total float and free float, is called interfering float

d. All the above.

49. Pick up the correct statement from the following:

a. F.I.C. stands for flight information centre

b. Radius of control area is 160 km

c. Radius of control zone is 40 km

d. All the above.

50. In instrument landing system

a. L.O.M. and L.M.M. are installed on one side and Localizer antenna on the other side

b. L.O.M. and Localizer are installed on one side and L.M.M. on the other side

c. Localizer and L.M.M. are installed on one side and L.O.M. on the other side

d. None of these.