***Sub Engineer***

1. The longitudinal section of the surface of bubble tube is

a. straight

b. circular

c. parabolic

d. elliptic.

2. The systematic errors which persist and have regular effects in the performance of a survey operation, are due to

a. carelessness

b. faulty instrument

c. inattention

d. none of these.

3. You have to observe an included angle with better accuracy than what is achievable by a vernier, you will prefer the method of

a. repetition

b. reiteration

c. double observations

d. exactness.

4. Planimeter is used for measuring

a. area

b. contour gradient

c. slope angle

d. none of these

5. Pick up the correct statement from the following:

a. Attrition test is done to find out the rate of wear of stones which are used in road construction

b. Crushing test is done to find out the compressive strength of the stone

c. Impact test is done to determine toughness of a stone

d. All the above.

6. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

7. The most commonly used synthetic abrasive is

a. aluminium carbide

b. boric acid

c. silicon

d. All of these.

8. Lacquer paints

a. are less durable as compared to enamel paints

b. consist of resin and nitro-cellulose

c. contain alcohol as thinner

d. all the above.

9. A masonry dam (density = 20, 000 N/m3) 6 m high, one metre wide at the top and 4 m wide at the base, has vertical water face. The minimum stress at the base of the dam when the reservoir is full, will be

a. 75 N/m2

b. 750 N/m2

c. 7500 N/m2

d. 75000 N/m2

10. Maximum principal stress theory for the failure of a material at elastic point, is known

a. St. Venant's theory

b. Rankine's theory

c. Haig's theory

d. Von Mises's theory.

11. If the depth of a simply supported beam carrying an isolated load at its centre, is doubled, the deflection of the beam at the centre will be changed by a factor of

a. 1/2

b. 8

c. 1/8

d. 4

12. Stress in members of statically determinate simple frames, can be determined by

a. method of joints

b. method of sections

c. graphical solution

d. all the above.

13. The maximum vacuum created at the summit of a syphon is

a. 2.7 m of water

b. 7.4 m of water

c. 5.5 m of water

d. none.

14. The velocity of the fluid particle at the centre of the pipe section, is

a. minimum

b. maximum

c. equal throughout

d. none of these.

15. The main assumption of Bernoulli's equation is :

a. The velocity of energy of liquid particle, across any cross-section of a pipe is uniform

b. No external force except the gravity acts on the liquid

c. There is no loss of energy of the liquid while flowing

d. All the above.

16. In Chezy's formula V = C mi

a. V is the mean velocity of flow

b. m is the hydraulic mean depth

c. i is the loss of head per unit length of pipe

d. All the above.

17. The ratio of settlement at any time 't' to the final settlement, is known as

a. co-efficient of consolidation

b. degree of consolidation

c. consolidation index

d. consolidation of undisturbed soil.

18. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

19. Pick up the correct statement applicable to plate load test

a. At the centre of the test pit, a hole is dug out whose size is kept equal to the size of the test plate

b. Bottom level of the hole dug at the centre of the test pit, is kept at the level of the actual formation

c. Ratio of the depth of the hole to the width of the test plate is the same as the ratio of the depth of the actual formation to the width of the formation

d. All the above.

20. The factor which affects the compaction, is

a. compacting content

b. method of compaction

c. type of soil

d. All the above.

21. The maximum area of tension reinforcement in beams shall not exceed

a. 0.15%

b. 1.5%

c. 4%

d. 1%

22. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

23. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

24. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

25. The single stage well point system of dewatering an excavation can be used if the depth of excavation does not exceed

a. 5 m

b. 10 m

c. 15 m

d. 20 m

26. Black cotton soil is unsuitable for foundations because its

a. bearing capacity is low

b. permeability is uncertain

c. particles are cohesive

d. property to undergo a volumetric change due to variation of moisture content.

27. Efflorescence in cement is caused due to an excess of

a. iron oxide

b. silica

c. alkalis

d. magnesium oxide.

28. The void ratio of

a. single size coarse aggregate is roughly 0.45.

b. graded coarse aggregate is roughly 0.040

c. fine aggregate is roughly 0.45

d. all the above.

29. The maximum pressure which the pipe can withstand without any leakage during hydrostatic pressure test, is called

a. Working pressure

b. Design pressure

c. Test pressure

d. Hydrostatic pressure.

30. Pick up the correct statement from the following :

a. The odour of water sample is generally measured by a term called odour intensity

b. The colour of water sample may be detected by Nessler tube

c. Specific conductivity of water measures the amount of dissolved salt

d. All the above.

31. The non-clog pump which permits solid matter to pass out with the liquid sewage, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

32. The width of a rectangular sewer is twice its depth while discharging 1.5 m/sec. The width of the sewer is

a. 0.68 m

b. 0.88 m

c. 1.36 m

d. 1.76 m.

33. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

34. Which one of the following statements is correct ?

a. The canal system and the drainage system are complementary

b. The canal system runs on ridges, gets divided into smaller channels and spreads its water on the land

c. The drainage system runs along the lowest line, collects drainage water from the slopes and joins the main stream

d. All the above.

35. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

36. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

37. The width of the right of way, is influenced by

a. shoulders or berms

b. classification of road

c. availability of funds

d. all the above.

38. Normal formation width of a hill road for one-way traffic, is

a. 4.8 m

b. 6.6 m

c. 7.2 m

d. 8 m

39. The weight of vehicles affects

a. ruling gradient

b. limiting gradient

c. design of bridges

d. all the above.

40. The extra widening of pavements recommended by Indian Roads Congress for hill roads having radius 150 metres, is

a. 1.0 m

b. 0.5 m

c. 0.0 m

d. 2.0 m

41. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

42. The floor area includes the area of the balcony up to

a. 100%

b. 75%

c. 50%

d. 25%

43. Pick up the excavation where measurements are made in square metres for payment.

a. surface dressing up to 15 cm depths

b. Surface excavation up to 30 cm depths

c. Both (b) and (c)

d. Both (a) and (b)

44. The area of a sloping surface of a protective embankment of mean height d, side slopes S : 1 and length L is

a. d x d x s

b. d2 x (ds)2

c. L.D 1 + s2

d. 2 LD 1 + s2

45. In the given figure, the network of a project represents

a. activity of an excavation of a footing

b. activity of an excavation which starts at event No. 1 and ends at even No. 2

c. activity of excavation which takes 8 units of time

d. none of these.

46. For the execution of a project, a contractor is

a. a person

b. a firm

c. an agency

d. all the above.

47. Pick up the correct statement from the following:

a. PERT analysis is event oriented

b. CPM does not make any allowance for the uncertainties in the duration of time

c. In CPM, the time is related to cost

d. All the above.

48. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

49. The bearing of the runway at threshold is 290°, the runway number is

a. N 70° W

b. 290°

c. 29°

d. W 20° N

50. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.