***Sub Engineer***

1. The back staff reading on a B.M. of R.L. 500.000 m is 2.685 m. If foresight reading on a point is 1.345 m, the reduced level of the point, is

a. 501.345 m

b. 501.340 m

c. 504.030 m

d. 502.585 m.

2. The 'fix' of a plane table station with three known points, is bad if the plane table station lies

a. in the great triangle

b. outside the great triangle

c. on the circumference of the circumscribing circle

d. none of these.

3. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

4. Magnetic bearing of a survey line at any place

a. remains constant

b. changes systematically

c. varies differently in different months of the year

d. is always greater than true bearing.

5. Linseed oil is used in paints as

a. thinner

b. vehicle

c. base

d. drier.

6. Pick up the correct statement from the following:

a. Quick lime is obtained by burning pure lime stone:

b. Hydraulic lime is obtained by burning lime stone containing clay 5% to 30%

c. Poor lime is obtained by burning lime stone containing impurities more than 5%

d. All the above.

7. Varnish is a transparent or semi-transparent solution of resinuous substances in

a. alcohol

b. linseed

c. turpentine

d. all the above.

8. The main constituent of fly-ash, is

a. aluminium oxide

b. silica

c. ferrous oxide

d. All of these.

9. A rolled steel joist is simply supported at its ends and carries a uniformly distributed load which causes a maximum deflection of 10 mm and slope at the ends of 0.002 radian. The length of the joist will be,

a. 12 m

b. 14 m

c. 16 m

d. 18 m

10. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

11. The nature of the stress in horizontal members of the truss shown in below figure may be

a. tensile

b. shear

c. zero

d. all the above.

12. A three hinged parabolic arch hinged at the crown and springings, has a horizontal span of 4.8 m and a central rise of 1 m. It carries a uniformly distributed load of 0.75 tonne per metre over half left hand span. The horizontal thrust at the support will be

a. 1.08 tonnes

b. 1.8 tonnes

c. 0.8 tonnes

d. none of these.

13. The Empirical formula  for discharge over large rectangular weirs, is known as

a. Francis formula

b. Bazin formula

c. Rehbook formula

d. Kutter's formula.

14. In pipe lines, a surge tank is provided

a. to relieve the pressure due to water hammer

b. to provide additional water head

c. to overflow the pipe line when suddenly closed

d. to remove the frictional loss in pipe.

15. For maximum horse power of a nozzle, the head supplied must be equal to

a. head loss in the pipe due to friction

b. twice the head loss in the pipe due to friction

c. thrice the head loss in the pipe due to friction

d. four times the head loss in the pipe due to friction.

16. The magnitude of water hammer in a pipe depends upon

a. length of the pipe line

b. elastic properties of the pipe material

c. elastic properties of the following liquid

d. all the above.

17. A soil sample has passing 0.075 mm sieve = 60% liquid limit = 65% and plastic limit = 40%. The group index of the soil, is

a. 5

b. 20

c. 40

d. none of these.

18. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

19. The capillary rise of water

a. depends upon the force responsible

b. increases as the size of the soil particles increases

c. decreases as the size of the soil particles decreases

d. is less in wet soil than in dry soil.

20. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

21. Post tensioning system

a. was widely used in earlier days

b. is not economical and hence not generally used

c. is economical for large spans and is adopted now a days

d. none of these.

22. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

23. A foundation rests on

a. base of the foundation

b. subgrade

c. foundation soil

d. both (b) and (c)

24. The design of a retaining wall assumes that the retained earth

a. is free from moisture

b. is not cohesives

c. consists of granular particles

d. all the above.

25. During percussion drilling

a. ground water observations are hindered due to entry of the slurry in the soil below the bottom of the hole

b. caving or mixing of strata are caused in soft soils or cohesionless soils

c. the soil to a considerable depth below the bottom of the hole gets disturbed

d. all the above.

26. Pick up the consideration to the taken while designing a hospital from the following :

a. the operation theatre unit to be detached as it requires sterlized zone but near the ward for the patients and doctor

b. the mortuary should be detached from the main circulation with a postmortem room

c. causalty unit should be provided a separate entrance

d. all the above.

27. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

28. An aggregate is said to be flaky if its least dimension is less than

a. 2/5th of mean dimension

b. 3/5th of mean dimension

c. 4/5th of mean dimension

d. none of these.

29. Average annual rainfall at any station is the average of annual rainfall over a period of

a. 14 years

b. 21 years

c. 28 years

d. 35 years.

30. Surface water is obtained from

a. well

b. springs

c. artesian well

d. rain.

31. Pick up the incorrect statement from the following :

a. The process of decomposing the organic matter under controlled anaerobic conditions, is called sludge digestion

b. Sludge digestion is carried out in sludge tank

c. The gases produced in sludge digestion process, contain 75% carbon dioxide

d. The gases produced in sludge digestion process, contain 75% methane.

32. The disintegrating pump in which solid matter is broken up before passing out, is

a. centrifugal pump

b. reciprocating pump

c. pneumatic ejector

d. none of these.

33. A deficit of sediments in flowing water may cause a river

a. meandering type

b. aggrading type

c. degrading type

d. sub-critical type.

34. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

35. The useful moisture of soil, is equal to its

a. field capacity

b. saturation capacity

c. moisture content at permanent wilting point

d. difference between filed capacity and permanent wilting point within the root zone of plants.

36. Pick up the incorrect statement from the following:

a. In free flooding irrigation, water is admitted at one corner of a field and is allowed to spread over the entire area

b. In check method of irrigation, the field is divided into smaller compartments and water is admitted to each in turn

c. In furrow irrigation water is admitted between the rows of plants in the field

d. None of these.

37. The pavement width of a road depends upon

a. terrain

b. type of traffic

c. number of lanes

d. all the above.

38. The head light of vehicles should be such that its lower beam illuminates objects at

a. 20 m

b. 30 m

c. 40 m

d. 50 m

39. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

40. Pick up the correct statement from the following:

a. The height of head light above road surface, is taken as 0.75 m

b. The beam of head light is up to one degree upwards from the grade of road

c. While designing the valley curve, the height of the object is assumed nil

d. All the above

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. The reduced levels of points, 30 metres apart along the longitudinal section of a road portion between chainages 5 and 9 are shown in the given figure. If there is a uniform up-gradient of the road 120 in 1, the chainage of the point with no filling or cutting is

a. (6 + 15) chains

b. (6 + 12) chains

c. (6 + 18) chains

d. None of these.

43. Pick up the correct statement from the following:

a. Area of canal in cutting = BD + Sd2 where B = bed width, d = depth of cutting and S is the side slope

b. Area of the bank of canal = B1d1 + Sd2 where B1, d1 and S are the width of bank, height of the bank above N.S.L. and side slope respectively

c. If F.S.L. is above N.S.L the canal is called partly in cutting and partly in filling and berms are designed as 2d where d is full supply depth

d. All the above.

44. Pick up the correct statement regarding the centre line method of estimating a building

a. Product of the centre line of the walls and area of cross-section of any item, gives total quantity of the item

b. The centre line is worked out separately for different sections of walls of a building

c. The centre line length is reduced by half the layer of main wall joining the partition wall

d. All the above.

45. Construction team means

a. an architect

b. an owner

c. a contractor

d. all the above.

46. Henry Gantt developed Bar charts for planning and scheduling of projects in

a. 1900

b. 1920

c. 1940

d. 1950

47. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

48. Completion of an activity on CPM network diagram, is generally known

a. Event

b. Node

c. Connector

d. All the above.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. The lift off distance is the distance along the centre of the runway between the starting point and

a. end of the runway

b. end of stopway

c. point where air craft becomes air borne

d. point where air craft attains a height of 10.7 m