***Sub Engineer***

1. Pantagraph is used for

a. measuring distances

b. measuring areas

c. enlarging or reducing plans

d. setting out right angles

2. An internal focussing type surveying telescope, may be focussed by the movement of

a. objective glass of the telescope

b. convex-lens in the telescope

c. concave lens in the telescope

d. plano-convex lens in the telescope.

3. Imaginary line passing through points having equal magnetic declination is termed as

a. isogon

b. agonic line

c. isoclinic line

d. none of these.

4. The horizontal angle between true meridian and magnetic meridian, is known

a. bearing

b. magnetic declination

c. dip

d. convergence.

5. Lime stones are generally known as

a. aqueous rocks

b. sedimentary rocks

c. stratified rocks

d. all the above.

6. The portion of the brick without a triangular corner equal to half the width and half the length, is called

a. closer

b. queen closer

c. king closer

d. squint brick.

7. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

8. Pick up the correct statement from the following:

a. Solder material is an alloy which melts at a temperature above 400°C

b. Brazing is done at temperature above 600°C to 1100°C

c. Brazing joint is stronger than the solder joint

d. all the above.

9. A material which obeys Hook's law, is subjected to direct stress σ0. At its elastic limit, the following statement is true,

a. Maximum shear stress =

b. Strain energy =

c. Shear strain energy =

d. All the above.

10. The forces acting normally on the cross section of a bar shown in the given figure introduce

a. compressive stress

b. tensile stress

c. shear stress

d. none of these.

11. A shaft turning 150 r.p.m. is subjected to a torque of 150 kgm. Horse power transmitted by the shaft is

a. π

b. 10 π

c. π2

d. 1/π

12. A steel rod of 2 cm diameter and 5 metres long is subjected to an axial pull of 3000 kg. If E = 2.1 x 106, the elongation of the rod will be

a. 2.275 mm

b. 0.2275 mm

c. 0.02275 mm

d. 2.02275 mm.

13. In an inclined pipe, the pressure difference at its two ends is due to

a. exit head drop

b. frictional loss head

c. elevation head

d. all the above.

14. Discharge with velocity of approach, over a rectangular weir is

a.  Cd x L x 2g [(H + ha)3/2 + ha3/2]

b.  Cd x L x 2g [(H + ha)3/2 - ha3/2]

c.  Cd x L x 2g [(H - ha)3/2 + ha3/2]

d.  Cd x L x 2g [(H - ha)3/2 - ha3/2]

15. Hydraulic ram is a device

a. for lifting water without an electric motor

b. for accelerating water flow

c. for lifting heavy loads

d. none of these.

16. The notch angle for maximum discharge over a triangular notch, is

a. 30°

b. 60°

c. 90°

d. 120°

17. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

18. A plane inclined at an angle φ to the horizontal at which the soil is expected to stay in the absence of any lateral support, is known as

a. natural slope line

b. repose line

c. the φ line

d. all the above.

19. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

20. Pick up the correct statement from the following:

a. A soil classification system is meant to provide an accepted and systematic method of describing the various types of soils eliminating personal factors

b. The first category of soil classification is based on grain size of the soil

c. The second category of soil classification is based on fine as well as coarse grains

d. All the above.

21. The maximum diameter of a bar used in a ribbed slab, is

a. 6 mm

b. 20 mm

c. 22 mm

d. 24 mm

22. Enlarged head of a supporting column of a flat slabs is technically .known as

a. top of the column

b. capital

c. drop panel

d. none of these.

23. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

24. An under-reinforced section means

a. Steel is provided at the under side only

b. Steel provided is insufficient

c. Steel provided on one face only

d. Steel will yield first.

25. The minimum width of a stair in residential buildings, is

a. 70 cm

b. 85 cm

c. 100 cm

d. 120 cm.

26. The type of pointing in which upper side of mortar joints is kept about 12 mm inside the face of the masonry and bottom is kept flushed with face of wall, is

a. truck pointing

b. recessed pointing

c. struck pointing

d. grooved pointing.

27. An aggregate is said to be flaky, if its least dimension is less than

a. 3/4 mean dimension

b. 3/5 mean dimension

c. 5/8 mean dimension

d. none of these.

28. The most useless aggregate is one whose surface texture is

a. smooth

b. granular

c. glassy

d. honey combed and porous.

29. The nitrate concentration in domestic water supplies, is generally limited to

a. 15 ppm

b. 25 ppm

c. 30 ppm

d. 45 ppm.

30. For 3.25 x 10-2 cumecs discharging from a well, a pump is installed to lift water against a total head of 30 m. The minimum required horse power, is

a. 15 H.P.

b. 18 H.P.

c. 20 H.P.

d. 25 H.P.

31. The clarigesters are

a. circular septic tanks

b. rectangular septic tanks

c. circular Imhoff double tanks with bottom hoppers

d. circular Imhoff double storey tanks without bottom hoppers.

32. If the depletion of oxygen is found to be 5 ppm after incubating a 2.5% solution of sewage sample for 5 days at 21°C, B.O.D. of the sewage is

a. 100 ppm

b. 150 ppm

c. 200 ppm

d. 250 ppm.

33. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

34. Lane's weighted creep theory assumes

a. double weightage to horizontal creep and one weightage to vertical creep

b. triple weightage to horizontal creep and one weightage to vertical creep

c. triple weightage to vertical creep and one weightage to horizontal creep

d. double weightage to vertical creep and one weightage to horizontal creep.

35. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

36. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

37. The basic formula for determination of pavement thickness was first suggested by

a. Spanglar

b. Picket

c. Kelly

d. Goldbeck

38. The shape of a vertical curve, is

a. parabolic

b. elliptical

c. circular

d. spiral

39. Full amount of extra width of a pavement on the curve, is provided at

a. beginning of the transition curve

b. centre of the transition curve

c. beginning of the circular curve

d. centre of the circular curve.

40. In cement concrete pavements, tensile stress is due to :

a. bending or deflection under wheel loads

b. difference in temperature of the top and bottom of pavement

c. contraction of slab during falling temperature

d. all the above.

41. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

42. Pick up the item of work not included in the plinth area estimate

a. Room area

b. Verandah area

c. W.C. area

d. Courtyard area.

43. The diameter of a domestic sewer pipe laid at gradient 1 in 100 is recommended

a. 100 mm

b. 150 mm

c. 200 mm

d. 175 mm

44. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

45. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

46. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

47. Critical path lies along the activities having total float

a. positive

b. negative

c. zero

d. same.

48. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

49. For Class A Air port the difference of reduced levels of higher and lower edges of the conical surface, is

a. 25 m

b. 50 m

c. 75 m

d. 100 m

50. The maximum length and pavement strength of the runway is that of

a. A 1

b. B 2

c. B 3

d. G 7