***Sub Engineer***

1. The angle between two plane mirrors of optical square, is

a. 30°

b. 45°

c. 60°

d. 90°.

2. Contour lines of different elevations can unite to form one line, only in the case of

a. a vertical cliff

b. a saddle

c. a water shed line

d. a hill top.

3.  is a conventional sign of

a. mosque

b. temple

c. church

d. idgah.

4. Pick up the incorrect statement from the following :

a. while measuring a distance with a tape of length 100.005 m, the distance to be increasing by 0.005 m for each tape length

b. an increase in temperature causes a tape to increase in length and the measured distance is too large

c. the straight distance between end points of a suspended tape is reduced by an amount called the sag correction

d. a 100 m tape of cross section 10 mm x 0.25 mm stretches about 10 mm under 5 kg pull.

5. The cracks which extend from bark towards the sap wood in the cross section of a tree, are called

a. radial shakes

b. star shakes

c. heart shakes

d. cup shakes.

6. Pick up the correct statement from the following:

a. The low voltage porcelain is prepared by wet process

b. The high voltage porcelain is prepared by dry process

c. The low voltage porcelain is prepared by dry process

d. None of the above.

7. Pick up the synthetic resin from the following:

a. Phenolic resin

b. Resorcinol resin

c. Urea resin

d. All of these.

8. Formula for quick lime, is

a. CaCO3

b. CaO

c. CO3CO2

d. none to these.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. A material is said to be perfectly elastic if

a. it regains its original shape on removal of the load

b. It regains its original shape partially on removal of the load

c. it does not regain its original shape at all

d. none of these.

11. The value of Poisson's ratio always remains

a. greater than one

b. less than one

c. equal to one

d. none of these.

12. Along the principal plan subjected to maximum principal stress

a. maximum shear stress acts

b. minimum shear stress acts

c. no shear stress acts

d. none of these.

13. For an irrotational flow, the equation  is given by

a. Cauchy-Riemann

b. Reynold

c. Laplaces

d. Bernoulli.

14. The property of steam function ψ is :

a. the flow around any path in the fluid is zero for continuous flow

b. the rate of change of ψ with distance in an arbitrary direction, is proportional to the component of velocity normal to that direction

c. the velocity vector may be found by differentiating the stream function

d. all the above.

15. A non-uniform steady flow is through

a. a long tube at a decreasing rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

16. In flow, the liquid particles may possess

a. potential energy

b. kinetic energy

c. pressure energy

d. all the above.

17. Pick up the correct statement from the following:

a. A soil having pH value more than 7 is an acidic soil

b. A soil having pH value less than 7 is an acidic soil

c. A soil having pH value more than 7 is an alkaline soil

d. A soil containing chemicals for the manufacture of portland cement is preferred.

18. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

19. A soil mass is said to be in plastic equilibrium if

a. it is stressed to maximum

b. it is on the verge of failure

c. it is in plastic stage

d. it starts flowing.

20. If W1, W2, W3 and W4 are the sequential weights obtained during observations in pycnometer method for determining water content, the formula to be used, is

a. W =

b. W =

c. W =

d. W =

21. The anchorage value of a hook is assumed sixteen times the diameter of the bar if the angle of the bend, is

a. 40°

b. 45°

c. 60°

d. all the above.

22. The maximum area of tension reinforcement in beams shall not exceed

a. 0.15%

b. 1.5%

c. 4%

d. 1%

23. For normal cases, stiffness of a simply supported beam is satisfied if the ratio of its span to its overall depth does not exceed

a. 15

b. 20

c. 25

d. 30

24. A ribbed slab is provided for

a. a plain ceiling

b. thermal insulation

c. acoustic insulation

d. all the above.

25. The window which is provided in flat roof of a room, is known

a. dormer window

b. lantern window

c. louvered window

d. sky window.

26. Weep holes are provided in retaining and breast walls

a. to drain off the water from the filling

b. to ventilate the stone masonry

c. to add architectural beauty

d. to reduce the weight of the earth retained

27. Pick up the correct statement from the following:

a. Sand obtained from flooded pits, need not be washed before use

b. Sea shore sand contains chlorides which cause effloresance

c. The chloride in sea shore sand and shingle may cause corrosion of reinforcement if the concrete is porous

d. All the above.

28. If the various concrete ingredients i.e. cement, sand and aggregates are in the ratio of 1:3:6, the grade of concrete, is

a. M 100

b. M 150

c. M 200

d. M 250

29. For least effect on the water table, the tube wells must be dug one in every

a. 0.75 sq km

b. 1.0 sq km

c. 1.25 sq km

d. 1.5 sq km.

30. The maximum non-verticality of the bore of a well 200 m deep, may be permitted up to

a. 50 cm

b. 75 cm

c. 100 cm

d. 150 cm.

31. The digested sludge from septic tanks, is removed after a maximum period of

a. 3 years

b. 3.5 years

c. 4 years

d. 5 years.

32. The detention time (t) of a settling tank, may be defined as the time required for

a. a particle to travel along its length

b. a particle to travel from top surface to bottom sludge zone

c. the flow of sewage to fill the tank

d. none of these.

33. Bligh's theory of seepage assumes

a. equal weightage to the horizontal and vertical creep

b. more weightage to horizontal creep than vertical creep

c. less weightage to horizontal creep than vertical creep

d. loss of head follows the sine curve.

34. The main cause of silting up a channel,

a. inadequate slope

b. defective head regulator

c. defective outlets

d. all the above.

35. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

36. For the conditions enumerated to provide a crossing at C1 You will probably provide

a. an aqueduct

b. a super-passage

c. a syphon aqueduct

d. none of these.

37. If R is the radius of a main curve and L is the length of the transition curve, the shift of the curve, is

a. L2/24 R

b. L3/24 R

c. L4/24 R

d. L/12 R

38. The distance travelled by a moving vehicle during perception and brake reaction times, is known as

a. sight distance

b. stopping distance

c. lag distance

d. none of these.

39. Design of flexible pavements is based on

a. mathematical analysis

b. empirical formulae

c. a compromise of pure theory and pure empirical formula

d. none of these.

40. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. The rate of payment is made for 100 cu m (per % cu m) in case of

a. Rock cutting

b. Excavation in trenches for foundation

c. Earth work in filling the plinth

d. all the above.

43. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

44. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. Pick up the correct statement from the following:

a. The difference of latest occurence time and earliest expected time, is called slack

b. The activities connecting the events having zero slack, lie on the critical path

c. The critical path consumes the maximum time

d. All the above.

47. In the given figure, the network of a project represents

a. activity of an excavation of a footing

b. activity of an excavation which starts at event No. 1 and ends at even No. 2

c. activity of excavation which takes 8 units of time

d. none of these.

48. Frederick W. Taylor introduced a system of working known as

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

49. Pick up the incorrect statement from the following:

a. L.M.M. is installed at 1.0 km from the threshold

b. Localizer antenna is installed at 300 m from the other end of the runway

c. Glide slope antenna is installed at the centre of the runway about 150 m on one side

d. None of these.

50. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10