***Sub Engineer***

1. Prismatic compass is considered more accurate than a surveyor's compass, because

a. it is provided with a sliding glass in the object vane

b. its graduations are in whole circle bearings

c. it is provided with a prism to facilitate reading of its graduated circle

d. both (c) and (d).

2. Bergchrund is a topograhical feature in

a. plains

b. water bodies

c. hills

d. glaciated region

3. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

4. Reduced bearing of a line is an angle between

a. north line and given line measured clockwise

b. north line and given line measured anticlockwise

c. east or west and the given line

d. given line and the part of the meridian whether N end or S end, lying adjacent to it.

5. According to IS 399-1963, the weight of the timber is specified at

a. 8% moisture content

b. 10% moisture content

c. 12% moisture content

d. 14% moisture content.

6. Bitumen in

a. solid state, is called asphalt

b. semi fluid state, is called mineral tar

c. fluid state, is called petroleum

d. all the above.

7. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

8. The cement becomes unsound by the presence of excess

a. sulphur

b. magnesia

c. lime

d. All of these.

9. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

10. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

11. According to Unwin's formula, the diameter d of a rivet of plate of thickness t is :

a. d = 6.05 t

b. d = 1.5 t + 4

c. d = 5 t

d. d = t + 1.5

12. The neutral axis of a beam cross-section must

a. pass through the centroid of the section

b. be equidistant from the top of bottom films

c. be an axis of symmetry of the section

d. none of these.

13. If H is the difference of liquid levels on two sides of an orifice (width b, depth d), the discharge through the orifice will be

a. Cdbd22gH

b. Cdb2d2gH

c. Cdbd2gh

d. Cdbd.2g H1/2.

14. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

15. A rectangular channel 6 m wide and 3 m deep and having a bed slope as 1 in 2000 is running full. If Chezy's constant C = 54.8, pick up the correct specification of the channel from the following :

a. hydraulic mean depth = 1.5 m

b. Velocity of flow = 1.5 m/sec

c. Rate of flow = 27 m3/sec

d. All the above.

16. Pick up the incorrect statement from to following regarding triangular notch :

a. For measuring low discharge, it gives more accurate result

b. Only one reading (i.e. H) is required for computation of discharge

c. Ventilation is necessary

d. None of these.

17. Pick up the correct statement from the following:

a. When stress decreases, void, ratio decreases

b. When stress decreases, coefficient of permeability decreases

c. When stress decreases, coefficient of volume change decreases

d. When stress decreases void ratio, co-effi-cients of per-meability and volume change decrease.

18. The minimum water content at which the soil retains its liquid state and also possesses a small shearing strength against flowing, is known

a. liquid limit

b. plastic limit

c. shrinkage limit

d. permeability limit.

19. Pick up the incorrect definition from the following:

a. The rotation of soil particles into stable state while remoulding, is known as the thiostropy of soil

b. The water content at which a soil changes from the liquid state to solid state, is known liquid limit of the soil

c. The water content at which a soil flows, is known plastic limit of the soil

d. None of these.

20. Pick up the correct statement from the following :

a. Liquid limit minus plastic limit, is known as plasticity index of the soil

b. Plastic limit minus shrinkage limit, is known as shrinkage index of the soil

c. The ratio of the plasticity index to the flow limit, is known as toughness index of the soil

d. All the above.

21. The maximum ratio of span to depth of a cantilever slab, is

a. 10

b. 12

c. 14

d. 16

22. A foundation is called shallow if its depth, is

a. half of its width

b. three-fourth of its width

c. equal to its width

d. all the above.

23. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

24. On an absolutely rigid foundation base, the pressure will

a. be more at the edges of the foundation

b. be uniform

c. not be uniform

d. be zero at the centre of the foundation.

25. For providing a raft foundation, the following activities are involved ramming the foundation bedexcavation of the soil upto required depthlaying the reinforcement over the foundation bedcuring the cement concrete placed over reinforcementpouring the cement concrete over the reinforcement. The correct sequence is

a. 1, 2, 3, 4, 5

b. 5, 4, 3, 2, 1

c. 2, 1, 3, 5, 4

d. 3, 2, 5, 1, 4.

26. An arch constructed with finely dressed stones, is known

a. ashlar arch

b. rubble arch

c. gauged arch

d. axed arch.

27. Under normal conditions using an ordinary cement, the period of removal of the form work, is :

a. 14 days for bottom slabs of spans 4.6 m and more

b. 21 days for bottom beams over 6 m spans

c. 2 days for vertical sides of columns

d. all the above.

28. For preparing ordinary concrete, the quantity of water used, is

a. 10% by weight of aggregates plus 10% of weight of cement

b. 5% by weight of aggregates plus 30% of weight of cement

c. 30% by weight of aggregates plus 10% of weight of cement

d. none of these.

29. The ensure proper growth of children's teeth, the quantity of flouride used in water mains, is

a. 1 mg/litre

b. 2 mg/litre

c. 3 mg/litre

d. 4 mg/litre

30. The capability of a soil mass of full width and depth to transmit water, is known

a. porosity

b. permeability

c. transmissibility

d. none of these.

31. A manhole is classified as shallow if its depth is between

a. 0.5 to 0.7 m

b. 0.7 to 0.9 m

c. 0.9 to 1.20 m

d. 1.2 to 1.50 m.

32. If the flame of a miner's safety lamp in the upper layers of the sewer forms an explosive, the sewer certainly contains

a. Carbon dioxide

b. Methane

c. Oxygen

d. None of these.

33. The discharge over a trapezoidal crest of a Sarda fall with free over fall conditions, is

a. 0.45 2g . LH3/2 (H/B)1/3

b. 0.45 2g . LH2/3 (H/B)2/3

c. 0.45 2g . LH3/2 (H/B)1/6

d. 0.45 2g . LH1/6 (H/B)3/2

34. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

35. For a standing crop, the consumptive use of water is equal to the depth of water

a. transpired by the crop

b. evaporated by the crop

c. transpired and evaporated by the crop

d. used by the crop in transpiration, evaporation and also the quantity of water evaporated from adjacent soil.

36. When a canal and a drainage approach each other at the same level, the structure so provided, is

a. an aqueduct

b. a syphon

c. a level crossing

d. inlet and outlet.

37. If cross slope of a country is upto 10% the terain is classified as

a. plain

b. rolling

c. mountainous

d. steep.

38. Gradient resistance of moving vehicles along down slopes, is

a. + 7 kg/tonne

b. + 9 kg/tonne

c. - 9 kg/tonne

d. - 7 kg/tonne

39. Degree of a road curve is defined as the angle in degrees subtended at the centre by an arc of

a. 20 metres

b. 25 metres

c. 30 metres

d. 50 metres

40. The period of long term plan for the development of roads in India, known as Bombay Plan (Aug. 1958), is

a. 10 years

b. 15 years

c. 20 years

d. 25 years

41. Pick up the incorrect statement from the following:

a. Dimensions are measured to the nearest 0.01 m

b. Areas are measured to the nearest 0.01 sq.m

c. Cubic contents are measured to the nearest 0.1 cm m

d. Weights are measured to the nearest 0.001 tonnes

42. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

43. The expected out turn of cement concrete 1 : 2 : 4 per mason per day is

a. 1.5 m3

b. 2.5 m3

c. 3.5 m3

d. 5.0 m3

44. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

45. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

46. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

47. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

48. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

49. The fuse large includes

a. passengers chamber

b. pilot's cabin

c. tail of aircraft

d. all the above.

50. For the proposed air port, the survey project provides

a. topograhic plan

b. grading plan

c. drainage plan

d. all the above.