***Sub Engineer***

1. A lemniscate curve will not be transitional throughout, if its deflection angle, is

a. 45°

b. 60°

c. 90°

d. 120°

2. True meridian of different places

a. converge from the south pole to the north pole

b. converge from the north pole to the south pole

c. converge from the equator to the poles

d. run parallel to each other.

3. Two contour lines, having the same elevation

a. cannot cross each other

b. can cross each other

c. cannot unite together

d. can unite together.

4. In optical reading instruments

a. the vertical circle is usually continuous from 0° to 359°

b. the readings increase when the telescope is elevated in the face left position

c. the readings decrease when the telescope is elevated in the face right position

d. all the above.

5. The most commonly used base for timber painting, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

6. The timber having maximum resistance against white ants, is obtained from

a. chir

b. shisham

c. sal

d. teak.

7. Asbestos cement

a. is brittle

b. warps due to changes in humidity

c. strength is lowered when saturated by water

d. all the above.

8. For slaking of 10 kg of CaO, the theoretical amount of water is

a. 2.2 kg

b. 1.5 kg

c. 3.2 kg

d. None of these.

9. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

10. A shaft rotating N.R.M. under a torque T, transmits a power

a.  Newton metres/sec

b.  Newton metres/min

c.  Newton metres/min

d.  Newton metres/sec

11. For the beam shown in below figure, the maximum positive bending moment is nearly equal to negative bending moment when L1 is equal to

a. 1.0 L

b. 0.7 L

c. 0.5 L

d. 0.35 L.

12. The section modulus of a rectangular light beam 25 metres long is 12.500 cm3. The beam is simply supported at its ends and carries a longitudinal axial tensile load of 10 tonnes in addition to a point load of 4 tonnes at the centre. The maximum stress in the bottom most fibre at the mid span section, is

a. 13.33 kg/cm2 compressive

b. 26.67 kg/cm2 tensile

c. 26.67 kg/cm2 compressive

d. none of these.

13. The pressure less than atmospheric , pressure, is known

a. suction pressure

b. vacuum pressure

c. negative gauge pressure

d. all the above.

14. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

15. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

16. The metacentric height of a body equals the distance between

a. the centre of gravity and centre of buoyancy

b. the metacentre and centre of gravity

c. the centre of buoyancy and metacentre

d. none of these.

17. A flow net may be utilised for the determination of

a. seepage

b. hydrostatic pressure

c. seepage pressure

d. all the above.

18. The weight of a pycnometer containing 400 g sand and water full to the top is 2150 g. The weight of pycnometer full of clean water is 1950 g. If specific gravity of the soil is 2.5, the water content is

a. 5%

b. 10%

c. 15%

d. 20%

19. The clay soil mainly consists of

a. Montomorillonite

b. Illites

c. Vermiculite

d. All the above.

20. The compressibility of clays, is caused due to:

a. expulsion of double layer water from in between the grains

b. sliping of particles to new positions of greater density

c. bending of particles as elastic sheets

d. all the above.

21. If the maximum shear stress at the end of a simply supported R.C.C. beam of 6 m effective span is 10 kg/cm2, the share stirrups are provided for a distance x from either end where x is

a. 50 cm

b. 100 cm

c. 150 cm

d. 200 cm

22. Total pressure on the vertical face of a retaining wall of height h acts parallel to free surface and from the base at a distance of

a. h/4

b. h/3

c. h/2

d. 2h/3

23. To ensure uniform pressure distribution, the thickness of the foundation, is

a. kept uniform throughout

b. increased gradually towards the edge

c. decreased gradually towards the edge

d. kept zero at the edge.

24. The load stress of a section can be reduced by

a. increasing the total perimeter of bars

b. replacing larger bars by greater number of small bars

c. replacing smaller bars by greater number of greater bars

d. none of these.

25. In places where the soil is soft and has small resistance to the flow of concrete, which one of the following types of piles, is used

a. vibro pile

b. pressure pile

c. Franki pile

d. pedestal pile.

26. You are asked to design and supervise a truss for a factory to have spans 6 m to 9 m. The type of the truss you will use, is

a. queen post truss

b. king post truss

c. collar truss

d. none of these.

27. Pick up the correct statement from the following:

a. Cement should be mixed for at least one minute

b. 10% of water is placed in the rotating drum before adding dry material

c. 10% of water is added after placing the other ingredients in the drum

d. All the above.

28. Concrete gains strength due to

a. chemical reaction of cement with sand and coarse aggregates

b. evaporation of water from concrete

c. hydration of cement

d. All the above.

29. Grade aqueducts are not allowed to run

a. full

b.  th full

c.  full

d.  th full.

30. A water channel supported above the ground over trestles, is generally called

a. flume

b. canal

c. adueduct

d. tunnel

31. Sewer pipes need be checked for

a. minimum flow

b. maximum flow

c. both (a) and (b)

d. none of these.

32. The width of a settling tank with 2 hour detention period for treating sewage 378 cu m per hour, is

a. 5.5 m

b. 6 m

c. 6.5 m

d. 7 m.

33. Borrow pits should preferably be located in

a. field on the left side of the canal

b. field on the right side of the canal

c. fields on both sides of the canal

d. central half width of the section of the canal.

34. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

35. Pick up the correct statement from the following:

a. If the setting of an outlet is higher than that required for proportionality, the outlet is hyper-proportional

b. If the setting of an outlet is lower than that required for proportionality, the outlet is sub-proportional

c. If the flexibility is zero, it is a rigid module

d. All the above.

36. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

37. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

38. The minimum value of camber provided for thin bituminous surface hill roads, is

a. 2.2%

b. 2.5%

c. 3.0%

d. 3.5%

39. Pick up the incorrect statement from the following:

a. Tresguet did not provide the top camber for the drainage of surface water

b. Tresguet provided the top camber for the drainage of surface water

c. Telford provided two layers of stones in the central 5.4 m width and one layer was provided on the sides

d. Macadam provided a camber to the formation at the dug-up state, to drain percolated water.

40. Following type of pavement is generally known as flexible pavement

a. water-bound macadam roads

b. stabilised soil roads

c. road constructed with various layers of building material well compacted

d. all the above.

41. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

42. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

43. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

44. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

47. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

48. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

49. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)

50. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)