***Sub Engineer***

1. The smaller horizontal angle between the true meridian and a survey line, is known

a. declination

b. bearing

c. azimuth

d. dip.

2. Under ordinary conditions, the precision of a theodolite traverse is affected by

a. systematic angular errors

b. accidental linear errors

c. systematic linear errors

d. accidental angular errors.

3. The surface of zero elevation around the earth, which is slightly irregular and curved, is known as

a. mean sea level

b. geoid surface

c. level surface

d. horizontal surface.

4. Pick up the correct statement from the following :

a. A level line lies in level surface

b. A horizontal surface is normal to the direction of gravity at only one point

c. A horizontal line is tangential to the level surface

d. All the above.

5. Pick up the volcanic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

6. A badly mixed cement concrete results in

a. segregation

b. bleeding

c. honey combing

d. none to these.

7. Bitumen completely dissolves in

a. chloroform

b. benzol

c. coaltar

d. All of these.

8. The presence of sand in brick earth prevents:

a. cracking of bricks

b. shrinkage of bricks

c. warping of bricks

d. none of these.

9. A steel bar 5 m x 50 mm is loaded with 250, 000 N. If the modulus of elasticity of the material is 0.2 MN/mm2 and Poisson's ratio is 0.25, the change in the volume of the bar is :

a. 1.125 cm3

b. 2.125 cm3

c. 3.125 cm3

d. 4.125 cm2

10.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

12. A member is balanced at its end by two inclined members carrying equal forces. For equilibrium the angle between the inclined bars must be

a. 45°

b. 60°

c. 90°

d. 120°

13. The maximum vacuum created at the summit of a syphon is

a. 2.7 m of water

b. 7.4 m of water

c. 5.5 m of water

d. none.

14. The gases are considered incompressible if Match number is

a. equal to 1.0

b. equal to 1.5

c. is more than 0.5

d. less than 0.2

15. Fluids change the volume under external presssure due to

a. plasticity

b. viscosity

c. compressibility

d. none of these.

16. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

17. The specific gravity of quartz, is

a. 2.65

b. 2.72

c. 2.85

d. 2.90

18. The angle of internal friction is maximum for

a. angular-grained dense sand

b. round-grained dense sand

c. round-grained loose sand

d. clays.

19. If drainage is permitted throughout the test, during the application of both normal, and shear stresses so that full consolidation occurs and no excess pore pressure is set up at any stage of the test, is known as

a. quick test

b. drained test

c. consolidated undrained test

d. none of these.

20. The liquid limit and plastic limit exist in

a. sandy soils

b. silty soils

c. gravel soils

d. clay soils.

21. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

22. If the shear stress in a R.C.C. beam is

a. equal or less than 5 kg/cm2, no shear reinforcement is provided

b. greater than 4 kg/cm2, but less than 20 kg/cm2, shear reinforcement is provided

c. greater than 20 kg/cm2, the size of the section is changed

d. all the above.

23. The maximum ratio of span to depth of a slab simply supported and spanning in two directions, is

a. 30

b. 35

c. 40

d. 15

24. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

25. Pile foundation is generally provided if soil is

a. compressible

b. water logged

c. made up

d. all the above.

26. A concrete structure is set on fire and the temperature raises to 1000°C. The strength of concrete as compared to original strength reduces to

a. 10%

b. 15%

c. 20%

d. 25%.

27. The top diameter, bottom diameter and the height of a slump mould are :

a. 10 cm, 20 cm, 30 cm

b. 10 cm, 30 cm, 20 cm

c. 20 cm, 10 cm, 30 cm

d. 20 cm, 30 cm, 10 cm

28. If 50 kg of fine aggregates and 100 kg of coarse agregates are mixed in a concrete whose water cement ratio is 0.6, the weight of water required for harsh mix, is

a. 10 kg

b. 12 kg

c. 14 kg

d. 15 kg.

29. Ground water from artesian wells

a. contains dissolved salts

b. may be saltish and hard

c. generally require lesser treatment

d. all the above.

30. For a network of distribution pipes of a proposed township shown in the below figure, the layout of distribution system, is

a. dead end system

b. grid-iron system

c. ring system

d. radial system.

31. Pick up the correct statement from the following :

a. Rational method of estimating peak run off, may be used precisely for areas less than 50 hectares

b. The period after which the entire area starts contributing to the run off, is called the time of concentration

c. The time required by water to reach the outlet of a catchment from the most remote point, is called time of concentration

d. All the above.

32. For a continuous flow type of sedimentation tanks

a. length of the tank is normally kept 4 to 5 times the width

b. maximum horizontal flow velocity is limited to 0.3 m/minute

c. capacity of the tank for 24 hours, is kept equal to maximum daily flow of sewage

d. all the above.

33. According to Lacey, in regime conditions

a. silt is kept in suspension by vertical components of eddies

b. entire cross-section of the channel is generated at all points by the forces normal to the wetted perimeter

c. both (a) and (b)

d. neither (a) nor (b).

34. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

35. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

36. If the straight sides of a triangular section of a lined canal with circular bottom of radius R, make an angle θ with horizontal, the perimeter of the canal is

a. 2R(θ + tan θ)

b. R(θ + cos θ)

c. 2R(θ + cos θ)

d. none of these.

37. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

38. If C is basic capacity per lane, V is velocity in km/hour, S is stopping distance pluslength of the vehicles in metres, the formula  is applicable to

a. district roads

b. two lane roads

c. two lane roads in one direction

d. none of these.

39. If the number of lanes on the carriageway of a road is more than two, the total width of lane ways is equal to 3.0 m

a. + 0.70 m

b. + 0.80 xn

c. + 0.90 m

d. + 1.50 m

40. If the radii of a compound curve and a reverse curve are respectively the same, the length of common tangent

a. of compound curve will be more

b. of reverse curve will be more

c. of both curves will be equal

d. none of these.

41. The trap which is provided to disconnect the house drain from the street sewer is called

a. Intercepting trap

b. Interception manhole

c. Intercepterchamber

d. All the above.

42. The damp proof course (D.P.C.) is measured in

a. Cub.m

b. Sq m

c. Metres

d. None of these

43. Due to change in price level, a revised estimate is prepared if the sanctioned estimate exceeds

a. 2.0%

b. 2.5%

c. 4.0%

d. 5.0%

44. In the mid-section formula

a. The area of mid-sections is calculated by using mean depth

b. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two sections

c. The volume of the earth work is calculated by multiplying the mid-section area by the distance between the two original sections

d. (a), (b) and (c) of the above.

45. Pick up the correct network for the activities of pouring concrete, erection of form work, removal of form work and curing of concrete from the following:

a. 1. Pouring of Concrete  2. Erection of form work 3. Curing of concrete 4. Removal of form work

b. 1. Erection of form work 2. Pouring of concrete 3. Curing of concrete 4. Removal of form work

c. 1. Removal of form work  2. Erection of form work 3. Pouring of concrete 4. Curing of concrete

d. 1. Pouring of concrete 2. Curing of concrete 3. Erection of form work 4. Removal of form work

46. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

47. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

48. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

49. The maximum length and pavement strength of the runway is that of

a. A 1

b. B 2

c. B 3

d. G 7

50. The coordinates of the ends of a runway are (5000, 5000) and (8000, 7000). The co-ordinates of an other runway are (4600, 5100) and (7000, 5300). The co-ordinates of the A.R.P. are

a. (6500, 6000)

b. (5800, 5200)

c. (61500, 5600)

d. (8000, 7000)