***Sub Engineer***

1. In precision theodolite traverse if included angles are read twice and the mean reading accepted using both verniers having a least count of 30". Assuming the instrument to be in perfect adjustment, linear measurements correct to 6 mm per 30 metre tape duly corrected for temperature, slope and sag, the angular error of closure not to exceed (where n is the number of traverse legs)

a. 50" n

b. 30" n

c. 60" n.

2. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

3. In an adjusted level, when the bubble is central, the axis of the bubble tube becomes parallel to

a. line of sight

b. line of collimation

c. axis of the telescope

d. None of these.

4. The systematic errors which persist and have regular effects in the performance of a survey operation, are due to

a. carelessness

b. faulty instrument

c. inattention

d. none of these.

5. The commonly used lime in white washing, is

a. fat lime

b. hydraulic lime

c. lime

d. quick lime.

6. The most fire resistant paints are :

a. enamel paints

b. aluminium paints

c. asbestos paints

d. cement paints.

7. Stainless steel resists corrosion due to

a. sulphur

b. vanadium

c. chromium

d. maganese.

8. The commonly used colour pigment in paints, is

a. carbon black

b. iron oxide

c. lamp black

d. all the above.

9. The tangential component of stress on an plane inclined θ° to the direction of the force, may be obtained by multiplying the normal stress by

a. cos θ

b. tan θ

c. sin θ cos θ

d. sin2θ

10. A steel rod of sectional area 250 sq. mm connects two parallel walls 5 m apart. The nuts at the ends were tightened when the rod was heated to 100°C. If αsteel = 0.000012/C°, Esteel = 0.2 MN/mm2, the tensile force developed at a temperature of 50°C, is

a. 80 N/mm2

b. 100 N/mm2

c. 120 N/mm2

d. 150 N/mm2

11. The radius of gyration of a rectangular section is not proportional to

a. square root of the moment of inertia

b. square root of the inverse of the area

c. square root of the moment of inertia divided by area of the section

d. none of these.

12. The neutral axis of a beam cross-section must

a. pass through the centroid of the section

b. be equidistant from the top of bottom films

c. be an axis of symmetry of the section

d. none of these.

13. An ideal fluid is

a. incompressible

b. compressible

c. compressible and non-viscous

d. slightly affected by surface torque.

14. Falling drops of water become spheres due to

a. adhesion

b. cohesion

c. surface tension

d. viscosity.

15. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

16. Hydrostatic pressure on a dam depends upon its

a. depth

b. shape

c. material

d. both (b) and (c).

17. A stratum of clay 2 m thick will get consolidated 80% in 10 years. For the 80% consolidation of 8 m thick stratum of the same clay, the time required is

a. 120 years

b. 140 years

c. 160 years

d. 180 years.

18. The soil which contains finest grain particles, is

a. coarse sand

b. fine sand

c. silt

d. clay.

19. Compression of soil occurs rapidly if voids are filled with

a. air

b. water

c. partly with air and partly with water

d. none of these.

20. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

21. Minimum spacing between horizontal parallel reinforcement of the same size should not be less than

a. one diameter

b. 2.5 diameters

c. 3 diameters

d. 3.5 diameters

22. A very comfortable type of stairs is

a. straight

b. dog legged

c. geometrical

d. open newel.

23. The maximum ratio of span to depth of a cantilever slab, is

a. 10

b. 12

c. 14

d. 16

24. The number of treads in a flight is equal to

a. risers in the flight

b. risers plus one

c. risers minus one

d. none of these.

25. Pick up the correct statement from the following :

a. the first coat of stucco plaster is called scratch coat

b. the second coat of stucco plaster is called brown coat

c. the third coat of stucco plaster is called white coat

d. all the above.

26. The type of pointing in which upper side of mortar joints is kept about 12 mm inside the face of the masonry and bottom is kept flushed with face of wall, is

a. truck pointing

b. recessed pointing

c. struck pointing

d. grooved pointing.

27. The aggregate impact value of the aggregate used in

a. building concrete is less than 45

b. road pavement concrete is less than 30

c. runway concrete is less than 30

d. all the above.

28. If the effective working time is 7 hours and per batch time of concrete is 3 minutes, the output of a concrete mixer of 150 litre capacity, is

a. 15, 900 litres

b. 16, 900 litres

c. 17, 900 litres

d. 18, 900 litres

29. Water having pH value as 6, is

a. alkaline

b. acidic

c. neutral

d. none of these.

30. Continuous flow of water can be expected from

a. gravity springs

b. surface springs

c. artesian springs

d. none of these.

31. Pick up the correct statement from the following :

a. The circular sewers are provided for separate sewerage system

b. The circular sewers work efficiently if the sections run at least half full

c. Two sewers of different shapes are said to be hydraulically equivalent if they discharge at the same rate while running full on the same grade

d. All the above.

32. The normal value of over flow rates for plain primary sedimentation tanks, ranges between

a. 40, 000 to 50, 000 litres/sqm/day

b. 50, 000 to 60, 000 litres/sqm/day

c. 80, 000 to 100, 000 litres/sqm/day

d. none of these.

33. In a barrage, the crest level is kept

a. low with large gates

b. high with large gates

c. high with no gates

d. low with no gates.

34. An outlet which maintains a constant discharge irrespective of fluctuation in the water levels of the supplying channel or water course, is known as

a. non-modular outlet

b. semi-modular outlet

c. flexible modular outlet

d. right modular outlet.

35. The depth of rice root zone, is

a. 60 cm

b. 70 cm

c. 80 cm

d. 90 cm

36. The ratio of the rate of change of discharge of an outlet and parent channel, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

37. The advantage of providing superelevation on roads, is

a. increased volume of traffic

b. reduced maintenance cost of the roads

c. draining off rain water quickly

d. all the above.

38. Traffic surveys are carried out

a. to know the type of traffic

b. to determine the facilities to traffic regulations

c. to design proper drainage system

d. all the above.

39. If the radius of curvature of a hill road is 50 m, the percentage grade compensation should be

a. 70/R

b. 75/R

c. 80/R

d. 100/R

40. If x% is the gradient of an alignment and y% is the gradient after proper superelevation along a curved portion of a highway, the differential grade along the curve, is

a. (x - y)%

b. (y - x)%

c. (x x y)%

d. (y + x)%

41. The value of 'A' of Indian type W.C. shown in the given figure is :

a. 25 cm

b. 30 cm

c. 40 cm

d. 45 cm

42. The reduced levels of points, 30 metres apart along the longitudinal section of a road portion between chainages 5 and 9 are shown in the given figure. If there is a uniform up-gradient of the road 120 in 1, the chainage of the point with no filling or cutting is

a. (6 + 15) chains

b. (6 + 12) chains

c. (6 + 18) chains

d. None of these.

43. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

44. According to ISI method of measurement, the order of the sequence is

a. Length, breadth, height

b. Breadth, length, height

c. Height, length, breadth

d. None of these.

45. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

46. The critical activity has

a. maximum float

b. minimum float

c. zero float

d. none of these.

47. In CPM analysis,

a. uncertainties are not allowed

b. activities are represented by arrows

c. beginning and end of an activity, are denoted by nodes

d. all the above.

48. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

49. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.

50. Pick up the component not applicable to aeroplanes

a. engines

b. air screw

c. fuse lage

d. none of these.