***Sub Engineer***

1. The angle of intersection of a curve is the angle between

a. back tangent and forward tangent

b. prolongation of back tangent and forward tangent

c. forward tangent and long chord

d. back tangent and long chord.

2. Angles to a given pivot station observed from a number of traverse stations when plotted, the lines to the pivot station intersect at a common point

a. angular measurements are correct and not the linear measurements

b. linear measurements are correct and not the angular measurements

c. angular and linear measurements are correct and not the plotting of traverse

d. angular and linear measurements and also plotting of the traverse are correct.

3. Probable accidental error in precise levelling as recommended by International Geodetic Association, should not exceed (where k is in kilometers.)

a. ± 0.5 k mm

b. ± 1 k mm

c. ± 2 k mm

d. ± 5 k mm.

4. The accuracy of measurement in chain surveying, does not depend upon

a. length of the offset

b. scale of the plotting

c. importance of the features

d. general layout of the chain lines.

5. The operation of removal of impurities or clay adhering to iron ores, is known as

a. dressing

b. calcination

c. roasting

d. smelting.

6. The size of mould for bricks, is generally kept

a. a little large to specified size

b. a little small to specified size

c. equal to specified size

d. 10% larger than specified size

7. Pick up the correct statement from the following:

a. The average crushing strength of hand moulded bricks is 6000 t/m2

b. The average tensile strength of hand moulded brick is 200 t/m2

c. The average shearing strength of hand moulded brick is 600 t/m2

d. All the above.

8. Dorry's testing machine is used for

a. crushing test of stone

b. hardness test of stone

c. impact test of stone

d. water absorption test.

9. A simply supported rolled steel joist 8 m long carries a uniformly distributed load over it span so that the maximum bending stress is 75 N/mm2. If the slope at the ends is 0.005 radian and the value of E = 0.2 x 106 N/mm2, the depth of the joist, is

a. 250 mm

b. 300 mm

c. 350 mm

d. 400 mm

10. For beams of uniform strength, if depth is constant,

a. width b ∝ M

b. width b ∝ M

c. width b ∝ 3 M

d. width b ∝

11. In the cantilever truss as shown in below figure, the horizontal component of the reaction at A, is

a. 30 tonnes

b. 60 tonnes

c. 90 tonnes

d. 120 tonnes

12. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

13. When the whole fluid mass rotates either due to fluid pressure or gravity or rotation previously imparted, the motion is known as

a. free vortex

b. forced vortex

c. non-potential vortex

d. rotational vortex.

14. Total pressure on the top of a closed cylindrical vessesl completely filled with liquid, is directly proportional to

a. (radius)2

b. (radius)3

c. (radius)4

d. none of these.

15. One metric slug is equal to

a. 1 kg wt

b. 9.81 kg wt

c. 9.81 kg mass

d. 0.98 kg wt.

16. An open container filled with water is moved vertically downward with a uniform linear acceleration. The pressure at its bottom will be

a. greater than static pressure

b. equal to static pressure

c. lesser than static pressure

d. none of these.

17. The vane shear test is used for the in-situ determination of the undrained strength of the intact fully saturated

a. sands

b. clays

c. gravels

d. highly organic soils.

18. The void ratio of a soil sample decreases from 1.50 to 1.25 when the pressure is increased from 25 tonnes/m2 to 50 tonnes/m2, the coefficient of compressibility is

a. 0.01

b. 0.02

c. 0.05

d. 0.001

19. If there is no impervious boundary at the bottom of a hydraulic structure, stream lines tend to follow :

a. a straight line

b. a parabola

c. a semi-ellipse

d. a semi-circle.

20. According to Coulomb's wedge theory, the active earth pressure slides the wedge

a. down and outwards on a slip surface

b. up and inwards on a slip surface

c. horizontal upward and parallel to base

d. horizontal inward and parallel to base.

21. If the ratio of the span to the overall depth does not exceed 10, the stiffness of the beam will ordinarily be satisfactory in case of a

a. simply supported beam

b. continuous beam

c. cantilever beam

d. none of these.

22. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

23. The effective width of a column strip of a flat slab, is

a. half the width of the panel

b. radius of the column

c. diameter of the column

d. none of these.

24. For M 150 grade concrete (1:2:4) the moment of resistance factor is

a. 8.50

b. 7.50

c. 5.80

d. none of these.

25. The highest line of sloping roof, where two oposite slopes meet, is known as

a. rafter

b. ridge

c. crown

d. eave.

26. Pick up the incorrect statement from the following :

a. In Flemish bond every header in each course lies centrally over every stretcher of the underlying course

b. In English bond, stretchers are laid in every course

c. In English bond, headers and stretchers are laid in alternate courses

d. None of these.

27. Pick up the correct statement from the following:

a. Consistency does not affect the workability of concrete

b. If the slump increases, workability decreases

c. If the concrete mix is dry, the slump is maximum

d. None of these.

28. The minimum percentage of chemical ingredient of cement is that of

a. magnesium oxide

b. iron oxide

c. alumina

d. lime

29. To remove very fine suspended particles from water, the method adopted is

a. screening

b. sedimentation

c. boiling

d. filtration.

30. Detention period of a settling tank is

a. time required for flow of water to fill the tank fully

b. average time for which water is retained in tank

c. ratio of volume of basin of a sedimentation tank to the rate of flow

d. all the above.

31. The ratio of minimum hourly flow to the average flow of sewage is

a. 1/4

b. 1/3

c. 1/2

d. 3/4

32. Pick up the correct statement from the following :

a. In treated sewage, 4 PPm of D.O. is essential

b. Only very fresh sewage contains some dissolved oxygen

c. The solubility of oxygen in sewage is 95% that is in distilled water

d. All the above.

33. According to Khosla, the exist gradient of surface flow

a. depends upon the b/d ratio

b. is independent of the b/d ratio

c. is independent of the depths of d/s cut off walls

d. none of these.

34. A fall in a canal bed is generally provided, if

a. ground slope exceeds the designed bed slope

b. designed bed slope exceeds the ground slope

c. ground slope is practically the same as the designed bed slope

d. none of these.

35. When a canal flowing under pressure is carried below a natural drainage such that its F.S.L. does not touch the underside of the supporting structure, the structure so provided, is called

a. syphon

b. aqueduct

c. super passage

d. syphon-aqueduct.

36. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

37. If a Lemniscate curve of transition throughout is introduced to connect two parallel roads, the maximum polar angle of the curve, is

a. 15°

b. 20°

c. 30°

d. 45°

38. Volume of traffic which is due to improvement carried out in adjacent area, is known as

a. development traffic

b. generated traffic growth

c. normal traffic growth

d. current traffic.

39. Thickness of broken centre line markings for a four lane road, is generally kept

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm

40. The inventor of road making as a building science, was

a. Sully

b. Tresguet

c. Telford

d. Macadam.

41. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

42. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

43. If the formation level of a highway has a uniform gradient for a particular length, and the ground is also having a longitudinal slope, the earthwork may be calculated by

a. Mid-section formula

b. Trapezoidal formula

c. Prismoidal formula

d. All the above.

44. The expected out turn of 12 mm plastering with cement mortar is

a. 4.0 sq m

b. 6.0 sq m

c. 8.0 sq m

d. 10 sq m

45. Critical path method

a. provides a realistic approach to daily problems

b. avoids delays which are very common in bar charts

c. was invented by Morgan R. Walker of Dupot and James E. Kalley or Remington U.S.A. in 1957

d. All the above.

46. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

47. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

48. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

49. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.

50. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.