***Sub Engineer***

1. Ramsden eye-piece consists of

a. two convex lenses short distance apart

b. two concave lenses short distance apart

c. one convex lens and one concave lens short distance apart

d. two plano-convex lenses short distance apart, with the convex surfaces facing each other.

2. The slope correction for a 3° slope for a length of 100 m, is

a. -0.11 m

b. -0.12 m

c. -1.87 m

d. -0.137 m

3. The ratio of the distances at which a stated length can be distinguished by the telescope and the human eye, respectively, is called

a. brightness of telescope

b. magnification of telescope

c. resolving power of telescope

d. none of these.

4. Centering error of a theodolite produces an error

a. in all angles equally

b. which does not vary with the direction or pointing

c. which varies with the direction of pointing and inversely with the length of sight

d. none of these.

5. The presence of original rounded surface on the manufactured piece of timber, is called

a. Wane

b. Torn grain

c. Diagonal grain

d. Chipmark.

6. Pick up the correct statement from the following:

a. The distinct plane of division along which a stone can easily be split, is called natural bed of stone

b. The natural bed of sedimentary rocks is along the planes of stratification

c. The natural bed of igneous rocks is not defined

d. All the above.

7. The main ingredient of a good quality brick earth, is

a. magnesia

b. lime

c. silica

d. alumina.

8. Pick up the correct statement from the following:

a. The average crushing strength of hand moulded bricks is 6000 t/m2

b. The average tensile strength of hand moulded brick is 200 t/m2

c. The average shearing strength of hand moulded brick is 600 t/m2

d. All the above.

9. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

10. The forces in the members of simple trusses, may be analysed by

a. graphical method

b. method of joints

c. method of sections

d. all the above.

11. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

12. If a rectangular beam measuring 10 x 18 x 400 cm carries a unformly distributed load such that the bending stress developed is 100 kg/cm2. The intensity of the load per metre length, is

a. 250 kg

b. 260 kg

c. 270 kg

d. 280 kg.

13. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

14. If H is height of the liquid above the sill, the effect of end contractions, according to Francis formula, is

a. 0.1 H

b. 0.2 H

c. 0.3 H

d. 0.4 H

15. In fluids, steady flow occurs when

a. conditions of flow change steadily with time

b. conditions of flow do not change with time at a point

c. conditions of flow remain the same at adjacent point

d. velocity vector remains constant at a point.

16. A syphon is used

a. to connect water reservoirs at different levels intervened by a hill

b. to supply water to a town from higher level to lower level

c. to fill up a tank with water at higher level from a lower level

d. none of these.

17. The expression  ρω is used for

a. dry density

b. bulk density

c. degree of saturation

d. optimum water content.

18. A critical hydraulic gradient may occur when

a. flow is in upward direction

b. seepage pressure is in upward direction

c. effective pressure is zero

d. all the above.

19. Pick up the correct statement from the following:

a. The ratio of the liquid limit minus the natural water content to the plasticity index of soils, is called consistency index

b. The ratio of natural water content minus its plastic limit to its plasticity index is called liquidity index

c. The ratio between plasticity index and flow index (i.e. slope of flow curve in case of liquid limit), is called toughness index

d. All the above.

20. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

21. The diameter of the column head support a flat slab, is generally kept

a. 0.25 times the span length

b. 0.25 times the diameter of the column

c. 4.0 cm larger than the diameter of the column

d. 5.0 cm larger than the diameter of the column

22. If the loading on a prestressed rectangular beam, is uniformly distributed, the tendon to be provided should be .

a. parabolic with convexity downward

b. parabolic with convexity upward

c. straight above centroidal axis

d. none of these.

23. A raft foundation is provided if its area exceeds the plan area of the building by

a. 20%

b. 30%

c. 40%

d. 50%

24. The advantage of reinforced concrete, is due to

a. fire-resisting and durability

b. economy because of less maintenance cost

c. moulding in any desired shape

d. All the above.

25. The form work from the slabs excluding props, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days

26. In soft clay of low bearing capacity, the type of steel pile generally used, is

a. screw pile

b. disc pile

c. pipe pile

d. raking pile.

27. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

28. Pozzolanic properties exist in

a. fly ash

b. pumicite

c. diatomaceous clay

d. all the above.

29. Suction lift of a pump depends upon

a. water temperature

b. velocity of water in suction pipe

c. frictional losses

d. all the above.

30. Disinfection of drinking water is done to remove

a. odour

b. colour

c. bacterias

d. all the above.

31. Oxidation process results in the formation of

a. carbon dioxide

b. nitrates

c. sulphates

d. all the above.

32. The asbestos cement sewers are

a. light in weight

b. not structurally strong to bear large compressive stress

c. susceptible to corrosion by sulphuric acid

d. All the above.

33. The width of a meander belt is the transverse distance between

a. apex point of one curve and apex point of the reserve curve

b. apex point and the crossing

c. two banks of meandering river

d. none of these.

34. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

35. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

36. The field capacity of a soil depends upon

a. capillary tension in soil

b. porosity of soil

c. both (a) and (b)

d. neither (a) nor (b).

37. Longitudinal pavement lines marked broken in white paint

a. are for the guidance of the drivers

b. are not to be crossed over

c. may be crossed over at the discretion of the driver

d. all the above.

38. Newly constructed pavement with ordinary Portland cement attains its working strength after

a. 14 days

b. 21 days

c. 28 days

d. 42 days

39. If brakes of vehicles are effective, the vehicle-running at 30 km/hour comes to a stop in

a. 10 metres

b. 12 metres

c. 15 metres

d. 18 metres

40. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

41. The item of the brick structure measured in sq.m, is

a. Reinforced brick work

b. Broken glass coping

c. Brick edging

d. Brick work in arches.

42. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

43. Pick up the correct statement from the following:

a. In order to check up the average depth of excavation, 'Dead mans' are left at the mid-widths of borrow pits

b. The earthwork calculation in excavation is made from the difference in levels obtained with a level

c. The earth work in excavation to form the road embankment includes the formation of correct profile and depositing the soil in layers

d. All the above.

44. The value of 'C' of Indian type W.C. shown in the given figure is :

a. 400 mm

b. 450 mm

c. 500 mm

d. 550 mm

45. PERT is

a. limited of event oriented diagrams

b. used for research and development projects

c. based on three time estimates for activities linking up two events

d. all the above.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

48. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

49. Speed of

a. sound varies 2.4 km p.h. per degree centrigrade rise in temperature

b. sound at 0°C is called one Mach

c. an aircraft flying at 2975 km p.h., is called 2.5 Mach

d. All the above.

50. If the width of the approach area near the runway end is 150 m, the width of the approach area at a distance of 3 kilometres from runway end will be

a. 1500 m

b. 1200 m

c. 1000 m

d. 800 m