***Sub Engineer***

1. The curvature of the earth's surface, is taken into account only if the extent of survey is more than

a. 100 sq km

b. 160 sq km

c. 200 sq km

d. 260 sq km.

2. While rotating the theodolite in the horizontal plane, the bubble of the bubble tube takes up the same position in its tube, it indicates

a. the rotation axis is vertical

b. the trummion axis is horizontal

c. the line of collimation is perpendicular to vertical axis

d. none of the above

3. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

4. Transition curves are introduced at either end of a circular curve, to obtain

a. gradually decrease of curvature from zero at the tangent point to the specified quantity at the junction of the transition curve with main curve

b. gradual increase of super-elevation from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

c. gradual change of gradient from zero at the tangent point to the specified amount at the junction of the transition curve with main curve

d. none of these.

5. In a mortar, the binding material is

a. cement

b. sand

c. surkhi

d. cinder.

6. Plastic bitumen is generally used for

a. road pavements

b. expansion joints

c. crack fillings

d. none to these.

7. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

8. The rocks in which argil (or clay) predominates, are called

a. sillicious rocks

b. argillaceous rocks

c. calcareous rocks

d. igneous rocks.

9. The eccentricity (e) of a hollow circular column, external diameter 25 cm, internal diameter 15 cm for an eccentric load 100 t for non-development of tension, is

a. 3.00 cm

b. 3.50 cm

c. 4.25 cm

d. 5.0 cm

10. The ratio of tangential and normal components of a stress on an inclined plane through θ° to the direction of the force, is :

a. cos θ

b. tan θ

c. cos θ

d. sec θ

11. Pick up the correct assumption of the theory of simple bending

a. Transverse section of a beam remains plane before and after bending

b. The material of the beam is homogeneous and isotropic

c. The resultant pull or thrust on transverse section of a beam is zero

d. All the above.

12. The stress in the wall of a cylinder in a direction normal to its longitudinal axis, due to a force acting along the circumference, is known as

a. longitudinal stress

b. hoop stress

c. circumferential stress

d. ultimate stress.

13. The discharge through an internal mouth piece is more if its length is

a. <  diameter

b. < diameter

c. ≥ diameter

d. none of these.

14. A non-uniform steady flow is through

a. a long tube at a decreasing rate

b. an expanding tube at constant rate

c. an expanding tube at increasing rate

d. a long pipe at increasing rate.

15. To avoid the force of surface tension in an inclined manometer, the minimum angle of inclination is

a. 2°

b. 3°

c. 4°

d. 5°

16. The discharge through a 100 mm diameter external mouth piece fitted to the side of a large vessel is 0.05948 m3/s. The head over the mouth piece is

a. 2 m

b. 2.5 m

c. 3.0 m

d. 4.0 m.

17. The change of moisture content of soils, changes the

a. value of the angle of repose

b. amount of compaction required

c. cohesive strength of soil

d. all the above.

18. Pick up the correct statement from the following:

a. The freezing of water is accompained by a volume increase of 9%.

b. Below freezing point, higher soil suction develops.

c. The magnitude of frost heave decreases as the degree of saturation of soil decreases

d. All the above.

19. 260 g of wet soil was taken in a pycnometer jar of weight 400 g in order to find the moisture content in the soil, with specific gravity of soil particles 2.75. The weight of soil and remaining water filled in pycnometer without air bubbles was 1415 g and the weight of pycnometer filled with water alone was 1275 g. The moisture content in the soil is

a. 24.2%

b. 18.2%

c. 53.8%

d. none of these.

20. The fluid generally used for grouting is

a. clay suspension

b. sodium silicate

c. bitumen emulsion

d. all the above.

21. The diameter of main bars in R.C.C. columns, shall not be less than

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

22. In a combined footing for two columns carrying unequal loads, the maximum hogging bending moment occurs at

a. more loaded column

b. a point equidistant from either column

c. a point of the maximum shear force

d. a point of zero shear force.

23. A short column 20 cm x 20 cm in section is reinforced with 4 bars whose area of cross section is 20 sq. cm. If permissible compressive stresses in concrete and steel are 40 kg/cm2 and 300 kg/cm2, the Safe load on the column, should not exceed

a. 4120 kg

b. 41, 200 kg

c. 412, 000 kg

d. none of these.

24. Pick up the incorrect statement from the following. The intensity of horizontal shear stress at the elemental part of a beam section, is directly proportional to

a. area of the section

b. distance of the C.G. of the area from its neutral axis

c. moment of the beam section about its neutral axis

d. width of the beam.

25. The inclined surface of an abutment to receive the arch, is known as

a. skew back

b. soffit

c. spandril

d. haunch.

26. Arches in the form of masonry arcs struck from more than four centres, are called

a. two curved arches

b. gothic arches

c. ogee arches

d. drop gothic arches.

27. The impurity of mixing water which affects the setting time and strength of concrete, is

a. sodium chlorides

b. sodium carbonates and bicarbonates

c. calcium chorides

d. calcium bicarbonates

28. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

29. For a network of distribution pipes of a proposed township shown in the below figure, the layout of distribution system, is

a. dead end system

b. grid-iron system

c. ring system

d. radial system.

30. The bacterias which require free oxygen for their survival, are called

a. Aerobic bacterias

b. Anaerobic bacterias

c. Faculative bacteria

d. None of these.

31. If the diameter of a sewer is 100 mm, the gradient required for generating self cleansing velocity, is

a. 1 in 60

b. 1 in 100

c. 1 in 120

d. none of these.

32. For evaporation and measurement of settlable solids, the apparatus used, is

a. a jar

b. a breaker

c. a test tube

d. an Imhoff cone.

33. In Montague type fall

a. a straight glacis is provided

b. a circular glacis is provided

c. a parabolic glacis is provided

d. no glacis is provided.

34. The uplift pressure on the roof of an inverted syphon, is maximum when

a. drain in running dry

b. canal is running dry

c. canal is running with F.S.L.

d. drain is running with H.F.L.

35. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

36. Pick up the correct statement from the following:

a. Static level of water in an open well indicates the position of the water table

b. Saturated pores of a soil in the capillary zone above water table have negative pressure

c. At water table, hydrostatic pressure is zero

d. All the above.

37. If the coefficient of friction on the road surface is 0.15 and a maximum super-elevation 1 in 15 is provided, the maximum speed of the vehicles on a curve of 100 metre radius, is

a. 42.44 kg/hour

b. 52.44 km/hour

c. 62.44 km/hour

d. 72.44 km/hour

38. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

39. According to Highway Research Board of U.S.A. practical land width, is

a. 3.0 m

b. 3.3 m

c. 3.6 m

d. 4.5 m

40. The efficiency of the brakes of a vehicle depends upon

a. condition of road surface

b. condition of the tyres

c. presence of the show moisture

d. all the above.

41. The value of 'C' of Indian type W.C. shown in the given figure is :

a. 400 mm

b. 450 mm

c. 500 mm

d. 550 mm

42. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

43. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

44. As per Indian Standard Specifications, the peak discharge for domestic purposes per capita per minute, is taken

a. 1.80 litres for 5 to 10 users

b. 1.20 litres for 15 users

c. 1.35 for 20 users

d. All the above.

45. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

46. The three time estimates for the activities of the network shown in the given figure are shown above their arrows. The earliest expected time for the event 4, is

a. 19

b. 14

c. 24

d. None of these.

47. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

48. The artificial activity which indicates that an activity following it, cannot be started unless the preceding activity is complete, is known as

a. event

b. free float

c. dummy

d. constant

49. Two single runways may be arranged so as to have

a. L-shape

b. T-shape

c. X-shape

d. All the above.

50. Pick up the correct statement from the following:

a. Minimum turning radius of aircrafts decides the size of the apron and the radius of the curves at taxi-ends

b. Take off and landing distances for an aircraft, determine the minimum runway length

c. The length of the normal haul of the air craft decides the frequency of operation

d. All the above.