***Sub Engineer***

1. In case of a direct vernier scale

a. graduations increase in opposite direction in which graduations of the main scale increase

b. smallest division is longer than smallest division of the main scale

c. graduations increase in the same direction in which graduations of the main scale increase

d. none of these.

2. Prolongation of chain line across an obstruction in chain surveying, is done by

a. making angular measurements

b. drawing perpendiculars with a chain

c. solution of triangles

d. all the above.

3. During levelling if back sight is more than foresight

a. The forward staff is at lower point

b. The back staff is at lower point

c. The difference in level, cannot be ascertained.

d. none of these.

4. In a constant level tube, size of the bubble remains constant because upper wall is

a. of relatively larger radius

b. of relatively smaller radius

c. flat

d. convex downwards.

5. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

6. Pick up the correct characteristic of Pyroxene from the following:

a. It converts to chlorine by hydration

b. Its density is 2.3 to 3.6 g/cm2

c. Its hardness is 5 to 6

d. All the above.

7. Cast steel is manufactured by

a. Cementation process

b. Crucible process

c. Bessemer process

d. Open hearth process.

8. The initial setting time of hydraulic lime, is

a. 60 minutes

b. 90 minutes

c. 120 minutes

d. 150 minutes.

9. In plastic analysis, the shape factor for a circular section, is

a. 1.5

b. 1.6

c. 1.7

d. 1.75

10. For a strongest rectangular beam cut from a circular log, the ratio of the width and depth, is

a. 0.404

b. 0.505

c. 0.606

d. 0.707

11. The number of points of contraflexure in a simple supported beam carrying uniformly distributed load, is

a. 1

b. 2

c. 3

d. 0

12. The shear stress at any section of a shaft is maximum

a. at a distance r/2 from the centre

b. at the top of the surface

c. at a distance 3/4 r from the centre

d. none of these.

13. For a long pipe, the head loss

a. at the entrance is ignored

b. at the outlet is ignored

c. at the entrance and outlet both are ignored

d. due to friction is ignored.

14. Total head of a liquid particle in motion is the sum of

a. potential head and kinetic head

b. kinetic head and pressure head

c. potential head and pressure head

d. potential head, kinetic head and pressure head.

15. Most economical section of a circular channel for maximum velocity, is if,

a. depth of water = 0.810 diameter

b. hydraulic mean depth = 0.304 diameter

c. wetted perimeter = 2.245 diameters

d. all the above.

16. A water tank partially filled with water is being carried on a truck moving with a constant horizontal acceleration. The level of the water

a. falls on the back side of the tank

b. remains the same at both sides of the tank

c. rises on the back side and falls on the front side

d. none of these.

17. The triaxial apparatus is usually used for

a. unconsolidated-undrained test

b. consolidated-undrained test

c. drained test

d. all the above tests.

18. The maximum net pressure intensity causing shear failure of soil, is known

a. safe bearing capacity

b. net safe bearing capacity

c. net ultimate bearing capacity

d. ultimate bearing capacity.

19. Which one of the following statements is true for Mohr-Coulomb envelope ?

a. The generalised Mohr theory suggests that, though the shear stress depends on the normal stress, the relation is not linear

b. Coulomb and Mohr suggest that a definite relationship exists among the principal stress and the angle of internal friction

c. For an ideal pure friction material, the straight line passes through the origin.

d. All the above.

20. The Terzaghi's general bearing capacity equation for a continuous footing is given by (where Nc, 2V& and Ny are bearing capacity factors.)

a. qf = cNc + γDNq + 0.5γBNγ

b. qf = cNc - γDNq + 0.5γBNγ

c. qf = cNc + γDNq - 0.5γBNγ

d. qf = cNc - γDNq - 0.5γBNγ

21. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

22. The effective span of a simply supported slab, is

a. distance between the centres of the bearings

b. clear distance between the inner faces of the walls plus twice the thickness of the wall

c. clear span plus effective depth of the slab

d. none of these.

23. The pitch of the main bars in a simply supported slab, should not exceed its effective depth by

a. four times

b. five times

c. six times

d. seven times.

24. The toe projection of foundation slabs is taken

a. as one third of the base

b. as one sixth of overall height of the wall

c. equal to heel slab

d. below ground surface.

25. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

26. The depth of excavation of foundations, is generally measured with a

a. ranging rod

b. steel tape

c. levelling staff

d. bonning rod.

27. The percentage of the aggregate of F.M. 2.6 to be combined with coarse aggregate of F.M. 6.8 for obtaining the aggregates of F.M. 5.4, is

a. 30%

b. 40%

c. 50%

d. 60%.

28. 'Ware house pack' of cement means

a. pressure exertion of the bags of upper layers

b. pressure compaction of the bags on lower layers

c. packing the ware house

d. none of these.

29. The fire demand of a city may be worked out by

a. Freeman formula

b. Under Writers formula

c. Bustan's formula

d. All the above.

30. Gravity conduits

a. follow the hydraulic gradient line

b. are carried on trestles in valleys and depressions

c. are carried through tunnels in deep cuttings

d. All the above.

31. In a city the ratio of the drainage to sewage is 20, the percentage discharge passing through non-monsoon periods, is

a. 5

b. 10

c. 15

d. 20

32. Flocculated particles do not change their

a. size

b. shape

c. weight

d. none of these.

33. In a canal syphon, the flow is

a. under atmospheric pressure

b. pipe flow

c. with critical velocity

d. under negative pressure.

34. To control the silt entry into a distributary at head regulator, King's vanes are provided which are the walls

a. of height l/3rd to l/4th the depth of water in the main canal

b. spaced at interval of  times their heights

c. extending about one metre beyond a line drawn at an inclination of 2 : 1 to the axis of main canal

d. all the above.

35. Lacy's regime condition is obtained if

a. silt grade in the channel is variable

b. discharge in the channel is variable

c. silt charge in the channel is variable

d. channel flows in unlimited, incoherent alluvium of the same character as that transported material.

36. For the design of major hydraulic structures on the canals, the method generally preferred to, is based on

a. Bligh's theory

b. Electrical analogy method

c. The relaxation method

d. Khosla's method of independent variables.

37. When load is applied on concrete pavement

a. away from edges, the maximum bending moment is negative

b. away from edges, the maximum bending moment cause compression

c. on the edges, the maximum stress is parallel to the edge of the slab

d. none of these.

38. The minimum cross fall of shoulders is kept

a. 1.0%

b. 1.5%

c. 2.5%

d. 3%

39. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

40. Minimum permissible speed on high speed roads, is decided on the basis of

a. 15 percentile cumulative frequency

b. 20 percentile cumulative frequency

c. 30 percentile cumulative frequency

d. 40 percentile cumulative frequency.

41. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

42. Pick up the correct statement in case of water supply.

a. Cutting through walls and floors are included with the item

b. Pipes are classified according to their sizes and quality

c. In laying pipes, the method of jointing and fixing is specifically specified

d. All the above.

43. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

44. Pick up the item whose weight is added to the weight of respective item, is

a. Brackets

b. Bolts

c. Distance separators

d. All the above.

45. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

46. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

47. PERT analysis is based on

a. optimistic time

b. pessimistic time

c. most likely time

d. all the above.

48. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. According to the International Civil Aviation Organisation (I.C.A.O.) the strength of runway pavements, have been coded by

a. Last Seven English alphabets

b. First Seven English alphabets

c. First seven numbers

d. None of these.