***Sub Engineer***

1. An ideal transition curve is

a. cubic parabola

b. cubic spiral

c. clothoid spiral

d. true spiral.

2. The intercept of a staff

a. is maximum if the staff is held truly normal to the line of sight.

b. is minimum if the staff is held truly normal to the line of sight.

c. decreases if the staff is tilted away from normal

d. increases if the staff is tilted towards normal.

3. In quadrantal bearing system, back bearing of a line may be obtained from its forward bearing, by

a. adding 180°, if the given bearing is less than 180°

b. subtracting 180°, if the given bearing, is more than 180°

c. changing the cardinal points, i.e. substituting N for S and E for W and vice-versa

d. none of these.

4. Geodetic surveying is undertaken

a. for developing the science of geodesy

b. making use of most accurate instruments and methods of observation

c. for determination of accurate positions on the earth's surface of system of control points

d. all the above.

5. Pick up the correct statement from the following:

a. Bull's trench kiln a trench excavated in ground

b. Hoffman's kiln is constructed overground

c. Tunnel Kiln is constructed as a tunnel

d. All the above.

6. French polish is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

7. The commonly used lime in white washing, is

a. fat lime

b. hydraulic lime

c. lime

d. quick lime.

8. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

9. The S.F. diagram of a loaded beam shown in the given figure is that of

a. a simply supported beam with isolated central load

b. a simply supported beam with uniformly distributed load

c. a cantilever with an isolated load at the free end

d. a cantilever with a uniformly distributed load.

10. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

11. Reactions at the supports of a structure can be determined by equating the algebraic sum of

a. horizontal forces to zero

b. vertical forces to zero

c. moment about any point to zero

d. all the above.

12. For structural analysis, Maxwell's reciprocal theorem can be applied to :

a. plastic structures

b. elastic structures

c. symmetrical structures

d. all the above.

13. Hydrostatic pressure on a dam depends upon its

a. depth

b. shape

c. material

d. both (b) and (c).

14. Hydraulic coefficient of an orifice means the coefficient of

a. velocity

b. contraction

c. resistance

d. all the above.

15. The ratio of frictional factor and coefficient of friction used in general equation for a head loss in a pipe, is

a. 1

b. 2

c. 3

d. 4

16. If u, v, w are the components of the velocity v of a moving particle, the equation  represents

a. one diamensional flow

b. two dimensional flow

c. three dimensional flow

d. none of these.

17. Chemical weathering of soil is caused due to

a. carbonation

b. hydration

c. leaching

d. all the above.

18. The effective size of particles of soil is denoted by

a. D10

b. D20

c. D30

d. D60

19. The angle between the directions of the failure and the major principal plane, is equal to

a. 90° + effective angle of shearing resistance

b. 90° + half of the angle of shearing resistance

c. 45° - half of the angle of shearing resistance

d. 45° + half of the angle of shearing resistance.

20. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

21. The spacing of transverse reinforcement of column is decided by the following consideration.

a. The least lateral dimension of the column

b. Sixteen times the diameter of the smallest longitudinal reinforcing rods in the column

c. Forty-eight times the diameter of transverse reinforcement

d. All the above.

22. In testing a pile by load test, pile platform is loaded with one and half times the design load and a maximum settlement is noted. The load is gradually removed and the consequent rebound is measured. For a safe pile, the net settlement (i.e. total settlement minus rebound) per tonne of test load should not exceed

a. 15 mm

b. 20 mm

c. 25 mm

d. 30 mm

23. The number of treads in a flight is equal to

a. risers in the flight

b. risers plus one

c. risers minus one

d. none of these.

24. An R.C.C. beam of 6 m span is 30 cm wide and has a lever arm of 55 cm. If it carries a U.D.L. of 12 t per m and allowable shear stress is 5 kg/cm2, the beam

a. is safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined bars

d. needs revision of section

25. Pick up the correct statement from the following :

a. the pile driven in sand is called sand pile

b. the drilled hole filled with sand is called sand pile

c. the sand piles are used for bearing purposes

d. None of these.

26. The piece of a brick cut with its one corner equivalent to half the length and half the width of a full brick, is known as

a. queen closer

b. bevelled closer

c. king closer

d. half king closer.

27. The process of hardening the concrete by keeping its surface moist is known

a. wetting

b. curing

c. compacting

d. none of these.

28. Expansion joints are provided if the length of concrete structures exceeds

a. 15 m

b. 15 m

c. 35 m

d. 45 m

29. Acidity in water is caused due to

a. Free CO2

b. Iron sulphate

c. Aluminium sulphate

d. All the above.

30. The dilution ratio at which the odour is hardly detectable is generally called threshold odour number and for public supplies it should not exceed

a. 3

b. 5

c. 7

d. 9

31. The discharge per unit plan area of a sedimentation tank, is generally called

a. over flow rate

b. surface loading

c. over flow velocity

d. all the above.

32. Stone ware pipes are

a. highly impervious

b. hydraulically efficient because of their smooth interior surface

c. specially suited to pressure pipes

d. none of these.

33. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

34. A minimum of 90 cm free board is provided if the discharge in the canal is between

a. 30 to 33 cumecs

b. 30 to 60 cumecs

c. Over 60 cumecs

d. Over 100 cumecs

35. The state of the soil when plants fail to extract sufficient water for their requirements, is

a. maximum saturated point

b. permanent wilting point

c. ultimate utilisation point

d. none of these.

36. The down stream expansion head of a guide bank is extended to subtend an angle at the centre, equal to

a. 30°

b. 45°

c. 60°

d. 90°

37. One-way streets are generally provided in crowded cities as, these

a. reduce delays to vehicles

b. permit higher speed

c. reduce the number of accidents

d. all the above.

38. If present A.D.T. is 5000 vehicles and annual increase is 10%, the average future flow after 5 years will be

a. 6050 vehicles

b. 7050 vehicles

c. 8050 vehicles

d. 9050 vehicles

39. Curves in the same direction separated by short tangents, are called

a. compound curves

b. transition curves

c. broken-back curves.

d. None of these.

40. Pick up the correct statement from the following:

a. Safety fences are provided on either side of a roadway if embankments are in excess of 6 metres

b. Safety fences are provided on outside of the curves of radii less than 750 m if the embankments are between 3 metres and 6 metres

c. Guard stones are provided at 2.5 metres intervals if embankments are between 1.6 metres to 3 metres

d. All the above.

41. The inspection pit or chamber is a manhole provided in a base drainage system

a. at every change of gradient

b. at every 30 m intervals

c. at the point where vertical soil pipe joins the house drain

d. All the above.

42. The concrete work for the following part of the building of specified thickness is measured in square metres

a. Floors

b. D.P.C.

c. Wall panels

d. All the above.

43. The measurement is not made in square metres in case of

a. D.P.C. (Damp proof course)

b. Form works

c. Concrete Jaffries

d. R.C. Chhajja.

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. The difference between the time avail-to do a job and the time required to do the job, is known as

a. event

b. float

c. duration

d. constraint.

46. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

47. Pick up the PERT event from the following:

a. Digging of foundation completed

b. Laying of concrete started

c. Laying of concrete completed

d. All the above.

48. The first method invented for planning projects, was

a. Bar chart method

b. Milestone chart

c. Critical path method (CPM)

d. Programme Evaluation and Review Technique (PERT)

49. The meterological condition which influences the size and location of an air port is

a. air density

b. reduced level

c. wind direction

d. all the above.

50. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.