***Sub Engineer***

1. The true meridian of a place is the line in which earth's surface is intersected by a plane through

a. east and west points

b. zenith and nadir points

c. north and south geographical poles

d. north and south magnetic poles.

2. The distance between terminal points computed from a subsidiary traverse run between them, is generally known, as

a. traverse leg

b. a base

c. traverse base

d. all the above.

3. Ranging in chain survey means

a. establishing an intermediate point on the line

b. determining the distance between end points

c. determining the offset distance

d. none of these

4. While surveying a plot of land by plane tabling, the field observations

a. and plotting proceed simultaneously

b. and plotting do not proceed simultaneously

c. and recorded in field books to be plotted later

d. all the above.

5. Name the type of cement from the following for canal linings :

a. sulphate resisting cement

b. rapid hardening cement

c. quick setting cement

d. pozzuolana cement.

6. In arches, stratified stones are placed so that their planes are

a. parallel

b. perpendicular

c. radial

d. none of these.

7. Plastic bitumen is generally used for

a. road pavements

b. expansion joints

c. crack fillings

d. none to these.

8. Wrought iron contains carbon upto

a. 0.25%

b. 1.0%

c. 1.5%

d. 2%.

9. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

10. The force in BF of the truss shown in given figure, is

a. 4t compression

b. 4.5t tension

c. 4.5t compression

d. zero.

11. As the elastic limit reaches, tensile strain

a. increases more rapidly

b. decreases more rapidly

c. increases in proportion to the stress

d. decreases in proportion to the stress.

12. The ratio of strengths of solid to hollow shafts, both having outside diameter D and hollow having inside diameter D/2, in torsion, is

a. 1/2

b. 1/16

c. 15/16

d. 3/8

13. In C.G.S. system the units of kinematic viscosity, is

a. stoke

b. poise

c. Newton

d. none of these.

14. In a venturimeter, the divergent cone is kept

a. shorter than convergent cone

b. equal to covergent cone

c. longer than convergent cone

d. none of these.

15. An orifice is taken as large if

a. (H2 - H1) > H

b. (H2 - H1) > H/4

c. (H2 - H1) > H/2

d. (H2 - H1) > H/3

16. Pick up the correct statement from the following :

a. Discharge over a triangular notch is proportional to H5/2

b. Discharge over a rectangular notch is proportional to H3/2

c. Both (a) and (b)

d. Neither (a) nor (b).

17. The minimum depth of building foundations on

a. sandy soils is 80 cm to 100 cm

b. clay soils is 90 cm to 160 cm

c. rocky soils is 5 cm to 50 cm

d. all the above.

18. Degree of saturation of a natural soil deposit having water content 15%, specific gravity 2.50 and void ratio 0.5, is

a. 50%

b. 60%

c. 75%

d. 80%

19. Negative skin friction on piles

a. is caused due to relative settlement of the soil

b. is caused in soft clays

c. decreases the pile capacity

d. all of the above.

20. The active earth pressure of a soil is proportional to (where φ is the angle of friction of the soil)

a. tan (45° - φ)

b. tan2 (45° + φ/2)

c. tan2 (45° - φ/2)

d. tan (45° + φ)

21. If a rectangular prestressed beam of an effective span of 5 meters and carrying a total load 3840 kg/m, is designed by the load balancing method, the central dip of the parabolic tendon should be

a. 10 cm

b. 15 cm

c. 20 cm

d. 25 cm

22. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

23. For normal cases, stiffness of a simply supported beam is satisfied if the ratio of its span to its overall depth does not exceed

a. 15

b. 20

c. 25

d. 30

24. If the average bending stress is 6 kg/cm2 for M 150 grade concrete, the length of embedment of a bar of diameter d according to I.S. 456 specifications, is

a. 38 d

b. 48 d

c. 58 d

d. 95 d

25. Pick up the incorrect statement from the following :

a. a long vertical load transferring concrete structure is called a concrete pile

b. in pile which transfers the load to the soil by the friction between the pile and the surrounding soil is called friction pile.

c. the pile which transfers the load to a hard rock bed at certain depth is called load bearing

d. none of the these.

26. An ordinary concrete may be made water proof by adding

a. impermo

b. snowcem

c. cico

d. all of these.

27. Horizontal construction joints in concrete walls are generally provided at

a. soffit level

b. window sill level

c. floor level

d. all the above.

28. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

29. The valve shown in the below figure, is generally known as

a. air valve

b. pressure relief valve

c. sluice valve

d. none of these.

30. Detention time for plain sedimentation tank usually ranges from

a. 4 to 8 hours

b. 6 to 10 hours

c. 8 to 12 hours

d. 12 to 16 hours.

31. For drainage pipes in buildings the test applied before putting them to use, is

a. water test

b. smoke test

c. straightness test

d. all the above.

32. Removal of oil and grease from sewage, is known

a. screening

b. skimming

c. filtration

d. none of these.

33. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

34. The most suitable location of a canal head work, is

a. boulders stage of the river

b. delta stage of the river

c. rock stage of the river

d. trough stage of the river.

35. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

36. For designing the venturi head,

a. width of the throat should not be less than the width obtained from the formula B = Q / 1.2d3/2, where d is the difference in levels of F.S.L. and sill

b. minimum throat width should not be less than 60 cm

c. length of the throat should be 2.5 times the depth measured to F.S.L. in the main canal

d. all the above.

37. Pick up the correct statement from the following:

a. The corss fall of the shoulder should be at least 0.5% steeper than camber

b. On superelevated sections, the shoulders should be provided a cross fall equal to camber

c. Earthern roads in general are provided steepest crossfall

d. All the above.

38. The minimum road width is taken

a. 9 m

b. 12 m

c. 16 m

d. 20 m

39. Over taking time required for a vehicle with design speed 50 km ph and overtaking acceleration 1.25 m/sec2 to overtake a vehicle moving at a speed 30 km ph, is

a. 5.0 secs

b. 6.12 secs

c. 225.48 secs

d. 30 secs

40. Normal formation width of a hill road for one-way traffic, is

a. 4.8 m

b. 6.6 m

c. 7.2 m

d. 8 m

41. The most reliable estiamte is

a. Detailed estimate

b. Preliminary estimate

c. Plinth area estimate

d. Cube rate estimate

42. The order of booking dimensions is

a. Length, breadth, height

b. Breadth, length, height

c. Height, breadth, length

d. None of these.

43. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

44. The value of 'B' of Indian type W.C. shown in the given figure is :

a. 45 cm

b. 50 cm

c. 30 cm

d. 25 cm

45. Which one of the following represents an event ?

a. concrete cured

b. fixing of door

c. plastering of walls

d. selecting sites

46. The main principle of an organisation, is

a. cohegency

b. effective control at all levels

c. delegation of authority

d. all the above.

47. The Overall in-charge of an organisation at the site responsible for the execution of the works, is

a. Engineer

b. Junior Engineer

c. Sub overseer

d. Assistant Engineer.

48. Pick up the correct statement from the following:

a. Programme Evaluation and Review Technique, is event oriented

b. Programme Evaluation and Review Technique is not event oriented

c. Critical Path Method is event oriented

d. Critical Path method is event oriented.

49. Airport elevation is the reduced level above M.S.L. of

a. control tower

b. highest point of the landing area

c. lowest point of the landing area

d. none of these.

50. The length of a runway under standard atmospheric conditions is 1800 m. If the actual reduced level of the site is 1200 m, the design length of the runway is

a. 2360 m

b. 2460 m

c. 2560 m

d. 2660 m