***Sub Engineer***

1. Diurnal variation of magnetic declination is

a. greater at equator than nearer the poles

b. less at equator than nearer the poles

c. less in summer than in winter

d. same at all latitudes and during different months.

2. In case of a double line river, contours are

a. stopped at the banks of the river

b. stopped at the edge of the river

c. drawn across the water

d. drawn by parabolic curves having their vertex at the centre of the water.

3. A level when set up 25 m from peg A and 50 m from peg B reads 2.847 on a staff held on A and 3.462 on a staff held on B, keeping bubble at its centre while reading. If the reduced levels of A and B are 283.665 m and 284.295 m respectively, the collimation error per 100 m is

a. 0.015 m

b. 0.030 m

c. 0.045 m

d. 0.060 m

4. The 10 mm markings on a levelling staff placed at 20 m are separated by

a.  radian

b.  radian

c.  radian

d.  radian.

5. Fibre glass

a. has a higher strength to weight ratio

b. is shock proof and fire retardent

c. does not decay

d. all the above.

6. Pick up the correct statement from the following:

a. The heating of a material to redness in contact with air, is known as calcination

b. The property of lime by which it sets or hardens in damp places having no free circulation of air is called setting

c. The product that remains after calcination of limestone, is called lime

d. All the above.

7. Water paint is a

a. colour wash

b. whiting

c. distemper

d. all the above.

8. To give a brilliant finish, the type of varnish used, is

a. spirit varnish

b. turpentine varnish

c. oil varnish

d. none of these.

9. A three hinged arch is generally hinged at its supports and

a. at one quarter span

b. at the crown

c. any where in the rib

d. none of these.

10. A load of 1960 N is raised at the end of a steel wire. The minimum diameter of the wire so that stress in the wire does not exceed 100 N/mm2 is :

a. 4.5 mm

b. 5.0 mm

c. 5.5 mm

d. 6.0 mm

11. A cantilever carries is uniformly distributed load W over its whole length and a force W acts at its free end upward. The net deflection of the free end will be

a. zero

b.  upward

c.  downward

d. none of these.

12. The property of a material by which it can be drawn to a smaller section, due to tension, is called

a. plasticity

b. ductility

c. elasticity

d. malleability.

13. Most economical section of a circular channel for maximum velocity, is if,

a. depth of water = 0.810 diameter

b. hydraulic mean depth = 0.304 diameter

c. wetted perimeter = 2.245 diameters

d. all the above.

14. In a short cylindercial external mouthpiece, the venacontracta occurs at a distance from the outlet of orifice equal to

a. diameter of the orifice

b. one-fourth the diameter of the orifice

c. one-third the diameter of the orifice

d. two-third the diameter of the orifice

15. A triangular notch is preferred to a rectangular notch because

a. its formula is simple to remember

b. it gives more accurate results for low discharge

c. it measures a wide range of flows accurately

d. all the above.

16. While applying the Bernoulli's equation  = total head, the work any section done on the flow system, if any

a. is added on the right side of the equation

b. is added on the left side of the equation

c. is ignored

d. none of these.

17. The shear resistance of a soil is con-tituted basically of the following component.

a. The frictional resistance to translocation between the individual soil particles at their contact point

b. To the structural relation to displacement of the soil because of the interlocking of the particles

c. Cohesion and adhesion between the surfaces of the soil particles

d. All the above.

18. According to IS : 2720 - 1965, the composition of a dispersing solution used in pipette analysis for determining the size of particles, is

a. sodium-hexametaphosphate 33 g, sodium carbonate 7 g and distilled water one litre

b. sodium-hexametaphosphate 7 g, sodium carbonate 33 g and distilled water one litre

c. sodium-hexametaphosphate 23 g, sodium carbonate 17 g and distilled water one litre

d. none of these.

19. The slip at critical angle, is generally known

a. δ 1-line

b. rupture plane

c. slip plane

d. all the above.

20. A phreatic line is defined as the line within a dam section below which there are

a. positive hydrostatic pressure

b. negative hydrostatic pressure

c. negative equipotential lines

d. none of these.

21. To ensure uniform pressure distribution, the thickness of the foundation, is

a. kept uniform throughout

b. increased gradually towards the edge

c. decreased gradually towards the edge

d. kept zero at the edge.

22. Thickened part of a flat slab over its supporting column, is technically known as

a. drop panel

b. capital

c. column head

d. none of these.

23. The amount of reinformcement for main bars in a slab, is based upon

a. minimum bending moment

b. maximum bending moment

c. maximum shear force

d. minimum shear force.

24. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

25. For brick construction, the lime-sand mortar, is

a. 1:2

b. 1:3

c. 1:4

d. 1:5.

26. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

27. On a grading curve, the gap grading is represented by

a. a horizontal line

b. a vertical line

c. N.W. inclined line

d. N.E. inclined line

28. For batching 1:3:6 concrete mix by volume, the ingredients required per bag of 50 kg cement, are:

a. 70 kg of sand and 140 litres of aggregates

b. 105 litres of sand and 140 litres of aggregates

c. 105 litres of sand and 210 litres of aggregates

d. none of these.

29. E. Coli bacterias die in water having pH greater than

a. 6.5

b. 7.5

c. 8.5

d. 9.5

30. The gaseous form of chlorine gets converted into liquid form when subjected to a pressure of

a. 6 kg/cm2

b. 7 kg/cm2

c. 8 kg/cm2

d. 10 kg/cm2.

31. The reduced levels of the string at the consecutive sight rails A and B are 203.575 m, 203.475 m respectively. If the difference of their R.D.s is 10 m, the gradient of the sewer line is

a. 1 in 100 upward

b. 1 in 500 upward

c. 1 in 100 downward

d. 1 in 503 upward.

32. In trickling filter, B.O.D. is reduced to

a. 40 to 60%

b. 60 to 80%

c. 80 to 90%

d. 95%.

33. The velocity of drainage water in the barrels of a syphon-aqueduct, is normally limited to

a. 1 to 2 m per second

b. 2 to 3 m per second

c. 3 to 4 m per second

d. 4 to 5 m per second.

34. If H and d are the water depth and drop in the bed level at a Sarda fall, the width B of the trapezoidal crest, is given by

a. B = 0.22 H + d

b. B = 0.33 H + d

c. B = 0.44 H + d

d. B = 0.55 H + d

35. The ratio of the head recovered to the head put in, is known as

a. efficiency

b. sensitivity

c. flexibility

d. modular limit.

36. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

37. Roughness index of roads, is expressed as

a. size of the stone on the pavement

b. number of patches on the pavement

c. cumulative deformation of surface per horizontal distance

d. type of the road surface.

38. Hill roads are

a. provided camber along spur curves

b. provided camber along valley curves

c. seldom provided a camber

d. None of these.

39. The head light of vehicles should be such that its lower beam illuminates objects at

a. 20 m

b. 30 m

c. 40 m

d. 50 m

40. The ability of a driver to stop the vehicle moving with the designed speed, depends upon:

a. brake reaction time

b. efficiency of the brakes

c. frictional resistance between road surface and vehicle

d. all the above.

41. In case of laying gullies, siphons, intercepting traps, the cost includes

a. Setting and laying

b. Bed concreting

c. Connection to drains

d. All of these.

42. While preparing a detailed estimate

a. Dimension should be measured correct to 0.01 m

b. Area should be measured correct to 0.01 sqm

c. Volume should be measured correct to O.Olcum

d. All the above.

43. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

44. The measurement is made for stone work in square metre in case of

a. Columns, lintels, copings

b. Building work

c. Dressed stones in Chajjas

d. (a) and (d) of the above.

45. Railway projects are treated as

a. light construction

b. heavy construction

c. industrial construction

d. none of these.

46. Mile Stone charts were invented in the year of

a. 1920

b. 1930

c. 1940

d. 1950

47. Which one of the following represents an activity

a. curing of concrete

b. setting of question paper

c. preparation of breakfast

d. all the above.

48. A critical ratio scheduling

a. determines the status of each activity

b. adjusts automatically changes in activity progress

c. is a dynamic system

d. none of these.

49. Pick up the correct statement from the following:

a. The wind data of an air port is depicted in the form of a chart known as wind rose

b. The landing and take off of the air craft is made against the wind direction

c. The product of wind loading and power loading is called air craft rating

d. All the above.

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.