***Sub Engineer***

1. The least count of a vernier scale is

a. sum of the smallest divisions of main and vernier scales

b. value of one division of the primary scale divided by total number of divisions of vernier scale

c. value of one division of vernier scale divided by total number of divisions of primary scale

d. none of these.

2. While viewing through a level telescope and moving the eye slightly, a relative movement occurs between the image of the levelling staff and the cross hairs. The instrument is

a. correctly focussed

b. not correctly focussed

c. said to have parallax

d. free from parallax.

3. Keeping the instrument height as 1  m, length of staff 4 m, the up gradient of the ground 1 in 10, the sight on the up slope must be less than

a. 25 cm

b. 20 m

c. 45 m

d. 10 m

4. In chain surveying, perpendiculars to the chain line, are set out by

a. a theodolite

b. a prismatic compass

c. a level

d. an optical square

5. Basalt is

a. sedimentary rock

b. metamorphic rock

c. extrusive igneous rock

d. intrusive igneous rock.

6. The rocks which are formed due to cooling of magma at a considerable depth from earth's surface are called

a. Plutonic rocks

b. Hypabyssal rocks

c. Volcanic rocks

d. Igneous rocks.

7. Stones used for the construction of retaining walls must be

a. soft

b. hard

c. light

d. heavy.

8. Second class bricks

a. are of dark brown colour

b. produce a metallic sound when struck

c. are well burnt

d. are under burnt.

9. The yield moment of a cross section is defined as the moment that will just produce the yield stress in

a. the outer most fibre of the section

b. the inner most fibre of the section

c. the neutral fibre of the section

d. the fibre everywhere

10. If a three hinged parabolic arch, (span l, rise h) is carrying a uniformly distributed load w/unit length over the entire span,

a. horizontal thrust is wl2/8h

b. S.F. will be zero throughout

c. B.M. will be zero throughout

d. all the above.

11. The maximum compressive stress at the top of a beam is 1600 kg/cm2 and the corresponding tensile stress at its bottom is 400 kg/cm2 . If the depth of the beam is 10 cm, the neutral axis from the top, is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

12. When loads are applied proportionately to a frame structure containing its members in one plane, the structure is called

a. grid frame

b. plane frame

c. space frame

d. truss frame.

13. Back water curve is caused if

a. friction head loss is more than the bed slope

b. pressure is due to weir in the channel

c. there is an increase in width of the channel

d. none of these.

14. Orifice-meter is used to measure

a. pressure at the point

b. discharge

c. average speed

d. velocity.

15. When two layers of a fluid separated by dy move over the other with a difference of velocity dv, causes a shearing stress , where μ is known as

a. absolute viscosity

b. dynamic viscosity

c. viscosity

d. all the above.

16. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

17. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

18. The inventor of the term soil mechanics, was

a. Kray

b. Dr. Karl Terzaghi

c. Leygue

d. Fellenius.

19. The density of soil can be increased

a. by reducing the space occupied by air

b. by elastic compression of soil grains

c. by expelling water from pores

d. All the above.

20. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

21. An R.C.C beam of 25 cm width has a clear span of 5 metres and carries a U.D.L. of 2000 kg/m inclusive of its self weight. If the lever arm of the section is 45 cm., the beam is

a. safe in shear

b. is safe with stirrups

c. is safe with stirrups and inclined members

d. needs revision of the section.

22. A singly reinforced concrete beam of 25 cm width and 70 cm effective depth is provided with 18.75 cm2 steel. If the modular ratio (m) is 15, the depth of the neutral axis, is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

23. High strength concrete is used in prestressed member

a. to ovecome bursting stresses at the ends

b. to provide high bond stresses

c. to overcome cracks due to shrinkage

d. all the above.

24. In a singly reinforced beam, the effective depth is measured from its compression edge to

a. tensile edge

b. tensile reinforcement

c. neutral axis of the beam

d. longitudinal central axis.

25. Slate

a. is a metamorphic rock

b. splits into thin sheets along its bedding planes

c. has a smooth surface and contains alumina and silica

d. possesses good water absorption capacity

26. The form work from the sides of beams can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days.

27. Non-uniform compaction may cause the concrete

a. porous

b. non-homogeneous

c. reduced strength

d. all the above.

28. For the construction of thin R.C.C. structures, the type of cement to be avoided, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. sulphate resisting cement.

29. Pick up the correct statement from the following :

a. The ratio of the reservoir capacity to the total inflow of water in it, is generally called capacity inflow ratio

b. Small capacity reservoirs on large rivers generally, silt less

c. Large capacity reservoirs on small rivers generally silt more

d. All the above.

30. By adding 1 ml of orthotolidine solution to 100 ml chlorinated water taken after contact period, the residual chlorine makes the colour of solution

a. yellowish

b. greenish

c. bluish

d. reddish

31. For house drainage minimum gradient is

a. 1 in 60

b. 1 in 80

c. 1 in 10

d. 1 in 400

32. Dry weather flow is :

a. average monthly rate of flow

b. average annual rate of flow

c. water supply allowance per capita

d. none of these.

33. In gravity canals, F.S.L. is

a. always at the ground level

b. always below the ground level

c. generally 4 to 5 metres above the ground level

d. only a few cm above the ground level.

34. In a syphon aqueduct

a. drainage passes over the canal and F.S.L. of the canal is below the bottom of the drainage trough

b. drainage passes over the canal and F.S.L. of the canal is above the bottom of the drainage trough

c. canal passes over the drainage and H.F.L. of the drainage is above the bottom of the canal trough

d. canal passes over the drainage and H.F.L. of the drainage is below the bottom of the canal trough.

35. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

36. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

37. For a poorly graded sub-grade soil, thickness of sub-base, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

38. Which one of the following transportation systems is not used for the conveyance of passengers in India?

a. roads

b. shipping

c. air lines

d. pipe lines.

39. While calculating the overtaking sight distance, the height of the object above road surface, is assumed

a. 50 cm

b. 75 cm

c. 100 cm

d. 120 cm

40. The most commonly adopted method to provide super-elevation on roads, is by pivoting the road surface about

a. outer edge so that the inner edge is lowered

b. crown so that outer edge is raised and inner edge is lowered

c. inner edge so that outer edge is raised

d. none of these.

41. While estimating the qualities for the construction of a building, the correct metric unit is

a. Cubic metre for area

b. Square metres for volume

c. Kilogram for weight

d. Litre for capacity

42. Pick up the incorrect statement regarding a master trap from the following :

a. It is provided a cleaning eye at the top of the trap

b. The height of fresh air inlet pipe fixed vertically with wall is 3 m

c. The mica flap valve which opens inwards only, is fitted at the top of the inlet pipe

d. The water seal is less than that of ordinary traps.

43. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

44. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

45. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

46. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

47. Power stations are generally treated as

a. light construction

b. heavy construction

c. industrial construction

d. electrical construction.

48. For completion of a project, the critical path of the network represents

a. minimum time

b. maximum time

c. maximum cost

d. minimum cost.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. To cope up high temperature of 196°C, the taxi ways and aprons are constructed with

a. rubberised tar concrete

b. plain concrete

c. epoxy asphalt concrete

d. all the above.