***Sub Engineer***

1. For a curve of radius 100 m and normal chord 10 m, the Rankine's deflection angle, is

a. 0°35'.95

b. 1°25'.53

c. 1°35'.95

d. 2°51'.53.

2. Closed contours of decreasing values towards their centre, represent

a. a hill

b. a depression

c. a saddle or pass

d. a river bed.

3. Perpendicularity of an offset may be judged by eye, if the length of the offset is

a. 5 m

b. 10 m

c. 15 m

d. 20 m.

4. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

5. Steel contains carbon approximately

a. 1.50% to 5.6%

b. 0.05% to 1.75%

c. 0.25%

d. none to these.

6. A bull nose brick is not used for

a. rounding off sharp corners

b. pillars

c. decoration purpose

d. arches.

7. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

8. Bullet proof glass is made of thick glass sheet sandwiched by a layer of

a. steel

b. stainless steel

c. high test plastic

d. chromium plate.

9. For determining the support reactions at A and B of a three hinged arch, points B and C are joined and produced to intersect the load line at D and a line parallel to the load line through A at D'. Distances AD, DD' and AD' when measured were 4 cm, 3 cm and 5 cm respectively. The angle between the reactions at A and B is

a. 30°

b. 45°

c. 60°

d. 90°

10. A material which obeys Hook's law, is subjected to direct stress σ0. At its elastic limit, the following statement is true,

a. Maximum shear stress =

b. Strain energy =

c. Shear strain energy =

d. All the above.

11. For a given material Young's modulus is 200 GN/m2 and modulus of rigidity is 80 GN/m2. The value of Poisson's ratio is

a. 0.20

b. 0.25

c. 0.30

d. 0.40

12. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

13. Barometres are used to measure

a. difference in pressure at two points

b. atmospheric pressure

c. very low pressure

d. very high pressure.

14. Centre of buoyancy is

a. centroid of the floating body

b. centroid of the fluid displaced

c. centre of pressure of the displaced liquid

d. none of these.

15. Energy equation is usually applicable to

a. non-uniform flow

b. turbulent flow

c. laminar flow

d. steady flow.

16. The best side slope for most economical trapezoidal section, is

a. 30°

b. 45°

c. 60°

d. None of these.

17. Through a point in a loaded soil, the principal stress is maximum on

a. minor principal plane

b. intermediate principal plane

c. major principal plane

d. none of these.

18. A triaxial shear test is prefered to direct shear test, because

a. it can be performed under all three driange conditions with complete control

b. precise measurement of pore pressure and change in volume during test, is not posible

c. stress distribution on the failure plane, is non uniform

d. none of these.

19. If a soil undergoes a change in shape and volume by application of external loads over it, but recovers its shape and volume immediately after removal of the load, the property of the soil is said to be

a. Resilience of soils

b. Elasticity of soils

c. Compressibility of soils

d. None of these.

20. Si particles

a. show dilatancy

b. swell when moist

c. possess high strength when dry

d. disintegrate easily.

21. A short column 20 cm x 20 cm in section is reinforced with 4 bars whose area of cross section is 20 sq. cm. If permissible compressive stresses in concrete and steel are 40 kg/cm2 and 300 kg/cm2, the Safe load on the column, should not exceed

a. 4120 kg

b. 41, 200 kg

c. 412, 000 kg

d. none of these.

22. A flat slab is supported

a. on columns

b. on beams and columns

c. on columns monolithicaily built with slab

d. all the above

23. A continuous beam shall be deemed to be a deep beam if the ratio of effective span to overall depth, is

a. 2.5

b. 2.0

c. less than 2

d. less than 2.5

24. A per IS : 1343, total shrinkage for a pretensioned beam, is

a. 3.0 x 10-3

b. 3.0 x 10-4

c. 3.0 x 10-5

d. 3.5 x 10-5

25. For a wall carrying heavy load on low bearing capacity soil,

a. thick concrete bed is provided

b. reinforced concrete bed is provided

c. (a) and (c) of the above

d. (b) and (c) of the above.

26. The under surface of an arch, is called

a. soffit

b. intrados

c. haunch

d. back.

27. For the construction of cement con-certe dams, the maximum permissible size of the aggregates, is

a. 40 mm

b. 50 mm

c. 60 mm

d. 70 mm

28. The maximum thickness of concrete floor of a cement warehouse, is

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

29. The force which develops in a pressure conduit supported on trestles, is

a. tension

b. compression

c. temperature stress

d. flexural stress.

30. Air inlet valve in water mains, is generally provided at

a. upstream of sluice valve

b. down stream of sluice valve

c. valley of the pipe

d. both (a) and (c) of above.

31. Aerobic activity is maximum

a. in freshly produced sewage

b. at sewer pipes

c. in sewer treatment plants

d. none of these.

32. Which one of the following gases is most significant as air pollutant

a. Carbondioxide

b. Oxygen

c. Nitrogen

d. Sulphurdioxide.

33. Irrigation canals are generally aligned along

a. ridge line

b. contour line

c. valley line

d. straight line.

34. A current meter measures the velocity of flow, if it is held

a. on the surface of water

b. at the bottom of channel

c. at any point within the cross-section

d. none of these.

35. Pick up the correct sequence of the part of a canal system from the following

a. Head work-distributary-branch canal-minor

b. Head works-main canal-branch canal-dis-tributary-minor

c. Head works-main canal-branch canal-minor-distributary

d. Heads works-branch canal-main canal distributary, minor.

36. For smooth entry of water in a canal, the angle between head regulator and water is generally kept

a. 80°

b. 90°

c. 110°

d. 120°

37. Thickness of broken centre line markings for a four lane road, is generally kept

a. 12 cm

b. 15 cm

c. 18 cm

d. 20 cm

38. The ideal shape of a transition curve, is

a. clothoid

b. cubic spiral

c. cubic parabola

d. lamniscate

39. Shoulders for high traffic volume roads, should

a. be stable throughout the year to be used by vehicles in the case of emergency

b. support and protect the ends of carriage ways

c. not allow entrance of water to sub-grade

d. all the above.

40. A district road with a bituminous pavement has a horizontal curve of 1000 m for a design speed of 75 km ph. The super-elevation is

a. 1 in 40

b. 1 in 50

c. 1 in 60

d. 1 in 70

41. The height of the sink of wash basin above floor level is kept

a. 60 cm

b. 70 cm

c. 75 cm to 80 cm

d. 80 cm

42. Berms are provided in canals if these are

a. fully in excavation

b. partly in excavtion and partly in embankment

c. fully in embankment

d. All the above.

43. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

44. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

45. An Executive Engineer may have powers upto

a. Rs. 25,000

b. Rs. 50,000

c. Rs. 100,000

d. Rs. 200,000

46. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

47. The most popular type of organisation used for Civil Engineering Constructions, is

a. line organisation

b. line and staff organisation

c. functional organisation

d. effective organisation.

48. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

49. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.

50. Pick up the correct statement from the following:

a. The distance between the points of intersection of the extreme tangents to the transition curve is kept greater than 7500 m x sum of grade changes at the point of intersection

b. The rate of change of grade is limited to 0.3% per 30 m length of the curve

c. According to I.C.A.O. the maximum longitudinal gradient along a runway is limited to 1.5%

d. All the above.