***Sub Engineer***

1. There are two stations A and B. Which of the following statements is correct :

a. the fore bearing of AB is AB

b. the back bearing of AB is BA

c. the fore and back bearings of AB differ by 180°

d. all the above.

2. Surveys which are carried out to depict mountains, rivers, water bodies, wooded areas and other cultural details, are known as

a. city surveys

b. topographical surveys

c. guide map surveys

d. plane surveys.

3. Geodetic surveying is undertaken

a. for developing the science of geodesy

b. making use of most accurate instruments and methods of observation

c. for determination of accurate positions on the earth's surface of system of control points

d. all the above.

4. The limiting length of an offset does not depend upon

a. accuracy of the work

b. method of setting out perpendiculars

c. scale of plotting

d. indefinite features to be surveyed.

5. Geologically, marble is known as

a. sedimentary rock

b. igneous rock

c. metamorphic rock

d. stratified rock.

6. The percentage of alumina and silica in good fire clay vary respectively is

a. 25, 75

b. 30, 70

c. 35, 65

d. All of these.

7. Acrylic is the name of

a. cellulose resin

b. alkyd resin

c. methyl metha crylate

d. cumarone-indene.

8. Quartzite, a metamorphic stone is

a. brittle

b. crystalline

c. compact

d. All the above.

9. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

10. Beams composed of more than one material, rigidly connected together so as to behave as one piece, are known as

a. Compound beams

b. Indeterminate beams

c. Determinate beams

d. Composite beams.

11. Pick up the correct assumption of the theory of simple bending

a. Transverse section of a beam remains plane before and after bending

b. The material of the beam is homogeneous and isotropic

c. The resultant pull or thrust on transverse section of a beam is zero

d. All the above.

12. A diagram which shows the variations of the axial load for all sections of the span of a beam, is called

a. shear force diagram

b. thrust diagram

c. stress diagram

d. none of these.

13. A cylinder 3 m in diameter and 4 m long retains water one side as shown in the below figure. If the weight of the cylinder is 2000 kgf, the vertical reaction at A is

a. 14,137 kgf

b. 5,863 kgf

c. 20,000 kgf

d. 18,000 kgf.

14. Dimensions of the dynamic viscosity (μ) are

a. MLT-2

b. M-1L-1T-1

c. ML-1T-2

d. ML-1T-1.

15. Gauge pressure is

a. absolute pressure - atmospheric pressure

b. absolute pressure + atmospheric pressure

c. atmospheric pressure - absolute pressure

d. none of these.

16. The gases are considered incompressible if Match number is

a. equal to 1.0

b. equal to 1.5

c. is more than 0.5

d. less than 0.2

17. In non-cohesive soil in passive state of plastic equilibrium

a. major principal stress is horizontal

b. minor principal stress is vertical

c. major principal stress is vertical

d. minor and major principal stresses are equally inclined to the horizontal.

18. Rankine's theory of active earth pressure assumes

a. ground surface is a plane which may be horizontal or inclined

b. back of the wall is vertical and smooth

c. wall yields about the base

d. all the above.

19. The length/diameter ratio of cylindrical specimens used in triaxial test, is generally

a. 1.5

b. 2

c. 2.5

d. 3

20. If dry density, water density and specific gravity of solids of a given soil sample are 1.6 g/cc, 1.84 g/cc and 2.56 respectively, the porosity of the soil sample, is

a. 0.375

b. 0.370

c. 0.380

d. 0.390

21. Minimum spacing between horizontal parallel reinforcement of different sizes, should not be less than

a. one diameter of thicker bar

b. sum of the diameters of ihinner and thicker bars

c. twice the diameter of thinner bar

d. none of these.

22. The load stress of a section can be reduced by

a. increasing the total perimeter of bars

b. replacing larger bars by greater number of small bars

c. replacing smaller bars by greater number of greater bars

d. none of these.

23. In a slab, the pitch of the main reinforcement should not exceed its effective depth

a. three times

b. four times

c. five times

d. two times.

24. Top bars are extended to the projecting parts of the combined footing of two columns L distance apart for a distance of

a. 0.1 L from the outer edge of column

b. 0.1 L from the centre edge of column

c. half the distance of projection

d. one-fourth the distance of projection.

25. The maximum permissible deflection of a timber beam supporting a roof, is

a. L/150

b. L/260

c. L/360

d. none of these.

26. The window which is provided on a sloping roof of a building, is called

a. dormer window

b. louvered window

c. rash window

d. air window.

27. Concrete containing

a. silicious aggregates, has higher co-efficient of expansion

b. igneous aggregates, has intermediate coefficient of expansion

c. lime stones, has lowest co-efficient of expansion

d. All the above.

28. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

29. Before constructing a dam, the factor to be considered for controlling sedimentation, is

a. construction of check dams

b. providing vegetation screens

c. providing under-sluices in the dam

d. All the above.

30. To control the growth of algae in reservoirs, the compound which is used, is

a. copper sulphate

b. lime solution

c. alum solution

d. all the above.

31. The factor responsible for purification of sewage in river is

a. Dissolved oxygen in water

b. Temperature

c. Turbulence

d. All the above.

32. If the discharge of a sewer running half is 628 1.p.s., i = 0.001, and n = 0.010, the diameter of the sewer, is

a. 1.49 m

b. 1.59 m

c. 1.69 m

d. 1.79 m.

33. The structure constructed to allow drainage water to flow under pressure through an inverted syphon below a canal, is called

a. super passage

b. aqueduct

c. super-aqueduct

d. syphon aqueduct.

34. The field capacity of a soil is 25%, its permanent wilting point is 15% and specific dry unity weight is 1.5. If the depth of root zone of a crop, is 80 cm, the storage capacity of the soil, is

a. 10 cm

b. 12 cm

c. 14 cm

d. 16 cm

35. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

36. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

37. The ratio of maximum deviation angle and maximum polar deflection angle of a Lemniscate curve, is

a. 3

b. 4

c. 5

d. 6

38. According to Indian Road Congress, the width of carriageway, is

a. 7.0 m for two lanes without raised kerbs

b. 7.5 m for two lanes with raised kerbs

c. 10.5 m for three lanes pavements

d. All the above

39. If the radius of curvature of a hill road is 50 m, the percentage grade compensation should be

a. 70/R

b. 75/R

c. 80/R

d. 100/R

40. To prevent compressive stresses in a rigid concrete pavement, the joint prvoided, is

a. expansion joint

b. contraction joint

c. hinged joint

d. all the above.

41. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

42. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

43. The 'centre line method' is specially adopted for estimating

a. Hexagonal buildings

b. Octagonal buildings

c. Other geometrical shaped buildings

d. All the above.

44. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

45. The technique for establishing and maintaining priorities among the various jobs of a project, is known

a. Event flow scheduling technique

b. Critical ratio scheduling

c. Slotting technique for scheduling

d. Short interval scheduling.

46. If the total float and duration of an activity are 5 and 10 days respectively, the particular activity can be

a. started 5 days later

b. completed 5 days later

c. performed at slower rate in 15 days

d. all the above.

47. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

48. Site order book is used for recording

a. instructions by the executive engineers

b. construction measurements

c. issue of store equipments

d. names of the casual labour.

49. The reduced level of the proposed site of an air port is 2500 m above M.S.L. If the recommended length by I.C.A.O. for the runway at sea level is 2500 m, the required length of the runway is

a. 2500 m

b. 3725 m

c. 3000 m

d. 3250 m

50. Effective length of a runway is the distance between

a. ends of the runway

b. point of intersection of the obstruction clearance line and the extended plane of the runway surface, and the other end of the runway

c. point of intersection of the glide path and the extended plane of the runway surface and the other end of the runway

d. ends of the clear way on either side.