***Sub Engineer***

1. The difference of level between a point below the plane of sight and one above, is the sum of two staff readings and an error would be produced equal to

a. the distance between the zero of gradient and the foot of the staff

b. twice the distance between the zero of graduation and the foot of the staff

c. thrice the distance between the zero of graduation and the foot of the staff

d. none of the above.

2. ABCD is a regular parallelogram plot of land whose angle BAD is 60°. If the bearing of the line AB is 30°, the bearing of CD, is

a. 90°

b. 120°

c. 210°

d. 270°

3. The direction of steepest slope on a contour, is

a. along the contour

b. at an angle of 45° to the contour

c. at right angles to the contour

d. none of these.

4. Under ordinary conditions, the precision of a theodolite traverse is affected by

a. systematic angular errors

b. accidental linear errors

c. systematic linear errors

d. accidental angular errors.

5. The most valuable timber may be obtained from

a. chir

b. shisham

c. sal

d. teak.

6. According to ISI, bitumen is classified into

a. 4 grades

b. 6 grades

c. 8 grades.

d. 10 grades.

7. Dextrin is

a. starch glue

b. albumin glue

c. rubber based adhesive

d. none to these.

8. For the manufacture of plywood, veneers are placed so that grains of adjacent veneers

a. run at right angles

b. parallel

c. inclined at 45°

d. inclined at 60°.

9. A compound truss may be formed by connecting two simple rigid frames, by

a. two bars

b. three bars

c. three parallel bars

d. three bars intersecting at a point.

10. Pick up the correct statement from the following:

a. The pitch of a close coil spring, is very small

b. The angle made by the coil with horizontal, is called the angle of helix

c. In the open coil helical spring, the angle of helix is comparatively large

d. All the above.

11. The width b and depth d of a beam cut from a wooden cylindrical log of 100 cm diameter for maximum strength are :

a. b = 57.73 cm d = 81.65 cm

b. b = 81.65 cm d = 57.73 cm

c. b = 50.00 cm d = 50.00 cm

d. b = 40.00 cm d = 80.00 cm

12. Failure of riverted joints is due to

a. Tearing of plates between rivets

b. Shearing of rivets

c. Crushing of rivets

d. All the above.

13. If v1 and v2 are the velocities of flow before and after sudden enlargement in a pipe, the head loss given by Carnot and Borda equation, is

a. (v22 - v12)/2g

b. (v1 - v2)2/2g

c. (v2 - v1)2/2g

d. (v1 + v2)2/2g.

14. For uniform flow in canals

a. there is a balance between the frictional loss and drop in elevation of the channel

b. bed and free water surfaces of a channel are parallel to each other

c. bed of channel represents the hydraulic gradient

d. all the above.

15. The total pressure force on a plane area is equal to the area multiplied by the intensity of pressure at its centroid, if

a. area is horizontal

b. area is vertical

c. area is inclined

d. all the above.

16. Poise is the unit of

a. viscosity

b. velocity gradient

c. mass density

d. kinematic viscous.

17. The internal molecular attraction of a soil, the cohesion

a. decreases as the moisture content increases

b. increases as the moisture content decreases

c. is more in well compacted clays

d. depends upon the external applied load.

18. 'Loess' is silty clay formed by the action of

a. water

b. glacier

c. wind

d. gravitational force.

19. Pick up the in-correct statement from the following: The soils which contain montmorillonite minerals

a. shrink more when dry

b. possess high plasticity

c. possess high coefficient of internal coefficient

d. None of these.

20. The degree of saturation of the soil sample stated in Q. No, 216, is

a. 60%

b. 62%

c. 64%

d. 66%

21. The thickness of the flange of a Tee beam of a ribbed slab is assumed as

a. width of the rib

b. depth of the rib

c. thickness of the concrete topping 0d) half the thickness of the rib

d. twice the width of the rib.

22. Cantilever retaining walls can safely be used for a height not more than

a. 4 m

b. 5 m

c. 6 m

d. 8 m

23. A pre-stressed concrete member is preferred because

a. its dimensions are not decided from the diagonal tensile stress

b. large size of long beams carrying large shear force need not be adopted

c. removal of cracks in the members due to shrinkage

d. all the above.

24. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

25. In case of multi-storeyed buildings, the forms to be removed first are

a. sides of beams and girders

b. column forms

c. bottom of beams and girders

d. all the above at the same time.

26. Vertical construction joints are provided where the shearing forces are minimum in the case of

a. slabs

b. beams

c. girders

d. all of these.

27. The specifications of a cement bag for storage, are

a. height 18 cm

b. plan area 3000 sq. cm

c. volume 35 litres

d. all the above.

28. An ideal ware house, is provided

a. water proof roof

b. few windows which remain generally closed

c. 15 cm thick concrete floor laid on a dry course of soling

d. all the above.

29. Cast iron pipes

a. resist corrosion satisfactorily

b. may last for 100 years

c. are normally manufactured in lengths of about 3.5 m

d. all the above.

30. Quality of water is said to be good if it is

a. colourless

b. free from pathogenic organism

c. tasteless

d. all the above.

31. Pick up the correct statement from the following :

a. The circular sewers are provided for separate sewerage system

b. The circular sewers work efficiently if the sections run at least half full

c. Two sewers of different shapes are said to be hydraulically equivalent if they discharge at the same rate while running full on the same grade

d. All the above.

32. The standard B.O.D. of water is taken for

a. 2 days

b. 3 days

c. 5 days

d. 10 days.

33. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

34. In case of a trapezoidal notch fall

a. top length of the piers should not be less than their thickness

b. splay up stream from the notch section is 45°

c. splay down stream from the notch section is

d. all the above.

35. A counter berm is

a. a horizontal benching provided on the inside slope

b. a horizontal benching provided on the outside slope

c. a vertical benching provided on the outer edge of the bank

d. a vertical benching provided on the inner edge of the bank.

36. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

37. Reconnaissance is best done with the help of

a. aerial photographic survey

b. condastral surveys

c. topographical surveys

d. triangulation surveys

38. Pick up the correct statement from the following:

a. Borrow pits are located out side the right of way

b. Borrow pits may be located on either side of the right of way

c. Spoil bank is located on one side of the right of way

d. All the above.

39. The pavement width of a road depends upon

a. terrain

b. type of traffic

c. number of lanes

d. all the above.

40. The difference in gradients after full super-elevation and the initial alignment of a road, is known as

a. rising gradient

b. compensated gradient

c. differential gradient

d. none of these.

41. Pick up the incorrect statement from the following:

a. No deduction is made for the openings upto 0.1 sq.m

b. No deduction is made for volumes occupied by pipes, not exceeding 100 sq.cm in cross-section

c. No deductions are made for the ends of dissimilar structures up to 500 sq.cm cross-sectional area

d. None of these.

42. The plinth area of a building not includes

a. area of stair cover

b. internal shaft for sanitary installations up to 2 sq m. in area

c. lift and wall including landing

d. area of cantilevered porch.

43. Pick up the correct statement from the following:

a. The diameter of the pipes is the nominal diameter of internal bore

b. All pipes are measured along the centre line of the pipes in metres

c. Lead caulked joints are enumerated separately

d. All the above.

44. The item of steel work which is measured in sq.m, is

a. Rolling shutters

b. Steel doors

c. Ventilators and glazing.

d. All the above.

45. Residential buildings are treated as

a. light construction

b. heavy construction

c. industrial construction

d. private construction.

46. Various activities of a project, are shown on bar charts by

a. vertical lines

b. horizontal lines

c. dots

d. crosses.

47. While scheduling a project by C.P.M.

a. required time for each activity is established

b. sequence of various activities is made according to their importance

c. net work is drawn by connecting the activities and the events

d. All the above.

48. Pick up the correct statement from the following:

a. Latest occurrence time is denoted by TL

b. Contractual obligation time is denoted by Ts

c. Latest occurrence time is taken as contractual obligation time

d. All the above.

49. Pick up the correct statement from the following:

a. Approximate geometric centre of the landing area, is called air port reference point

b. The boundaries of horizontal surface and conical surface are marked with reference to air port reference point

c. The location of the air port on a map is the position of air port reference point

d. All the above.

50. Pick up the correct statement from the following:

a. Landing speed is directly proportional to the wing loading

b. Wing loading remaining constant, the take off distance is directly proportional to the powder loading

c. Neither (a) nor (b)

d. Both (a) and (b)