***Sub Engineer***

1. Pick up the correct statement from the following :

a. The vertical distance between a maximum ordinate and the next forward maximum ordinate represents the whole volume of the embankment

b. The vertical distance between a minimum ordinate and the next forward maximum ordinate represents the whole volume of a cutting

c. The area enclosed by a loop of the curve and balancing line, measures the haul in that direction.

d. all the above.

2. The reduced level of a floor is 99.995 m, the staff reading on the floor is 1.505 m. If the inverted staff reading against the roof is 1.795 m, the floor level below the slab, is

a. 3.290 m

b. 3.300 m

c. 3.275 m

d. 2.790 m

3. If the chain line which runs along N-S direction is horizontal and the ground in E-W direction is sloping

a. it is possible to set offsets correctly on east side

b. it is possible to set offsets correctly on east side

c. it is not possible to set offsets correctly on west side

d. it is possible to set offsets correctly on both sides.

4. The line of collimation method of reduction of levels, does not provide a check on

a. intermediate sights

b. fore sights

c. back sights

d. reduced levels.

5. Good quality stones must

a. be durable

b. be free from clay

c. resist action of acids

d. all the above.

6. The slag which floats on the surface of the molten iron generally contains

a. Silica (SiO2) 35%

b. Alumina (Al2O3) 12%

c. MgO, CaSO4, KMnO2 and FeO 8%

d. all the above.

7. For lime concrete,

a. flexural strength at 90 days is 0.2 N/mm2

b. compressive strength at 90 days is 1.5 N/mm2

c. compressive strength at 26 days is 1.2 N/mm2

d. all the above.

8. The plastics made from cellulose resin

a. are as clear as glass

b. are tough and strong

c. possess excellent electrical properties

d. All the above.

9. The force in BC of the truss shown in the given figure, is

a. 3.0t tension

b. t tension

c. t compression

d. None of these.

10. Principal planes are subjected to

a. normal stresses only

b. tangential stresses only

c. normal stresses as well as tangential stresses

d. none of these.

11. The phenomenon of slow extension of materials having constant load, i.e. increasing with the time is called

a. creeping

b. yielding

c. breaking

d. none of these.

12. n and j are numbers of members and joints in a frame. It contains redundant members if

a. n = 3j - 2

b. n < 2j - 3

c. n < j - 2

d. n > 2j - 3

13. Orifice-meter is used to measure

a. pressure at the point

b. discharge

c. average speed

d. velocity.

14. If Cv, Cc, Cd and Cr are the hydraulic coefficients of an orifice, then

a. Cd = Cc . Cv

b. Cr = 1 + Cv2/Cd

c. Cv = Cc + Cd

d. Cc = Cv/Cd

15. The following is not a laminar flow

a. Flow in beds in ground water

b. Rise of water in plants through roots

c. Flow of blood in the arteries of human body

d. Flow in water pipe lines.

16. In two dimensional flow the components of velocity are given by u = ax; v = by. The stream lines will be

a. circular

b. parabolic

c. hyperbolic

d. elliptical.

17. If the specific gravity of a soil particle of 0.05 cm diameter is 2.67, its terminal velocity while settling in distilled water of viscosity, 0.01 poise, is

a. 0.2225 cm/sec

b. 0.2250 cm/sec

c. 0.2275 cm/sec

d. 0.2300 cm/sec

18. The maximum shear stress occurs on the filament which makes an angle with the horizontal plane equal to

a. 30°

b. 45°

c. 60°

d. 90°

19. Pick up the in-correct statement from the following: The soils which contain montmorillonite minerals

a. shrink more when dry

b. possess high plasticity

c. possess high coefficient of internal coefficient

d. None of these.

20. Failure of a slope occurs only when total shear force is

a. equal to total shearing strength

b. greater than total shearing strength

c. less than total shearing strength

d. none of these.

21. A column is regarded as long column if the ratio of its effective length and lateral dimension, exceeds

a. 15

b. 20

c. 25

d. 30

22. Dimensions of a beam need be changed if the shear stress is more than

a. 10 kg/cm2

b. 15 kg/cm2

c. 20 kg/cm2

d. 25 kg/cm2

23. If the sides of a slab simply supported on edges and spanning in two directions are equal, the maximum bending moment is multiplied by

a. 0.3

b. 0.4

c. 0.5

d. 0.7

24. The diameter of longitudinal bars of a column should never be less than

a. 8 mm

b. 10 mm

c. 12 mm

d. none of these.

25. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

26. Cast iron piles

a. are suitable for works under sea water

b. resist shocks or vibrations

c. are suitable for use as batter piles

d. are useful for heavy vertical loads.

27. If the engineer-in-charge approves, the 10 cm cubes may be used for the work test of concrete provided maximum nominal size of aggregate, does not exceed

a. 15 cm

b. 20 cm

c. 25 cm

d. 30 cm

28. Addition of pozzolana to ordinary port land cement, causes

a. reduction in chemical action with sulphates

b. increase in shrinkage

c. reduction bleeding

d. all the above.

29. Alum is chemically

a. Copper sulphate

b. Aluminium sulphate

c. Ferrous sulphate

d. Ferric sulphate.

30. Pick up the correct statement from the following :

a. The ratio of volume drained to the total volume of material drained, is known as specific yield

b. The volume of water retained by the soil against the pull of gravity, is called specific retention or field capacity

c. The sum of specific yield and specific retention, is equal to porosity

d. All the above.

31. The small sewers are cleaned by

a. flushing

b. cane rodding

c. wooden pills

d. none of these.

32. Pick up the incorrect statement from the following :

a. Septic tanks are generally provided a detention period of 12 to 36 hours

b. Septic tanks are completely covered and high vent shafts are provided for the escape of foul gases

c. Septic tanks are generally classified as tanks which work on the principle of anaerobic decomposition

d. None of these.

33. The depth of the crest of a scouring sluice below the crest of a head regulator, is generally kept

a. 1.20 m

b. 2.20 m

c. 3.20 m

d. 4.90 m

34. Pick up the correct statement from the following:

a. If the setting of an outlet is higher than that required for proportionality, the outlet is hyper-proportional

b. If the setting of an outlet is lower than that required for proportionality, the outlet is sub-proportional

c. If the flexibility is zero, it is a rigid module

d. All the above.

35. A hydraulic jump is generally formed when a stream moving with

a. a hyper-critical velocity meets a stream moving with a critical velocity

b. a hyper-critical velocity meets a stream moving with a hyper-critical velocity

c. A hyper-critical velocity meets a stream moving with a sub-critical velocity

d. a sub-critical velocity meets a stream moving with a hyper-critical velocity.

36. If average particle size of the silt in millimetres is m, the Lacey's silt factor f is proportional to

a. m3

b. m1/2

c. m1/3

d. m2/3

37. Round-abouts are not suitable if number of vehicles exceed

a. 3000

b. 4000

c. 5000

d. 6000

38. To ensure that bullock carts may not overturn on curves, the maximum value of super-elevation, recommended by I.R.C., is

a. 1 in 12

b. 1 in 15

c. 1 in 20

d. 1 in 25

39. Pick up the incorrect statement from the following. The width of the right-of-way is decided so as to accommodate

a. side slopes

b. horizontal curve

c. vertical curve

d. provision of future widening.

40. Pick up the correct statement from the following:

a. Seasonal cycle of traffic volume during April and November, is usually near the annual average

b. Mid-winter seasonal cycle of traffic is least

c. Mid-summer seasonal cycle of traffic is highest

d. All the above.

41. The expected out turn of 2.5 cm cement concrete floor per manson per day

a. 2.5 sqm

b. 5.0 sqm

c. 7.5 sqm

d. 10 sqm

42. Pick up the incorrect statement from the following:

a. Lead is the average horizontal straight distance between the borrow pit and the place of spreading soil

b. The lead is calculated for each block of the excavated area

c. The unit of lead is 50 m for a distance upto 500 m

d. The unit of lead is 1 km where the lead exceeds 2 km.

43. For 100 sq. m cement concrete (1 : 2: 4) 4 cm thick floor, the quantity of cement required, is

a. 0.90 m3

b. 0.94 m3

c. 0.98 m3

d. 1.00 m3

44. Cost of fittings and their fixing is specified for the following sanitary fittings

a. Flushing pipes

b. Lavatary basins

c. Sinks

d. All the above.

45. Frequency distribution curves

a. having a single lump, are called uninodal curves

b. if symmetrical, are called normal curves

c. if not symmetrical, are called skew curves

d. all the above.

46. The salient feature of functional organisation is

a. separation of planning and design part

b. each individual maintains functional efficiency

c. work is properly planned and distributed

d. all the above.

47. Critical Path Net Work helps an engineer

a. to concentrate his attention on critical activities

b. to divert the resources from non-critical advanced activities to critical activities

c. to be cautious for avoiding any delay in the critical activities to avoid delay of the whole project

d. all the above.

48. If TL is the latest allowable event occurrence time, total activity slack(s), is equal to

a. LST-EST

b. LFT-EFT

c. TL-EFT

d. all the above.

49. The best direction of a runway is along the direction of

a. longest line on wind rose diagram

b. shortest line on the wind rose diagram

c. line clear of wind rose diagram

d. none of these.

50. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.