***Sub Engineer***

1. Which one of the following procedures for getting accurate orientation is the most distinctive feature of the art of plane tabling

a. radiation

b. intersection

c. traversing

d. resection.

2. Stadia techeometry was discovered by James Watt in the year.

a. 1670

b. 1770

c. 1870

d. 1900.

3. If the length of a chain line along a slope of θ° is l, the required slope correction is

a. 2l cos2 θ/2

b. 2l sin2 θ/2

c. l tan2 θ/2

d. l cos2 θ/2.

4. The bearing of C from A is N 30° E and from B, 50 metres east of A, is N 60° W. The departure of C from A is

a. 50 m

b. 50 3 m

c. 25 3 m

d. 25 m

5. Age of a tree may be ascertained by

a. radius of its stem

b. circumference of its stem

c. number of branches

d. number of annual rings.

6. Pick up the correct statement from the following:

a. Styrene resin is light in weight

b. Styrene resin transmits ultraviolet waves of light

c. Styrene resin is used to manufacture utensils which are unaffected by chemicals

d. All the above.

7. Lime putty

a. is made by adding lime to water

b. can be used only upto three days

c. may be obtained from drying lime water mix passing through IS sieve No. 300

d. all of above.

8. The most commonly used base for timber painting, is

a. red lead

b. zinc white

c. white lead

d. titanium white.

9. A body is said to be in equilibrium if

a. it moves horizontally

b. it moves vertically

c. it rotates about its C.G.

d. none of these.

10. In case of principal axes of a section

a. sum of moment of inertia is zero

b. difference of moment inertia is zero

c. product of moment of inertia is zero

d. none of these.

11. For a cantilever with a uniformly distributed load W over its entire length L, the maximum bending moment is

a.  WL

b.  WL

c.  W2L

d.  WL2

12. In a solid arch, shear force acts

a. along the axis of the arch

b. perpendicular to the axis of arch

c. tangentially to the arch

d. none of these.

13. In an open tube, free surface of mercury remains

a. horizontal

b. curved upwards

c. curved downwards

d. none of these.

14. A fluid particle may possess the displacement of

a. translation

b. rotation

c. distortion

d. all the above.

15. Non-over flow double curvature concrete arch, is provided in

a. Bhakra dam

b. Hirakund dam

c. Nagarjuna Sagar dam

d. Idukki dam.

16. A piezometer opening in pipes measures

a. velocity head

b. static pressure

c. total pressure

d. negative static pressure.

17. For shear strength, triaxial shear test is suitable because

a. precise measurement of the pore pressure and volume change during the test is possible

b. stress distribution on the failure plane, is uniform

c. state of stress within the specimen during any stage of the test as well as at failure, is completely determined

d. all the above.

18. Tergazhi's theory of one dimensional consolidation assumes

a. coefficient of permeability is constant

b. excess pore water drains out only in the vertical direction

c. time lag in consolidation is due entirely to permeability

d. all the above.

19. During seepage through a soil, direction of seepage is always

a. parallel to equipotential lines

b. perpendicular to stream lines

c. perpendicular to equipotential lines

d. none of these.

20. A critical hydraulic gradient may occur when

a. flow is in upward direction

b. seepage pressure is in upward direction

c. effective pressure is zero

d. all the above.

21. The weight of a foundation is assumed as

a. 5% of wall weight

b. 7% of wall weight

c. 10% of wall weight

d. 12% of wall weight

22. A singly reinforced concrete beam of 25 cm width and 70 cm effective depth is provided with 18.75 cm2 steel. If the modular ratio (m) is 15, the depth of the neutral axis, is

a. 25 cm

b. 30 cm

c. 35 cm

d. 40 cm

23. In a simply supported slab the minimum spacing of distribution reinforcement, should be four times the effective thickness of the slab or

a. 30 cm

b. 40 cm

c. 50 cm

d. 60 cm

24. Pick up the correct statement from the following:

a. A pile is a slender member which transfers the load through its lower end on a strong strata

b. A pile is a slender member which transfers its load to the surrounding soil

c. A pile is a slender member which transfers its load by friction

d. A pile is a cylindrical body of concrete which transfers the load at a depth greater than its width.

25. Vertical construction joints are provided where the shearing forces are minimum in the case of

a. slabs

b. beams

c. girders

d. all of these.

26. In high mountaneous region, the type of roof generally recommended for buildings, is

a. shed type

b. gable type

c. gambrel type

d. mansord type.

27. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

28. For concreting tunnel linings, tran-portation of concrete is done by

a. wheel borrows

b. containers

c. pumps

d. belt conveyors.

29. Water may not contain much impurities if its source is

a. stream flowing in plains

b. lakes in lower regions

c. spring along hill slopes

d. none of these.

30. Pipes are laid parallel

a. to increase the capacity of the water supply

b. to provide a means of repairing without closing water supply

c. to meet the requirement of excessive supply

d. all the above.

31. For evaporation and measurement of settlable solids, the apparatus used, is

a. a jar

b. a breaker

c. a test tube

d. an Imhoff cone.

32. In case of sewer lines

a. water test is carried out to check water tightness of the joints

b. test for straightness is carried out with the help of a lamp and mirror

c. obstruction test is carried out with the. help of smooth ball of diameter 13 mm

d. all the above.

33. If the optimum depth of kor watering for a crop is 15.12 cm, the outlet factor for the crop for four week period in hectares per cumec, is

a. 1200

b. 1400

c. 1600

d. 2000

34. A river is said to be of

a. aggrading type if it builds up its bed to a certain slope

b. degrading type if it cuts its bed to a certain slope

c. meandering type if it flows in sinuous curve

d. all the above.

35. Pick up the correct statement from the following

a. In a level crossing, a crest with its top at the canal F.S.L. is provided across the drainage at its up-stream junction with canal

b. In a level crossing a regulator is provided across the drainage at its down-stream

c. In a level crossing, a cross regulator is provided on the canal below the crossing

d. All the above.

36. Pick up the incorrect statement from the following. Culturable commanded area is the gross area of an irrigation canal system less

a. populated area

b. alkaline area

c. forest area

d. fallow land.

37. Pick up the correct statement from the following:

a. During reconnaissance, the general route of the alignment is selected

b. After reconnaissance, a trace is cut for the alignment

c. Last stage is the detailed surveys for desired geometries'of the highway

d. All the above.

38. For the movement of vehicles at an intersection of two roads, without any interference, the type of grade separator generally preferred to, is

a. delta

b. trumpet

c. diamond interchange

d. clover leaf.

39. Width of a rotary round should be equal to

a. twice the width of narrowest radial road

b. width of the widest road

c. width of the widest road plus the width of one lane

d. width of the widest road plus 2 metres.

40. Alignment of highways in hilly regions, is decided on

a. number of river crossings

b. natural unstable areas

c. saddles or passes

d. all the above.

41. The detention period in a septic tank is assumed

a. 20 minutes

b. 25 minutes

c. 30 minutes

d. 40 minutes

42. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

43. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

44. According to Indian Standards Institute, the actual size of modular bricks is

a. 23 cm x 11.5 cm x 7.5 cm

b. 25 cm x 13 cm x 7.5 cm

c. 19 cm x 9 cm x 9 cm

d. 20 cm x 10 cm x 10 cm

45. A dummy activity

a. is artificially introduced

b. is represented by a dotted line

c. does not consume time

d. all the above.

46. The performance of a specific task in CPM, is known

a. Dummy

b. Event

c. Activity

d. Contract.

47. Pick up the incorrect statement from the following:

a. The difference between the maximum time available for the job and actual time it consumes, is called total float

b. The difference between the latest start time and earliest start time of an activity, is called total float

c. The difference between the earliest finish time of an activity and the earliest start time of its successor activity, is called free float of the activity

d. None of these.

48. The three time estimates for the activities of the network shown in the given figure are shown above their arrows. The earliest expected time for the event 4, is

a. 19

b. 14

c. 24

d. None of these.

49. The landing and take off of the air craft is made against the direction of wind. In no case the centre line of the runway should make an angle with the wind direction exceeding

a. 10°

b. 20°

c. 30°

d. 40°

50. If lift off distance of an craft is 2000 m, the clear way at the end of the runway should not be less than

a. 145 m

b. 152.5 m

c. 162.5 m

d. 172.5 m