***Sub Engineer***

1. The difference in the lengths of an arc and its subtended chord on the earth surface for a distance of 18.2 km, is only

a. 1 cm

b. 5 cm

c. 10 cm

d. 100 cm.

2. In case of reduction of levels by the height of instrument method,

a. ∑ B.S. - ∑ F.S. = difference in R.L.S of the first station and last station

b. ∑ (R.L. + I + F.S.) - first R.L = ∑ (H.I. + No. of R.L.s.)

c. both (a) and (b) above

d. neither (a) nor (b).

3. If vertical angles of inclined sights do not exceed 10° and non-verticality of the staff remains within 1°, stadia system of tacheometric observations are made on

a. staff normal

b. staff vertical

c. staff normal as well as vertical

d. none of these.

4. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

5. French polish is

a. oil paint

b. distemper

c. spirit varnish

d. none to these.

6. For the manufacture of Portland cement, the proportions of raw materials used, are

a. lime 63% ; silica 22% ; other ingredients 15%

b. lime 22% ; silica 63% ; other ingredients 15%

c. silica 40% ; lime 40% ; other ingredients 20%

d. silica 70% ; lime 20% ; other ingredients 10%.

7. A pug mill is used for

a. moulding brick earth

b. tempering brick earth

c. providing brick earth

d. all the above.

8. Black marble is generally found in the district of

a. Jaipur

b. Jabalpur

c. Jaisalmer

d. Pune.

9.  is the equation for Euler's crippling load if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

10. A truss containing j joints and m members, will be a simple truss if

a. m = 2j - 3

b. j = 2m - 3

c. m = 3j - 2

d. j = 3m - 2

11. A three hinged arch is loaded with an isolated load 1000 kg at a horizontal distance of 2.5 m from the crown, 1 m above the level of hinges at the supports 10 metres apart. The horizontal thrust is

a. 1250 kg

b. 125 kg

c. 750 kg

d. 2500 kg

12. The cross sections of the beams of equal length are a circle and a square whose permissible bending stress are same under same maximum bending. The ratio of their flexural weights is,

a. 1.118

b. 1.338

c. 1.228

d. 1.108

13. To avoid an interruption in the flow of a syphon, an air vessel is provided

a. at the inlet

b. at the outlet

c. at the summit

d. at any point between intet and outlet.

14. Reynold number is the ratio of initial force and

a. viscosity

b. elasticity

c. gravitational force

d. surface tension.

15. Pick up the correct statement regarding convergent divergent mouth piece from the following :

a. It converges upto Venacontracta and then diverges

b. In this mouth piece there is no loss of energy due to sudden enlargement

c. The coefficient of discharge is unity

d. All the above.

16. Dimensions of the dynamic viscosity (μ) are

a. MLT-2

b. M-1L-1T-1

c. ML-1T-2

d. ML-1T-1.

17. For a homogeneous earth dam 50 m high having 2 m free broad, a flow net was constructed and the results were : Number of potential drops = 2.4 Number of flow channels = 0.4. If coefficiency of permeability of the dam mateiral is 3 x 10-3 cm3/sec, the discharge per metre length of dam, is

a. 12 x 10-5 m3/sec

b. 24 x 10-3 m3/sec

c. 6 x 10-5 m3/sec

d. 24 x 10-5 m3/sec

18. Accurate determination of water content, is made by

a. calcium carbide method

b. sand bath method

c. alcohol method

d. oven-drying method.

19. For general engineering purposes, soils are classified by

a. particle size classification system

b. textural classification system

c. High Way Research Board (HRB), classification system

d. unified soil classification system.

20. The compression index of a soil

a. decreases with an increase in the liquid limit

b. increases with an increase in the liquid limit

c. decreases with an increase in the plastic limit

d. is not related with plastic limit.

21. The minimum thickness of a flat slab is taken

a. L/32 for end panels without drops

b. L/36 for end panels without drops

c. L/36 for interior panels without drop

d. all the above.

22. An R.C.C. beam not provided with shear reinforcement may develop cracks in its bottom inclined roughly to the horizontal at

a. 35°

b. 45°

c. 55°

d. 60°

23. An R.C.C. lintel is spanning an opening of 2 m span in a brick wall. The height of the roof is 2.9 m above the floor level and that of the opening is 2.1 m above the floor level. The lintel is to be designed for self weight plus

a. triangular load of the wall

b. UDL of wall

c. UDL of wall + load from the roof

d. triangular load + load from the roof.

24. In a slab, the pitch of the main reinforcement should not exceed its effective depth

a. three times

b. four times

c. five times

d. two times.

25. For the construction of flyovers in sandy soils, the type of foundation provided, is

a. raft footing

b. combined footing

c. pier footing

d. none of these.

26. The skirting/dado in a bath roof should be upto

a. ceiling

b. 15 cm above floor level

c. 200 cm

d. level of the tap.

27. Concrete is unsuitable for compaction by a vibrator if it is

a. earth moist

b. semi-plastic

c. plastic

d. none of these.

28. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

29. The factor affecting per capita demand, is

a. climatic conditions

b. pressure in water mains

c. cost of water

d. all the above.

30. Pickup the incorrect statement from the following :

a. The invert of pressure conduit is independent of the grade of the hydraulic gradient line

b. The pressure conduits may be taken uphill upto a maximum height of 8.3 m

c. Aqueducts and tunnels sections are generally kept circular

d. None of these.

31. Sewer pipes need be checked for

a. minimum flow

b. maximum flow

c. both (a) and (b)

d. none of these.

32. Sewer manholes are generally provided at

a. the change of gradient

b. the change of direction

c. the junctions of sewers

d. all of these.

33. Effective precipitation for a crop may be defined as

a. total precipitation minus the loss due to evaporation

b. total precipitation minus the loss due to infiltration

c. total precipitation during the crop period

d. available water stored in soil within root zone of the crop.

34. The top soil of a water logged field becomes more alkaline and infertile if its pH value is

a. 8

b. 9

c. 10

d. 11

35. Retrogression of the bed level of a river downstream a weir, occurs due to

a. heavy impact of water

b. increase of the bed level

c. less percentage of silt

d. soft soil strata.

36. Pick up the incorrect statement from the following

a. A bank in the cross-section of a canal is provided on the right

b. A bank in the cross-section of a canal is provided on the left

c. A 'dowla' in the cross-section of a canal is provided on the left

d. None of these.

37. Maximum number of passenger cars that can pass a given point on a road during one hour under the most ideal road way and traffic conditions, is known as

a. traffic density

b. basic capacity of traffic lane

c. possible capacity of traffic lane

d. all the above.

38. The camber on pavements, is provided by

a. parabolic method

b. straight line and parabolic at crown

c. elliptical method

d. circular method.

39. If the ruling gradient on any highway is 3%, the gradient provided on the curve of 300 metre radius, is

a. 2.25%

b. 2.50%

c. 2.75%

d. 3.00%

40. The advantage of providing superelevation on roads, is

a. increased volume of traffic

b. reduced maintenance cost of the roads

c. draining off rain water quickly

d. all the above.

41. If tensile stress of a steel rod of diameter D is 1400 kg/cm2 and bond stress is 6 kg/cm2, the required bond length of the rod is

a. 40 D

b. 50 D

c. 53 D

d. 59 D

42. The rate of an item of work depends on

a. Specifications of materials

b. Proportion of mortar

c. Method of construction

d. All the above.

43. The cross-sections for a highway is taken at

a. 30 metres apart

b. intermediate points having abrupt change in gradient

c. the starting end points of the curves

d. All the above.

44. Pick up the incorrect statement from the following:

a. The built up covered area at the floor level of any storey of a building is called plinth area

b. The usable covered area of the rooms of any storey of a building is called carpet area

c. The carpet area of a building along with area of its kitchen, pantry, store, lavatory, bath room and glazed verandah, is called floor area

d. None of these.

45. The estimated time required to perform an activity, is known as

a. event

b. dummy

c. duration

d. float.

46. Modular co-ordination of construction means proper

a. planning

b. designing

c. execution

d. all the above.

47. Pick up the incorrect statement from the following:

a. The various functions under each activity, are shown by one bar on Bar Charts

b. Bar chart establishes the interdependency of one event on another

c. Only approximate percentage of the completed work is reported

d. None of these,

48. The object of technical planning, is

a. preparation of estimates

b. initiating the procurement action of resources

c. taking remedial action for likely bottleneck in the execution

d. all the above.

49. Pick up the incorrect statement from the following:

a. Channelization of pavement is caused by constant use of tri-cycle gears of aircraft

b. Blast pads are used over cohesionless soils to resist erosion due to tremendous speed of the jet blast

c. Over-run areas of at least 300 m length on either side of the runway are provided

d. None of these.

50. According to the International Civil Aviation Organisation (I.C.A.O.), the runway lengths of aerodromes, have been coded by

a. Seven English alphabets

b. Last Seven English alphabets

c. First Seven English alphabets

d. First seven natural numbers.