***Sub Engineer***

1. While setting a plane table at a station it was found that the error in centering was 30 cm away from the ray of length 40 m drawn from the station. If the scale of the plan is 1 cm = 2 cm, the displacement of the end of the ray in plan from the true position will be

a. 0.02 cm

b. 0.15 cm

c. 02 cm

d. 0.1 cm

2. The ratio of the linear displacement at the end of a line, subtended by an arc of one second to the length of the line, is

a. 1:206 300

b. 1:3440

c. 1:57

d. 1:100.

3. Pick up the correct statement from the following :

a. the angle between the true meridian and the magnetic meridian is called magnetic variation

b. one end of the magnetic needle supported at its centre of gravity tends to dip down towards the. nearer magnetic pole of the earth

c. the magnet properly pivoted is balanced by means of riding weight movable along the needle

d. all the above.

4. A well conditioned triangle has no angle less than

a. 20°

b. 30°

c. 45°

d. 60°.

5. For obtaining vinyl chloride acetate, the method used, is

a. addition polymerization

b. condensation polymerization

c. co-polymerization

d. none of these.

6. The term frog means

a. an apparatus to lift the stone

b. a depression on a face of brick

c. vertical joint in a brick work

d. soaking brick in water.

7. Good quality sand is never obtained from

a. river

b. nala

c. sea

d. gravel powder.

8. If the iron ore contains clay as an impurity, the flux added during calcination, is

a. clay

b. lime stone

c. argillaceous iron ore

d. all the above.

9. The assumption in the theory of bending of beams, is :

a. material is isotropic

b. Young's modulus is same in tension as well as in compression

c. each layer is independent to expand or to contract

d. all the above.

10. The force in BF of the truss shown in given figure, is

a. 4t compression

b. 4.5t tension

c. 4.5t compression

d. zero.

11. Shear force for a cantilever carrying a uniformly distributed load over its length, is

a. triangle

b. rectangle

c. parabola

d. cubic parabola.

12. If a steel rod of 20 mm diameter and 5 metres long elongates by 2.275 mm when subjected to an axial pull of 3000 kg, the stress developed, is

a. 9.5541 kg/cm2

b. 95.541 kg/cm2

c. 955.41 kg/cm2

d. 9554.1 kg/cm2.

13. In flowing liquids pitot tubes are used measure

a. discharge

b. pressure

c. velocity

d. depth.

14. If the forces are due to inertia and gravity, and frictional resistance plays only a minor role, the design of the channels is made by comparing

a. Reynold number

b. Froude number

c. Weber number

d. Mach number.

15. For a most economical trapezoidal open channel, the half of the top width must be equal to

a. the bed width

b. one sloping side

c. the depth of flow

d. None of these.

16. For mountaneous regions having steep slope, wagons for carrying liquid are made with bottom

a. parallel to the road surface

b. parallel to the horizontal surface

c. inclined upward while moving upwards

d. inclined downward while moving downwards.

17. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

18. For general engineering purposes, soils are classified by

a. particle size classification system

b. textural classification system

c. High Way Research Board (HRB), classification system

d. unified soil classification system.

19. 'Drift' is the material picked up, mixed, disintegrated, transported and redeposited by

a. wind

b. gravitational force

c. glaciated water

d. all the above.

20. The zero atmospheric pressure is at

a. mean sea level

b. water table

c. phreatic surface

d. both (c) and (d) of the above.

21. In a cantilever retaining wall without a heel slab

a. thickness of the stem is kept same throughout

b. base slab is made 10 cm thicker than the stem

c. width of the base slab is kept 0.7 time the total height of the wall

d. all the above.

22. For a circular slab carrying a uniformly distributed load, the ratio of the maximum negative to maximum positive radial moment, is

a. 2

b. 3

c. 4

d. 5

23. If the shear stress in a R.C.C. beam is

a. equal or less than 5 kg/cm2, no shear reinforcement is provided

b. greater than 4 kg/cm2, but less than 20 kg/cm2, shear reinforcement is provided

c. greater than 20 kg/cm2, the size of the section is changed

d. all the above.

24. The advantage of reinforced concrete, is due to

a. fire-resisting and durability

b. economy because of less maintenance cost

c. moulding in any desired shape

d. All the above.

25. Black cotton soil is unsuitable for foundations because its

a. bearing capacity is low

b. permeability is uncertain

c. particles are cohesive

d. property to undergo a volumetric change due to variation of moisture content.

26. A temporary rigid structure having platforms to enable masons to work at different stages of a building, is known as

a. scaffolding

b. dead shore

c. raking shore

d. under pinning.

27. The standard sand now a days used in India, is obtained from

a. Jullundur (Punjab)

b. Hyderabad (Andhra Pradesh)

c. Ennore (Madras)

d. Cuttuck (Orissa).

28. According to the recommendations of IS : 456-1978, the expansion joints

a. are provided to ensure minimum resistance

b. are supported on separate columns

c. do not carry reinforcement across them

d. all the above.

29. A city supply includes

a. industrial and commercial water demands

b. demand for public uses and fire

c. water losses

d. all the above.

30. If the specific capacity of a well is 0.3183 x 10-3 per sec, the discharge from a well of 4 m diameter under a depression head of 4 m, is

a. 10 litres/sec

b. 12 litres/sec

c. 14 litres/sec

d. 16 litres/sec.

31. During putrifaction process of sewage the gas given off, is

a. hydrogen

b. ammonia

c. methane

d. all the above.

32. To maintain aerobic biological activity, the moisture content of the compost mass should be about

a. 45%

b. 50%

c. 55%

d. 60%.

33. For standing crops in undulating sandy fields, the best method of irrigation, is

a. sprinkler irrigation

b. free flooding

c. check method

d. furrow method

34. Groynes are generally built

a. perpendicular to the bank

b. inclined up stream up to 30°

c. inclined down stream upto 30°

d. all the above.

35. Pick up the correct statement from the following

a. The capacity of escapes should not be less than the capacity of the canal at its location

b. The escapes must lead the surplus water to natural drainages

c. The escapes are aligned to take advantage of contours of lower values

d. All the above.

36. A water-logged land is found suitable for cultivation due to

a. ease of tillage for preparation of the field for optimum condition of germination

b. absence of aeration of soil from root zone of the plant

c. regular supply of water to plants from the water table by capillary action

d. none of these.

37. Pick up the correct statement from the following:

a. The corss fall of the shoulder should be at least 0.5% steeper than camber

b. On superelevated sections, the shoulders should be provided a cross fall equal to camber

c. Earthern roads in general are provided steepest crossfall

d. All the above.

38. Formation width of a hill road is the distance between

a. outer end of side drain to outer end of parapet wall

b. outer end of side drain to inner end of parapet wall

c. inner end of side drain to outer end of parapet wall

d. inner end of side drain to inner end of parapet wall

39. The minimum design speed of various types of highways in plain terrain is the same as the ruling design speed of

a. rolling terrain

b. mountainous terrain

c. steep terrain

d. none of these.

40. On most smooth hard surfaced roads, rolling resistance to moving vehicles, ranges from

a. 7 kg to 9 kg/tonne

b. 9 kg to 11 kg/tonne

c. 11 kg to 13 kg/tonne

d. none of these.

41. In long and short wall method of estimation, the length of long wall is the centre to centre distance between the walls and

a. breadth of the wall

b. half breadth of wall on each side

c. one fourth breadth of wall on each side

d. None of these.

42. The ground surface slopes 1 in 50 along a proposed railway embankment 150 m in length. The height of the embankment at zero chainage is 0.5 m, the width is 11 m and side slopes 2:1. If the falling gradient of the embankment is 1 in 150, the quantity of the earthwork calculated by prismoidal formula, is

a. 3250 m3

b. 3225 m3

c. 3275 m3

d. 3300 m3

43. The brick work is not measured in cu m in case of

a. One or more than one brick wall

b. Brick work in arches

c. Reinforced brick work

d. Half brick wall.

44. If B is the width of formation, d is the height of the embankment, side slope S : 1, for a highway with no transverse slope, the area of cross-section is

a. B + d + Sd

b. Bd + Sd2

c. B x d - Sd1/2

d. (Bd + Sd2)

45. The time which results in the leasi, possible construction cost of an activity, is known

a. normal time

b. slow time

c. crash time

d. standard time.

46. Bar charts are suitable for

a. minor works

b. major works

c. large projects

d. all the Above.

47. While filling the tender for any work, the contractor considers

a. availability of construction materials

b. availability of labour

c. study of specifications

d. all the above.

48. The time by which activity completion time can be delayed without affecting the start of succeeding activities, is known as

a. duration

b. total flat

c. free float

d. interfering float.

49. Pick up the correct statement from the following:

a. Rubberised tar concrete hot blast as well as spillage

b. Epoxy asphalt concrete resists the jet fuels and provides more elastic pavement

c. Epoxy asphalt concrete sets in very small time

d. All the above.

50. The reduced levels of the ends A and B of a runway are 3025 m and 3035 m and that of its mid-point is 3015 m. The reduced level of the horizontal surface, is

a. 3060 m

b. 3075 m

c. 3015 m

d. none of these.