***Sub Engineer***

1. The best method of interpolation of contours, is by

a. estimation

b. graphical means

c. computation

d. all of these.

2. Simpson's rule for calculating areas states that the area enclosed by a curvilinear figure divided into an even number of strips of equal width, is equal to

a. one third the width of a strip, multiplied by the sum of two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

b. one third the width of a strip, multiplied by the sum of two extreme offsets, four times the sum of the remaining odd offsets, and twice the sum of the even offsets

c. one sixth the width of a strip, multiplied by the sum of the two extreme offsets, twice the sum of remaining odd offsets and four times the sum of the even offsets

d. none of these

3. For the construction of highway (or railway)

a. longitudinal sections are required

b. cross sections are required

c. both longitudinal and cross sections are required

d. none of these.

4. In optical reading instruments

a. the vertical circle is usually continuous from 0° to 359°

b. the readings increase when the telescope is elevated in the face left position

c. the readings decrease when the telescope is elevated in the face right position

d. all the above.

5. When a brick is immersed in water for 24 hours and then dried, if

a. no grey or white deposits appear on the surface, the brick is free from soluble salts

b. 10 per cent surface is covered with grey or white deposits, the brick has slight efflorescence

c. 50 per cent surface is covered with grey or white deposits, the brick has serious efflorescence

d. All the above.

6. Inhaling of fly-ash over a long period causes

a. fibrosis of lungs

b. bronchitis

c. pneumonitis

d. All of these.

7. Pick up the plutonic rock from the following:

a. Granite

b. Dolerite

c. Basalt

d. All the above.

8. The cracks caused by shrinkage of the exterior surface of the wood exposed to atmosphere, are called:

a. radial shakes

b. heart shakes

c. wind cracks

d. twisted fibres.

9. In a shaft, the shear stress is not directly proportional to

a. radius of the shaft

b. angle of twist

c. length of the shaft

d. modulus of rigidity.

10.  is the equation of Euler's crippling load, if

a. both the ends are fixed

b. both the ends are hinged

c. one end is fixed and other end is free

d. one end is fixed and other end is hinged.

11. A composite member shown in below figure was formed at 25°C and was made of two materials a and b. If the coefficient of thermal expansion of a is more than that of b and the composite member is heated upto 45°C, then

a. a will be in tension and b in compression

b. both will be in compression

c. both will be in tension

d. a will be in compression and b in tension.

12. In a three hinged arch, the third hinge is generally kept at

a. crown of the arch

b. midpoint of the crown and left support hinge

c. midpoint of the crown and right support hinge

d. none of these.

13. Liquids

a. cannot be compressed

b. do not occupy definite shape

c. are not affected by change in pressure and temperature

d. none of these.

14. If cohesion between the molecules of a fluid is more than adhesion between the fluid and glass, the free level of fluid in a dipped glass tube will be

a. higher than the surface of liquid

b. same as the surface of liquid

c. lower than the surface of liquid

d. none of these.

15. A stepped notch is a combination of

a. rectangular notches of different sizes

b. triangular notches of different sizes

c. rectangular and triangular notches

d. all the above.

16. For steady flow in open channels, which one of the following does not change :

a. depth of flow

b. velocity of flow

c. rate of flow

d. All of these.

17. The compression resulting from a long term static load and consequent escape of pore water, is known as

a. compaction

b. consolidation

c. swelling

d. none of these.

18. The direct shear test suffers from the following disadvantage:

a. Pore water pressure cannot be measured

b. Shear stress on the failure plane is not uniform.

c. The area under the shear and vertical loads does not remain constant throughout the test

d. All the above.

19. The fundamental equation of air content (ac), degree of saturation (Sr) and void ratio (e), is

a. ac =

b. ac =

c. ac =

d. ac = 1 + e/Sr

20. Pick up the correct statement from the following:

a. The line which shows moisture content dry density relation for soil containing a constant percentage of air voids, is known as air void line

b. The weight of hammer used for compaction test is 25 kg

c. The free fall of hammer for compaction is 30.5 cm

d. All the above.

21. The reinforced concrete beam which has width 25 cm, lever arm 40 cm, shear force 6t/cm2, safe shear stress 5 kg/cm2 and B.M. 24 mt,

a. is safe in shear

b. is unsafe in shear

c. is over safe in shear

d. needs redesigning.

22. The design of heel slab of a retaining wall, is based on the maximum bending moment due to:

a. weight of the soil above it,

b. load of the surcharge, if any

c. upward soil reaction

d. all the above.

23. If the permissible compressive and tensile stresses in a singly reinforced beam are 50 kg/cm2 and 1400 kg/cm2 respectively and the modular ratio is 18, the percentage area At of the steel required for an economic section, is

a. 0.596%

b. 0.696%

c. 0.796%

d. none of these.

24. If diameter of a reinforcement bar is d, the anchorge value of the hook is

a. 8d

b. 12d

c. 16d

d. none of these.

25. In places where the soil is soft and has small resistance to the flow of concrete, which one of the following types of piles, is used

a. vibro pile

b. pressure pile

c. Franki pile

d. pedestal pile.

26. The arrangement of supporting an existing structure by providing supports underneath, is known as

a. shoring

b. underpinning

c. jacking

d. piling

27. The condition not applicable to water cement ratio law, is

a. concrete specimens may be tested at any temperature

b. concrete specimens need be of same age

c. concrete specimens need be of same size

d. none of these.

28. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

29. Hard water for public water supply is discarded because

a. it consumes more soap

b. it contains lot of turbidity

c. it contains pathogenic bacterias

d. it possesses bad taste and odour

30. Detention time for plain sedimentation tank usually ranges from

a. 4 to 8 hours

b. 6 to 10 hours

c. 8 to 12 hours

d. 12 to 16 hours.

31. D is the diameter of a circular sewer and α is the side of a square section sewer. If both are hydraulically equivalent, the relationship which holds good, is

a. πD8/3 = 4 b8/3

b. πD3/8 = 4 b3/8

c. πD2/3 = 4 b2/3

d. πD3/2 = 4 b3/2

32. The sewerage system originates from

a. house sewers

b. lateral sewers

c. branch sewers

d. main sewers

33. Useful soil moisture for plant growth, is

a. capillary water

b. gravity water

c. hygroscopic water

d. chemical water

34. For loss of head in a canal inverted syphon barrel, the factor in the Unwin formula  is a coefficient for loss of head due to

a. friction

b. exit

c. entry

d. gradient

35. For diversion of flood water of rivers, the type of canal constructed, is

a. ridge canal

b. perennial canal

c. inundation canal

d. canal.

36. To hold hydraulic jumps, baffle walls are provided in

a. Sarda type falls

b. English type falls

c. Montague type falls

d. Vertical type falls.

37. The ratio of maximum deviation angle and maximum polar deflection angle of a Lemniscate curve, is

a. 3

b. 4

c. 5

d. 6

38. Shoulders for high traffic volume roads, should

a. be stable throughout the year to be used by vehicles in the case of emergency

b. support and protect the ends of carriage ways

c. not allow entrance of water to sub-grade

d. all the above.

39. Retaining walls are generally constructed with dry rubble stones with 60 cm top width and

a. 1 : 2 front batter

b. 1 : 3 front batter

c. 1 : 4 front batter

d. 1 : 5 front batter

40. Maximum super-elevation on hill roads should not exceed

a. 7%

b. 8%

c. 10%

d. 15%

41. Size, capacity and materials need be specified for

a. Stop-cocks

b. Ferrules

c. Ball valves

d. All the above.

42. The expected out turn for earth work in excavation in ordinary soil per mazdoor per day is

a. 1.00 cubic meter

b. 2.00 cubic meter

c. 3.00 cubic meter

d. 4.00 cubic meter

43. The minimum width of a septic tank is taken

a. 70 cm

b. 75 cm

c. 80 cm

d. 90 cm

44. The assumption on which the trapezoidal formula for volumes is based, is

a. The end sections are parallel planes

b. The mid-area of a pyramid is half the average area of the ends

c. The volume of the prismoidal is over-es-timted and hence a prismoidal correction is applied

d. All the above.

45. Works costing less than Rs. 20,000 are treated as

a. projects

b. major projects

c. minor projects

d. all the above.

46. Pick up the correct statement from the following:

a. If the float is positive and the activity is delayed by a period equal to its total float, the completion of project is not delayed

b. If the float of an activity is negative, delay in its performance is bound to delay the completion of project

c. If the float of an activity is zero, the activity is critical and any delay in its performance will delay the whole project

d. All the above.

47. Pre-tender stage requires

a. selection of site

b. formalisation of alignment of work

c. formalisation of designs and preparation of estimate

d. all the above.

48. The main disadvantage of line organisation, is

a. rigid structure

b. extraordinary delay in communications

c. top level executions over work

d. all the above.

49. According to I.C.A.O. the slope of transitional surface at right angles to the centre line of runway, is kept

a. 1 in 5

b. 1 in 6

c. 1 in 7

d. 1 in 10

50. Pick up the correct statement from the following:

a. The propellers which are driven by turbine engines, are technically called turboprops

b. The aircrafts which obtain the thrust directly from turbine engine, are called turbo-jets

c. The aircraft which is filled with a fan infront of turbo-jet engine, is called turbofan propelled aeroplane

d. All the above.