***Sub Engineer***

1. In case of Raymond pile

a. diameter of top of piles varies from 40 cm to 60 cm

b. diameter of pile at bottom varies from 20 cm to 28 cm

c. thickness of outer shell depends upon pile diameter

d. all the above.

2. Queen closer may be placed

a. in stretcher course

b. in header course next to first brick

c. in stretcher course next to first brick

d. in any position.

3. Dado is usually provided in

a. bath rooms

b. living rooms

c. verandah

d. roofs.

4. The foundation in which a cantilever beam is provided to join two footings, is known as

a. strap footing

b. combined footing

c. raft footing

d. none of these.

5. The foundations are placed below ground level, to increase

a. strength

b. workability

c. stability of structure

d. all the above.

6. Stud(s) of a common wooden partition

a. are vertical wooden members

b. is the upper horizontal wooden member

c. is the lower horizontal wooden member

d. are the intermediate horizontal wooden members.

7. Pick up the correct statement from the following :

a. inclined borings are made for taking samples under existing structures

b. inclined borings are occasionally used instead of vertical holes.

c. the spacing of inclined borings is kept such that one bore hole is vertically above the bottom of an adjacent bore hole.

d. all the above.

8. Pick up the commonly adopted geophysical method in civil engineering from the following :

a. electrical resistivity method

b. gravitational method

c. magnetic method

d. both (a) and (b) of the above.

9. To ensure that supporting area of an offset footing of a boundary wall is fully compressive, the C.G. of load must act

a. at the centre of the base

b. within the middle third of the base

c. within the middle fifth of the base

d. neither (a), (b) nor (c).

10. The 9 cm x 9 cm side of a brick as seen in the wall face, is generally known as

a. face

b. front

c. header

d. side.

11. The taper of precast concrete pile should not be more than

a. 1 cm per metre length

b. 2 cm per metre length

c. 4 cm per metre length

d. 5 cm per metre length.

12. The under surface of an arch, is called

a. soffit

b. intrados

c. haunch

d. back.

13. The raft slab is projected beyond the outer walls of the structure by

a. 15 to 20 cm

b. 25 to 30 cm

c. 30 to 45 cm

d. 60 cm.

14. The process of making the back ground rough, before plastering, is

a. dubbing

b. hacking

c. blistering

d. peeling.

15. Black cotton soil is unsuitable for foundations because its

a. bearing capacity is low

b. permeability is uncertain

c. particles are cohesive

d. property to undergo a volumetric change due to variation of moisture content.

16. The loose pockets in soil mass can be bridged safely by providing a raft foundation provided the soft area is smaller than

a. one-third the column spacing

b. half the column spacing

c. three-fourth the column spacing

d. none of these.

17. The portion of a brick cut across the width, is called

a. closer

b. half brick

c. bed

d. bat.

18. The concrete slump recommended for beams and slabs ; is

a. 25 to 75 mm

b. 30 to 125 mm

c. 50 to 100 mm

d. none of these.

19. The member which is placed horizontally to support common rafter of a sloping roof, is

a. purlin

b. cleat

c. batten

d. strut.

20. Pick up the correct statement from the following:

a. Flush  door  is  generally  provided   in dinning room

b. Revolving door is generally provided in cinema halls

c. Sliding door is generally provided in show rooms

d. All the above.

21. Grillage foundation

a. is light and economical

b. does not require deep cutting as the required base area with required pressure intensity is obtained at a shallow depth

c. is constructed by rolled steel joists (R.S.J.) placed in single or double tier

d. all the above.

22. The exterior angle between outer faces of a wall, is known as

a. turn

b. junction

c. quion

d. all the above.

23. In jack arch floor, the rise is kept

a. 1/8th of the span

b. 1/10th of the span

c. 1/12th of the span

d. 1/15th of the span.

24. While designing a stair, the product of rise and going is approximately kept equal to

a. 420

b. 450

c. 500

d. 600.

25. For constructing a terrazo floor. Pick up the incorrect statement from the following :

a. a 32 mm thick layer of cement concrete (1 : 2 : 4) is laid on the base course and the surface is made smooth by trowelling

b. glass strips are driven into the layer according to the pattern required

c. after final grinding is over, oxalic acid mixed with water is spread over and rubbed hard with soft material

d. none of these.

26. The entrained concrete is used in lining walls and roofs for making

a. heat insulated

b. sound insulated

c. neither (a) nor (b)

d. both (a) and (b).

27. The pile which is provided with a bulb filled with concrete at its lower end, is known as

a. Mac-Arthur pile

b. Raymond pile

c. Franki pile

d. none of these.

28. In case of multi-storeyed buildings, the forms to be removed first are

a. sides of beams and girders

b. column forms

c. bottom of beams and girders

d. all the above at the same time.

29. For providing a raft foundation, the following activities are involved

a. 1, 2, 3, 4, 5

b. 5, 4, 3, 2, 1

c. 2, 1, 3, 5, 4

d. 3, 2, 5, 1, 4.

30. The maximum permissible deflection of a timber beam supporting a roof, is

a. L/150

b. L/260

c. L/360

d. none of these.

31. The angular steps used for changing direction of the stairs, are called

a. angular steps

b. winders

c. radial steps

d. circular steps.

32. During percussion drilling

a. ground water observations are hindered due to entry of the slurry in the soil below the bottom of the hole

b. caving or mixing of strata are caused in soft soils or cohesionless soils

c. the soil to a considerable depth below the bottom of the hole gets disturbed

d. all the above.

33. In English garden wall bond

a. one course of headers to three or five course of stretchers

b. queen closer in provided in each heading course

c. the middle course of stretchers is started with a header to give proper vertical joints

d. all the above.

34. A projecting piece usually provided to support a truss, is

a. cornice

b. coping

c. frieze

d. lintal.

35. The line of intersection of the surfaces of a sloping roof forming an external angle exceeding 180°, is

a. ridge

b. hip

c. valley

d. none of these.

36. A solid core of rock is formed in side the cylinder in the case of

a. auger boring

b. percussion drilling

c. diamond drilling

d. wash boring.

37. The single stage well point system of dewatering an excavation can be used if the depth of excavation does not exceed

a. 5 m

b. 10 m

c. 15 m

d. 20 m

38. The piece of a brick cut with its one corner equivalent to half the length and half the width of a full brick, is known as

a. queen closer

b. bevelled closer

c. king closer

d. half king closer.

39. The brick laid with its breadth parallel to the face of a wall, is known as

a. header

b. stretcher

c. closer

d. none of these.

40. A wooden block hinged on post outside a door, is known

a. cleat

b. stop

c. horn

d. none of these.

41. Pick up the incorrect statement from the following :

a. No  timbering is  required  for  shallow trenches

b. Shallow foundations can be constructed on made-up soil

c. Grillage   foundation   is   classified as a shallow foundation

d. Black cotton soil is very good for foundation bed.

42. In horizontal D.P.C, thickness of cement concrete (1 : 2 : 4) is

a. 4 cm

b. 6 cm

c. 8 cm

d. 10 cm.

43. Which one of the following factors is considered for the orientation of buildings :

a. the direction of the prevailing winds in the area

b. the exposure of the walls and roof of the buildings to the rays of sun

c. the extent up to which the sunrays penetrate with the verandah.

d. all the above.

44. Pick up the incorrect statement from the following :

a. Cement is added to lime mortar to increase its hydraulic properties only

b. Lime surkhi mortar is used for pointing the walls

c. Lime should be slaked before preparing lime mortar

d. High early strength concrete is generally used in cold weather.

45. Which one of the following rocks is used for monumental buildings :

a. granite

b. marble

c. sand stone

d. slate.

46. The Auger borings are not common

a. in soils that require lateral support

b. in cohesive soils

c. in soft soils

d. none of the above.

47. The form work from the underside of slabs, can be removed only after

a. 1 day

b. 4 days

c. 7 days

d. 14 days.

48. Arches in the form of masonry arcs struck from more than four centres, are called

a. two curved arches

b. gothic arches

c. ogee arches

d. drop gothic arches.

49. If (φ) is the angle of repose of soil of weight w kg/m3, the horizontal pressure p at a depth of h metres per metre length of wall, is

a.

b.

c.

d.

50. According to Rankine's formula, minimum depth of foundations, is

a.

b.

c.

d.