***Sub Engineer***

1. For quality control of Portland cement, the test essentially done is

a. soundness

b. tensile strength

c. consistency

d. all the above.

2. If 1500 g of water is required to have a cement paste 1875 g of normal consistency, the percentage of water is,

a. 25%

b. 30%

c. 35%

d. 40%

3. Under normal conditions using an ordinary cement, the period of removal of the form work, is :

a. 14 days for bottom slabs of spans 4.6 m and more

b. 21 days for bottom beams over 6 m spans

c. 2 days for vertical sides of columns

d. all the above.

4. Wp and Wf are the weights of a cylinder containing partially compacted and fully compacted concrete. If the compaction factor  is 0.95, the workability of concrete is

a. very low

b. low

c. high

d. none of these.

5. For given water content, workability decreases if the concrete aggregates contain an excess of

a. flat particles

b. elongated particles

c. flaky paticles

d. all the above.

6. M10 grade of concrete approximates

a. 1 : 3 : 6 mix

b. 1 : 1 : 2 mix

c. 1 : 2 : 4 mix

d. 1 : 1.5 : 3 mix

7. For ensuring quality of concrete, use

a. single sized aggegates

b. two sized aggregate

c. graded aggregates

d. coarse aggregates.

8. According to I.S. : 456, the number of grades of concrete mixes, is

a. 4

b. 5

c. 6

d. 7

9. The mixture of different ingredients of cement, is burnt at

a. 1200°C

b. 1400°C

c. 1600°C

d. 1800°C

10. The risk of segregation is more for

a. wetter mix

b. larger proportion of maximum size aggregate

c. coarser grading

d. all the above.

11. After casting, an ordinary cement concrete on drying

a. expands

b. mix

c. shrinks

d. none of these.

12. Hydration of cement is due to chemical action of water with

a. Tricalcium silicate and dicalcium silicate

b. Dicalcium silicate and tricalcium aluminate

c. Tricalcium aluminate and tricalcium alumino ferrite

d. All the above.

13. To obtain cement dry powder, lime stones and shales or their slurry, is burnt in a rotary kiln at a temperature between

a. 1200° and 1300°C

b. 1300° and 1400°C

c. 1400° and 1500°C

d. 1500° and 1600°C

14. Permissible compressive strength of M 300 concrete grade is

a. 150 kg/cm2

b. 200 kg/cm2

c. 250 kg/cm2

d. 300 kg/cm2

15. The standard sand now a days used in India, is obtained from

a. Jullundur (Punjab)

b. Hyderabad (Andhra Pradesh)

c. Ennore (Madras)

d. Cuttuck (Orissa).

16. The maximum amount of dust which may be permitted in aggregates is

a. 5% of the total aggregates for low workability with a coarse grading

b. 10% of the total aggregates for low workability with a fine grading

c. 20% of the total aggregates for a mix having high workability with fine grading

d. all the above.

17. Proper proportioning of concrete, ensures

a. desired durability

b. water tightness of the structure

c. resistance to water

d. all the above.

18. The bulk density of aggregates does not depend upon :

a. specific gravity of aggregates

b. grading of aggregates

c. size and shape of the container

d. none of these.

19. Curing

a. reduces the shrinkage of concrete

b. preserves the properties of concrete

c. prevents the loss of water by evaporation

d. all of the above.

20. While compacting the concrete by a mechanical vibrator, the slump should not exceed

a. 5.0 cm

b. 7.5 cm

c. 10 cm

d. 15 cm

21. Construction joints are provided

a. where the member is supported by other member

b. at 18 m apart in huge structures

c. in concrete wall at sill level of windows

d. all the above.

22. An aggregate is said to be flaky if its least dimension is less than

a. 2/5th of mean dimension

b. 3/5th of mean dimension

c. 4/5th of mean dimension

d. none of these.

23. The following proportion of the ingredients of concrete mix, is not in conformation to arbitrary method of proportioning

a. 1 : 2 : 4

b. 1 : 3 : 6

c. 1 : 2 : 8

d. 1 : 4 : 10

24. The increased cohesiveness of concrete, makes it

a. less liable to segregation

b. more liable to segregation

c. more liable to bleeding

d. more liable for surface scaling in frosty weather

25. The ratio of the length to breadth of a wooden float, is

a. 5.5

b. 6.5

c. 7.5

d. 8.5

26. To ensure constant moisture content in aggregates

a. height of each aggregate pile should not exceed 1.50 m

b. aggregate pile should be left for 24 hours before aggregates are used

c. conical heaps of aggregates should be avoided to prevent moisture variation

d. all the above.

27. Workability improved by adding

a. foaming agent

b. oily-agent

c. aluminium compound

d. all the above.

28. The commonly used material in the manufacture of cement is

a. sand stone

b. slate

c. lime stone

d. graphite.

29. If 20 kg of coarse aggregate is sieved through 80 mm, 40 mm, 20 mm, 10 mm, 4.75 mm, 2.36 mm, 1.18 mm, 600 micron, 300 micron and 150 micron standard sieves and the weights retained are 0 kg, 2 kg, 8 kg, 6 kg, 4 kg respectively, the fineness modulus of the aggregate, is

a. 7.35

b. 7.40

c. 7.45

d. none of these.

30. Curing a concrete for long period ensures better

a. strength

b. water resistance

c. water tightness and durability

d. all the above.

31. For the construction of cement concrete floor, the maximum permissible size of aggregate, is

a. 6 mm

b. 8 mm

c. 10 mm

d. 12 mm

32. The process of proper and accurate measurement of concrete ingredients for uniformity of proportion, is known

a. curing

b. mixing

c. batching

d. none of these.

33. Pick up the correct statement from the following:

a. Insufficient quantity of water makes the concrete unworkable

b. Excess quantity of water makes the concrete segregated

c. Excess quantity of water causes bleeding in concrete

d. All the above.

34. The preliminary test is repeated if the difference of compressive strength of three test specimens, exceeds

a. 8 kg/cm2

b. 10 kg/cm2

c. 12 kg/cm2

d. 15 kg/cm2

35. Pick up the correct proportions of chemical ingredients of cement

a. Lime : Silica : Alumina : Iron oxide : 63 : 22 : 6 : 3

b. Silica : Lime : Alumina : Iron oxide : 63 : 22 : 6 : 3

c. Alumina : Silica : Lime : Iron oxide : 63 : 22 : 6 : 3

d. Iron oxide : Alumina : Silica : Lime : 63 : 22 : 6 : 3

36. Transport of concrete by pumps, is done for a distance of

a. 200 m

b. 300 m

c. 400 m

d. none of these.

37. If X, Y and Z are the fineness modulli of coarse, fine and combined aggregates, the percentage (P) of fine aggregates to combined aggregates, is

a.

b.

c.

d.

38. Slump test is done for

a. clay

b. sand

c. lime

d. concrete.

39. The high strength of rapid hardening cement at early stage, is due to its

a. finer grinding

b. burning at high temperature

c. increased lime cement

d. higher content of tricalcium.

40. Pick up the correct statement from the following:

a. Water and aggregates are measured in litres

b. The finished concrete is measured in cubic metres

c. 20 bags of cement make one tonne

d. All the above.

41. Concrete mainly consists of

a. aggregates

b. admixture

c. water

d. all the above.

42. Vicat's apparatus is used for

a. consistency test

b. setting time test

c. soundness test

d. compressive strength test.

43. M 150 grade of concrete approximates

a. 1 : 1 :2 mix

b. 1 : 2 : 4 mix

c. 1 : 1.5 : 3 mix

d. none of these.

44. Workability of concrete is measured by

a. Vicat apparatus test

b. Slump test

c. Minimum void method

d. Talbot Richard test.

45. The rock which is not calcareous, is :

a. macl

b. chalk

c. laterite

d. none of these.

46. Internal friction between the ingredients of concrete, is decreased by using

a. fine aggregates

b. rich mix

c. more water and coarse aggregates

d. none of these.

47. For road pavements, the cement generally used, is

a. rapid hardening cement

b. low heat cement

c. blast furnace slag cement

d. none of these.

48. Construction joints are generally provided in concrete

a. retaining walls

b. lining of canals

c. lining of tunnels

d. all the above.